3GPP TSG-RAN WG4 Meeting #118	R4-2600596

# Gothenburg, SE, 9th-13th February, 2026

Title:	On n114 UE RF requirements

Source:	Qualcomm, FirstNet, APCO, AT&T

Agenda item:	5.8.4

Document for:	Discussion

# Background

A new Rel-20 WI [1] was created to ‘allow for public safety operation in line with the framework identified by the FCC within the frequency range, 4940-4990 MHz, within 5G NR’, for which ‘a new NR TDD operating band needs to be defined’. This new band is denoted as n114 in 3GPP.

In a previous WF [8], some aspects of the FCC regulation were translated to 3GPP language. A subsequent WF [10] captured a rethink in RAN4 on how to make this translation. In this contribution, we investigate an alternative to translate FCC regulation into 3GPP language.

# Discussion

## Measured vs declared power to set FCC mask

### 2.1.1	Background

On placement of the mask, the FCC regulation states [4]:

“The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission”

RAN4 initially agreed [8] that the 0 dB reference level (abbreviated as ‘0 dBr’ in this paper) associated with FCC mask M depends on the maximum output power for the UE’s power class, which is equivalent to a power declaration for certification:

This method allows for a fixed mask, which is desirable for protection of incumbents, but it does make an assumption that having just a subset of allocations that could meet the declared power would suffice. Specifically, if an allocation were backed off from the declared power to fit inside the mask, would it trigger extra scrutiny because of discrepancy between power of allocation under test and power level used to set the mask reference. Our previous work [9] showed that only a limited subset of allocations can meet the declared power level, which means many allocations could be subject to scrutiny if regulation interpretation differed from the assumption made. This may not be conservative enough for a standards body and for a regulation-facing requirement.

Another aspect is interpretation of ‘0 dB attenuation’ in the flat part of the mask spanning the central 90% of the channel. On the one hand it could be aggressively interpreted as ‘no attenuation required’ and allocations are only limited to the +21 dBm/MHz limit. On the other hand, this aspect too may not be conservative enough for a regulation-facing requirement, absent any clear way to treat such allocations in the regulation.

An alternative approach was to investigate mask location by PSD of the actual output of the signal [11]. With this method, narrow allocations may have higher PSD, and therefore a more relaxed mask. It is not evident that this relaxation for narrow allocations was foreseen during regulation freeze. A mask that can be located by UL PSD alone therefore may not be conservative enough, as also point out in [7].

A hybrid approach that combines the more conservative aspects of both approaches may therefore be more suitable for this band.

Observation 2-1: A conservative approach to regulation for n114 is to adopt a fixed mask for all allocations, where the fixed mask is determined by measured spectrum of a ‘max. allocation waveform’.

Specifically, the attributes retained from the earlier candidate approaches would be:

- 0 dBr of mask determined by some wide allocation centrally located in the channel, to be adopted as the ‘max. allocation’ for this special case

- A fixed mask for all allocations: RAN4 self-limits PSDs of narrow allocations to the mask 0 dB level determined above rather than being allowed up to 21 dBm/MHz

### 2.1.2	Max. output power vs max. allocation size

Using the guidelines above, since the mask is decided by some ‘max. allocation’, our first study evaluates sensitivity of output power as a function of the max. allocation. This type of trade-off was previously proposed [12].  The study was conducted on a PC3 UE using QPSK modulation and a CP-OFDM waveform, with bandwidths of 10 MHz and 30 MHz. The maximum allocation width was adjusted by removing RBs from both ends of the spectrum. As more RBs are removed, the ‘max. allocation’ specific to n114 operation (or ‘n114 max. allocation’) narrows accordingly. Since the FCC mask’s 0 dBr bandwidth is 90% of the UE bandwidth, any max allocation exceeding this limit fails emissions at all output powers. Thus, 10 MHz fails full-RB allocation, while 30 MHz fails any allocation using the first or last four RBs from both sides of the spectrum.

Figure 2.1.2-1 PC3 max. output power level for various sizes of ‘n114 max. allocation’

As seen in the above figure the output power level increases in proportion to the number of RBs removed.

Observation 2-2:  A narrower width of ‘max. allocation’ specific to n114 operation allows higher PSD for all allocations (i.e. higher 0dBr)

When the data is plotted as a percentage of the remaining RBs, it looks as follows.

Figure 2.1.2-2  PC3 max. output power level as a function of unused RBs

The figure above allows the 0 dBr level to be determined based on the percentage of used RBs. As shown in the next section, the usable RB region (determined by the chosen maximum allocation), directly defines the 0dBr value and hence the required back-off within.

### 2.1.3	Back-off requirement

The adoption of ‘fixed mask for all allocations’ (associated with point#2 in section 2.1.1) means that allocations narrower than the ‘n114 max. allocation’ are limited to the same PSD as the max. allocation. While this is unfortunate for UL coverage, it is conservative from an emissions perspective and therefore preferred.

Figure 2.1.3-1 illustrates the projected back-off requirements for three n114 maximum-allocation candidates examined in the previous section (46 RBs, 40 RBs, and 34 RBs) for a 10 MHz bandwidth.

[RB start=3, RB stop=48]

[RB start=6, stop=45]

[RB start=9, RB stop= 42]

c

ccv

c

ccvc

ccvc

c

ccvc

Figure 2.1.3-1 A-MPR plots for 3 different ‘n114 max allocation’ candidates (46, 40 and 34)

To improve clarity, the AMPR upper limit in the above plots was set to 10 dB, and blue triangles are used to mark each permissible allocation region based on the choice of max allocation.

[RB start=9, RB stop= 42]

[RB start=6, stop=45]

[RB start=3, RB stop=48]

ccvc

ccvc

ccvc

ccvc

Figure 2.1.3-1 A-MPR plots limited to a max value of 10dB

As expected these permissible allocation regions become smaller as the ‘n114 max allocation’ becomes narrower. The higher PSD set by the narrower max. allocations enables higher output powers for all allocations. For example, comparing the left-most and right-most plots shows all allocations associated with the narrower ‘n114 max. allocation’ (right-most plot) have roughly 1.6 dB higher output power capability.

Observation 2-3: Choosing a narrow ‘max. allocation’ size for n114 operation allows high UL power.

While there is no obvious optimal choice for size of the ‘n114 max. allocation’, one method is to choose some convenient reduction of SU, like to 80% of the 5G value. For example, from figure 2.1.2-2, PC3 UEs could be expected to transmit at least 17.5 dBm at their max. allocations for each configured BW.

We expect the A-MPR for different maximum allocations to satisfy the following relationship:

A-MPR = A-MPR_full allocation + 10·log₁₀(full allocation / LCRB)

where LCRB refers to the RB length associated with the selected n114 maximum allocation, which is shorter than the full-allocation RB length.

Observation 2-4: It is expected that the A-MPR will satisfy the following relationship for ‘max allocations’:
A-MPR = A-MPR_full allocation + 10·log₁₀(full allocation / LCRB)

where LCRB refers to the RB length associated with the selected n114 maximum allocation

Proposal 2-1: For each UE bandwidth of interest, select a maximum RB allocation and derive the back-off requirements based on the corresponding 0 dBr value.

## 2.3	UE hardware assumption

The WID sets out this objective: ‘Specify the UE RF requirements for the new bands. Reuse NR band n79 UE hardware assumption if possible’. From a UE perspective, since n114 is a subset of n79, reuse of hardware is possible, and a valid assumption. The transmit side requirements may impose new challenges (owing to different SEM) but that will be handled separately via NS. On the receive side, the implication of reusing n79 hardware is also adoption of n79 jammer requirements. While this assumption is equivalent to establishing a relaxation, as a special case it can be considered to enable relatively low-cost support on the UE side. Note that the relaxation only increases the susceptibility of the UE to interferers outside the channel, it does not cause more interference to neighbors.

Proposal 2-2: RAN4 to confirm assumption of reuse of n79 hardware for n114

# 3.	Conclusions

We have further studied the RF requirements for n114 GHz and the following observations and proposals are made

Observation 2-1: A conservative approach to regulation for n114 is to adopt a fixed mask for all allocations, where the fixed mask is determined by measured spectrum of a ‘max. allocation waveform’.

Observation 2-2:  A narrower width of ‘max. allocation’ specific to n114 operation allows higher PSD for all allocations (i.e. higher 0dBr)

Observation 2-3: Choosing a narrow ‘max. allocation’ size for n114 operation allows high UL power.

Observation 2-4: It is expected that the A-MPR will satisfy the following relationship for ‘max allocations’:
A-MPR = A-MPR_full allocation + 10·log₁₀(full allocation / LCRB)

where LCRB refers to the RB length associated with the selected n114 maximum allocation

Proposal 2-1: For each UE bandwidth of interest, select a maximum RB allocation and derive the back-off requirements based on the corresponding 0 dBr value.

Proposal 2-2: RAN4 to confirm assumption of reuse of n79 hardware for n114

# 4.	References

[1] RP-251887, “New WID on Introduction of NR TDD 4.9GHz Band for US Operation”, Fujitsu, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2] R4-2510412, “On adjacent spectrum protection requirements for the 4940-4990 MHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Aug. 2025

[3] R4-2510429, “Coexistence study for introduction of US 4.9GHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, 25th – 29th August, 2025

[4] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-I/section-90.210

[5] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y/section-90.1215

[6] R4-2510411, “General aspects regarding the 4940–4990 MHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Aug. 2025

[7]	R4-2514507, ‘On US 4.9 GHz UE RF requirements’, Qualcomm, RAN4 #116-Bis, Oct. 2025

[8]	R4-2515103, ‘Way Forward for [116bis][322] NR_TDD_4.9GHz_band’, Fujitsu, RAN4 #116-Bis, Oct. 2025

[9]	R4-2520262, ‘On US 4.9 GHz UE RF requirements’, Qualcomm, RAN4 #117, Nov. 2025

[10] R4-2523028, ‘Way Forward for [117][322] NR_TDD_4.9GHz_band’, Fujitsu, RAN4 #117, Nov. 2025

[11] R4-2520604, ‘Views on UE RF requirements for the US 4.9 GHz band’, Apple, RAN4 #117, Nov. 2025

[12] R4-2522107, ‘Introduction of US 4.9GHz band RF requirements’, Huawei, RAN4 #117, Nov. 2025