3GPP TSG-RAN WG4 Meeting #118	   R4-2601002

Gothenburg, Sweden, 9th – 13th Feb 2026

Agenda Item:	7.4.3.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for (e)RedCap UE enhancement

Document for:	Discussion

# Introduction

In the last meeting, RAN4 further discussed R20 RRM phase 6 WI according to the agreed WID shown below. The approved WF is captured in [1]. In this paper, we discuss and provide our views on the suggested RRM requirements for RedCap UE.

- Specify the following requirements for (e)RedCap Ues [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx €RedCap Ues and HD-FDD

# Discussion

## 2.1 Relaxed RLM/BFD RRM requirements

In NR, existing Rel-17 RLM/BFD relaxation for power saving has been specified only for non-RedCap UEs in FR1 and FR2. In this WI, it is agreed to discuss similar relaxation for (e)RedCap UEs. In our view, the relaxed RLM/BFD requirements for (e)RedCap can discussed taking into account the following aspects

- 2Rx (e)RedCap UE

- 1Rx (e)RedCap UE

In RAN4#116bis meeting, RAN4 agreed to re-use the existing (non-RedCap) R17 RLM/BFD relaxation requirements to 2Rx (e)RedCap UE for both FR1 and FR2, and for full-duplex FDD and TDD, as shown below.

Issue 2-2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE Agreement: - Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD.

On the other hand, the RLM/BFD relaxation for 1Rx (e)RedCap UE is still under discussion.

The difference between 1Rx and 2Rx RedCap UE from the legacy RLM/BFD requirements is that the evaluation requirement TEvaluate_out_SSB,RedCap for 1Rx RedCap UE is extended (doubled) compared to 2Rx RedCap UE (as shown below), compensating for the diversity gain.

Observation 1: The RLM/BFD evaluation requirement TEvaluate_out_SSB,RedCap for 1Rx RedCap UE is extended (doubled) compared to 2Rx RedCap UE

Therefore, relaxing RLM/BFD requirements for 1Rx RedCap UE may result in even longer delay requirements. However, RAN4 can limit this relaxation by setting up an upper bound such as 2400ms for a given condition of Max(TDRX, TSSB) as shown in the Table 1 below

Table 1: RLM/BFD relaxation for 1Rx (e)RedCap

| Condition | TEvaluate_out_SSB_Relax (ms) | K1’ | Max delay (ms) |
| --- | --- | --- | --- |
| Max(TDRX,TSSB) ≤20 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 4 | 30*4*20=2400 |
| 20ms< Max(TDRX,TSSB) ≤40 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 2 | 30*2*40=2400 |
| 40ms< Max(TDRX,TSSB) ≤160 ms | Ceil(30 P) Max(TDRX,TSSB) | No relax | 30*160 = 4800 |

Another option can be also considered is by introducing a finer relaxation step for a finer condition of Max(TDRX, TSSB) as show in the Table 2 below

Table 2: RLM/BFD relaxation for 1Rx (e)RedCap

| Condition | TEvaluate_out_SSB_Relax (ms) | K1’ | Max delay (ms) |
| --- | --- | --- | --- |
| Max(TDRX,TSSB) ≤20 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 4 | 30*4*20=2400 |
| 20ms< Max(TDRX,TSSB) ≤40 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 3 | 30*3*40=3600 |
| 40ms< Max(TDRX,TSSB) ≤60 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 2 | 30*2*60 = 3600 |
| 60ms< Max(TDRX,TSSB) ≤160 ms | Ceil(30 P) Max(TDRX,TSSB) | No relax | 30*160 = 4800 |

K1 = 4 for Max(TDRX, TSSB) ≤40ms and K1 = 2 for 40ms<Max(TDRX, TSSB) ≤80ms

Proposal 1: Relaxed RLM/BFD for 1Rx (e)RedCap UE based on Option1 or Option2:

- Option1: K1= 4 for Max(TDRX, TSSB) ≤20ms and K1= 2 for 20ms<Max(TDRX, TSSB) ≤40ms

| Condition | TEvaluate_out_SSB_Relax (ms) | K1 | Max delay (ms) |
| --- | --- | --- | --- |
| Max(TDRX,TSSB) ≤20 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 4 | 30*4*20=2400 |
| 20ms< Max(TDRX,TSSB) ≤40 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 2 | 30*2*40=2400 |
| 40ms< Max(TDRX,TSSB) ≤160 ms | Ceil(30 P) Max(TDRX,TSSB) | No relax | 30*160 = 4800 |

- Option2: K1= 4 for Max(TDRX, TSSB) ≤20ms, K1= 3 for 20ms<Max(TDRX, TSSB) ≤40ms, and K1= 2 for 40ms<Max(TDRX, TSSB) ≤60ms

| Condition | TEvaluate_out_SSB_Relax (ms) | K1’ | Max delay (ms) |
| --- | --- | --- | --- |
| Max(TDRX,TSSB) ≤20 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 4 | 30*4*20=2400 |
| 20ms< Max(TDRX,TSSB) ≤40 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 3 | 30*3*40=3600 |
| 40ms< Max(TDRX,TSSB) ≤60 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 2 | 30*2*60 = 3600 |
| 60ms< Max(TDRX,TSSB) ≤160 ms | Ceil(30 P) Max(TDRX,TSSB) | No relax | 30*160 = 4800 |

## 2.2 Support of less than 5 MHz (LT5) channel bandwidth

In the last meeting, RAN4 discussed the following issue for LT5 [1].

Issue 1-1-3: Soft combining receiver for 1Rx (e)RedCap UE Proposals - Option 1: Soft combining for 1 Rx UE should be investigated via simulations. - Option 2: Not consider soft combining

Soft combining was discussed intensively before in Rel-17 and it was never agreed. In addition, this feature is targeting a RedCap UE, i.e., cheaper UE with less capabilities compared to non-RedCap, which is more challenging to apply soft combining.

Proposal 2: Not to consider soft combining receiver for 1Rx (e)RedCap UE

# Summary

In this contribution, the following observations and proposals are concluded:

Observation 1: The RLM/BFD evaluation requirement TEvaluate_out_SSB,RedCap for 1Rx RedCap UE is extended (doubled) compared to 2Rx RedCap UE

Proposal 1: Relaxed RLM/BFD for 1Rx (e)RedCap UE based on Option1 or Option2:

- Option1: K1= 4 for Max(TDRX, TSSB) ≤20ms and K1= 2 for 20ms<Max(TDRX, TSSB) ≤40ms

| Condition | TEvaluate_out_SSB_Relax (ms) | K1 | Max delay (ms) |
| --- | --- | --- | --- |
| Max(TDRX,TSSB) ≤20 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 4 | 30*4*20=2400 |
| 20ms< Max(TDRX,TSSB) ≤40 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 2 | 30*2*40=2400 |
| 40ms< Max(TDRX,TSSB) ≤160 ms | Ceil(30 P) Max(TDRX,TSSB) | No relax | 30*160 = 4800 |

- Option2: K1= 4 for Max(TDRX, TSSB) ≤20ms, K1= 3 for 20ms<Max(TDRX, TSSB) ≤40ms, and K1= 2 for 40ms<Max(TDRX, TSSB) ≤60ms

| Condition | TEvaluate_out_SSB_Relax (ms) | K1’ | Max delay (ms) |
| --- | --- | --- | --- |
| Max(TDRX,TSSB) ≤20 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 4 | 30*4*20=2400 |
| 20ms< Max(TDRX,TSSB) ≤40 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 3 | 30*3*40=3600 |
| 40ms< Max(TDRX,TSSB) ≤60 ms | Ceil(30 K1’ P) Max(TDRX,TSSB) | 2 | 30*2*60 = 3600 |
| 60ms< Max(TDRX,TSSB) ≤160 ms | Ceil(30 P) Max(TDRX,TSSB) | No relax | 30*160 = 4800 |

Proposal 2: Not to consider soft combining receiver for 1Rx (e)RedCap UE

# References

- R4-2522652, WF on Rel-20 NR_RRM_Ph6, Ericsson.