3GPP TSG-RAN WG4 Meeting #118	R4-2601596

Gothenburg, Sweden, 9th – 13th February, 2026

Agenda item:		6.13.2.2

Source:	Nokia

Title:	On existing BS RF TX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#117 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the BS RF requirements for SBFD operation. The core part agreements are needed to be implemented to the performance part of the specification. Below is Nokia’s view of the existing BS RF TX requirements for SBFD operation.

# Discussion

## RF channels to be tested for configuration-flexible SBFD capable BS

The following was agreed during RAN4#117:

Agreement: - For configuration-flexible SBFD capable BS, FFS on below options - - Option 1: For conformance testing of SBFD slots, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed. - Option 2: Reuse existing RF channels for legacy requirements. - Option 3: Other options are not precluded

For the options mentioned above, it can be seen that those are not completely exclusive. Option 2 should be clearly understood and agreed to be applied to legacy requirements. And for SBFD operation to be consistent with legacy, option 1 should be adopted. In order to make sure that configuration-flexible SBFD-capable BS will fulfill the requirements, all RF channels need to be tested.

For conformance testing of SBFD slots, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

## Test signal configuration for FR1 and FR2-1

The following was agreed in RAN4#117:

Agreement: - For the test signal SCS configuration, FFS. - Adopt the below test signal configuration: SBFD-capable BS type; Configuration-limited. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; Option1: [30] kHz for FR1 [120] kHz for FR2-1 Option 2: Smallest supported subcarrier spacing. Subband configuration; All declared subband configurations. SBFD-capable BS type; Configuration-flexible. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; Option 1: [30] kHz for FR1 [120] kHz for FR2-1 Option 2: Smallest supported subcarrier spacing declared per operating band. Subband configuration; DUD; DU/UD. - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL,1/2.; - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL..

It was agreed that the test signal SCS configurations needs some further study. Currently there are two options listed in the WF:

- Option 1 with SCS of 30 kHz for FR1 and 120 kHz for FR2-1.

- Option 2 with smallest supported subcarrier spacing

Option 2 is following the existing legacy approach and option 1 is the approach maybe more suitable for SBFD-capable BS as there is agreement on the minimum channel bandwidth. 15 kHz SCS is not supported by all FR1 channel bandwidths that are applicable to SBFD operation whereas 30 kHz SCS is supported by all FR1 channel bandwidths that are applicable to SBFD operation. In that sense, we have a slight preference towards option 1. Similar reasoning applies also to FR2-1.

Adopt option 1 with 30 kHz SCS for FR1 and 120 kHz SCS for FR2-1.

# Conclusion

In this contribution, our views on the existing SBFD BS RF Tx requirements are presented. Our observations are listed below.

- For conformance testing of SBFD slots, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

- Adopt option 1 with 30 kHz SCS for FR1 and 120 kHz SCS for FR2-1.

# References

- R4-2523099 “Way Forward for SBFD BS conformance”, RAN4#117, Dallas, USA, 17th – 21st November, 2025.