3GPP TSG-RAN WG4 Meeting #118		R4-2602155
Gothenburg, Sweden, Feb. 9-13, 2026

Agenda item:			4.1.1

Source:	Moderator (Spreadtrum)

Title:	Topic summary for [118][113] R19_UERF_maintenance

Document for:	Information

Introduction

This is the summary for R19_UERF_maintenance under agenda 4.2 and except 4.2.7.

List of topics below:

- Discussion papers and corresponding CRs (26)

  - Draft CRs for 38.101-1 (7)

  - Draft CRs for 38.101-3 (2)

  - Draft CRs for 38.101-5 (2)

  - Draft CRs for 36.101 (2)

  - Draft CRs for 38.104 (1)

  - Draft CRs for 38.106 (1)

  - Draft CRs for 38.115-2 (1)

  - Draft CRs for 38.176-2 (1)

  - Draft CRs for 38.307 (1)

  - Draft CRs for 38.719-03-01 (1)

  - Draft CRs for 38.863 (2)

  - Others (5)

Topic #1: R19 spectrum related WI maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2600126 | Apple | CR for TR 38.863 corrections for the NTN NR band n252 NS flag references |
| R4-2600127 | Apple, Qualcomm | CR for TS 38.101-5 correction for the NTN NR L-band emission limits and A-MPR values |
| R4-2600128 | Apple | CR for TR 38.863 corrections for the NTN NR L-band NS flag references |
| R4-2600139 | Apple | CR for TS 36.101 bug fix: introduce uplink configurations for downlink fallbacks |
| R4-2600140 | Apple | CR for TS 38.101-1 bug fix: Correcting notation bugs and adding fallbacks (DL and/or UL) of combinations that have been introduced without the required fallbacks DL fallbacks : - CA_n40A-n41B (Parent: CA_n40A-n41B-n77A) Some ULs of these DL fallbacks are missing: - CA_n7A-n78A-n79A (Parent: CA_n1A-n7A-n78A-n79A, CA_n7A-n40A-n78A-n79A) - CA_n1A-n7A-n79A (Parent: CA_n1A-n7A-n78A-n79A) - CA_n7A-n40A-n79A (Parent: CA_n7A-n8A-n40A-n79A, CA_n7A-n28A-n40A-n79A, CA_n7A-n40A-n78A-n79A) - CA_n7A-n25A-n71A (Parent: CA_n7A-n25A-n66A-n71A, CA_n7A-n25A-n71A-n77A) - CA_n1A-n7A-n40A-n79A (Parent: CA_n1A-n7A-n40A-n78A-n79A) |
| R4-2600141 | Apple | CR for TS 38.101-3 bug fix: Correcting notation bugs and adding fallbacks or UL fallbacks of combinations that have been introduced without the required fallbacks DL fallbacks : - DC_1A-28A_n41A (Parent: DC_1A-28A-32A_n41A) - DC_1A-32A_n40A (Parent: DC_1A-32A_n40A-n41A) - DC_28A-32A_n40A (Parent: DC_28A-32A_n40A-n41A) - DC_1A-28A-32A_n40A (Parent: DC_1A-28A-32A_n40A-n41A) - DC_8A-38A_n1A-n78A (Parent: DC_3A-8A-38A_n1A-n78A) - DC_3A-38A_n1A-n78A (Parent: DC_3A-8A-38A_n1A-n78A) - DC_20A-38A_n1A-n78A (Parent: DC_8A-20A-38A_n1A-n78A) - DC_8A-20A-38A_n1A-n28A (Parent: DC_3A-8A-20A-38A_n1A-n28A) - DC_3A-8A-38A_n1A-n28A (Parent: DC_3A-8A-20A-38A_n1A-n28A) - DC_n77(2A)-n261A/G/H (Parent: DC_n25A-n77(2A)-n261A/G/H) Some ULs of these DL fallbacks are missing: - CA_n1A-n78A-n257H (Parent: CA_n1A-n78A-n257I) - CA_n40A-n77A-n257L (Parent: CA_n40A-n77A-n257M, CA_n40A-n77C-n257L, CA_n40B-n77A-n257L) - CA_n1A-n78A-n79A-n257H (Parent: CA_n1A-n78A-n257H) - CA_n28A-n41A-n79A-n257A (Parent: CA_n28A-n41A-n79A-n257G) - CA_n1A-n3A-n77A-n257I (Parent: CA_n1A-n3A-n28A-n77A-n257I, CA_n1A-n3A-n41A-n77A-n257I, CA_n1A-n3A-n77A-n79A-n257I) - CA_n28A-n41A-n79A-n257A (Parent: CA_n1A-n28A-n41A-n79A-n257A, CA_n3A-n28A-n41A-n79A-n257A) - CA_n3A-n28A-n77(2A)-n257A/G/H (Parent: CA_n3A-n28A-n77(2A)-n79A-n257A/G/H) - CA_n3A-n28A-n77A-n79A-n257I (Parent: CA_n3A-n28A-n77(2A)-n79A-n257I) - CA_n3A-n28A-n77(2A)-n257I (Parent: CA_n3A-n28A-n77(2A)-n79A-n257I) - CA_n28A-n41A-n79A-n257A (Parent: CA_n28A-n41A-n77A-n79A-n257A) |
| R4-2600142 | Apple | Proposal 1: In Table 7.3A.6-1 and for CA_n29-n71, consider adopting the Band n29 PC3 MSD changes highlighted in yellow for MSD due to cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n29; 685.5; 25; 15; 20 (RBstart=113); 719.5; 5; 39.56,9; ACLR1. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 9: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination. Proposal 2: In Table 7.3A.6-1a-1 and for CA_n29-n71, consider adopting the Band n29 PC2 MSD changes highlighted in yellow for MSD due to cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n29; 685.5; 25; 15; 20 (RBstart=113); 719.5; 5; 42.56,8 45.57,8; ACLR1. NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UEs supporting PC2 with 1Tx NOTE 7: Applicable to UEs supporting PC2 with 2Tx NOTE 8: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination. |
| R4-2600291 | NTT DOCOMO, INC. | N.A |
| R4-2600305 | NTT DOCOMO, INC. | N.A |
| R4-2600310 | NTT DOCOMO, INC. | N.A |
| R4-2600336 | CATT | CR for TS 38.101-1 to void the Note 6 in Table 5.3.5-1 According to the RP-250958, NOTE 6 was replaced by NOTE 12 |
| R4-2600337 | CATT | CR for TS 38.104 to void the NOTE 1 in table 5.3.5-1 According to the RP-250958, NOTE 6 was replaced by NOTE 12 |
| R4-2600339 | CATT | CR for TS 38.307 to introduce NR bands n110 CR R4-2502824 was agreed at RAN4#114 meeting and approved in CR pack RP-250627 at RAN#107. But the changes in R4-2502824 failed to be implemented into TS 38.307. This CR is the resubmission of R4-2502824. |
| R4-2600582 | Qualcomm Technologies | Main message of this draft LS: RAN4 kindly requests RAN2 to take the following actions: - Define signaling to enable UE Power Class 1 for inter-band UL CA for both NR and NR-DC configurations. - Enable PC1 support for inter-band UL CA for both NR and NR-DC from Release-17 |
| R4-2600583 | Qualcomm Technologies | Proposal 1: Send LS to RAN2 informing them that PC1 for inter-band CA should be enabled for both NR and NR-DC band combinations. Proposal 2: Enable PC1 support for inter-band UL CA for both NR and NR-DC from Release-17. |
| R4-2600584 | Viasat | CR for TS 38.101-5 updates related to combined L-band |
| R4-2600597 | ViaSat Satellite Holdings Ltd | Observation 1: EN 301 681 recommends use of Table 3a for compliance for newer MESs that support n251 and n255. Observation 2: EN 301 681 requires use of Table 3a (and not Table 3) when the UE supports both sub-bands, i.e., any combination of the following combinations: (n250/b250, n251/n251), (n250/b250, n255/b255), (n251/b251, n253/b253) or (n255/b255, n253/b253). Observation 3: For combined L-Band, i.e., n250 and n251, Table 3a should have been used to specify the potential A-MPR. Observation 4: ETSI TC SES LS requests 3GPP to consider Table 3a for UE supporting in operations in 1626.5 – 1660.5 MHz as well as 1668 – 1675 MHz. Observation 5: Table 3 has more stringent limits compared to Table 3a for all channel bandwidths compared to the general 3GPP SEM. Observation 6: Use of stringent limits in Table 3 requires significant additional power back-off for all channel bandwidths. Observation 7: Continued use of Table 3, against the recommendation of EN 301 681, will result in higher additional power back-off. Proposal 1: To align with EN 301 681 recommendation, update NS_10N emission table in TS 38.101-5 (Table 6.3.3.3.9-1) with changes (highlighted in blue) as shown in Table below and update the associated A-MPR values as part of Rel-19 maintenance work related to n251 at the RAN4 next meeting. Frequency range (MHz); Spectrum emission limit1 (dBm); Measurement bandwidth; NOTE (measurement method). 30 to 1000; -36; 100 kHz; Peak hold. 1000 to 1559; -31; 1 MHz; Average. 1559 to 1605; -40; 1 MHz; Average. 1605 to 1612.5; -40 to -28.5; 1 MHz; Average. 1612.5 to 1616.5; -25 to -20; 1 MHz; Average. 1616.5 to 1621.5; -20 to -16; 1 MHz; Average. 1621.5 to 1624.5; -30 to -27.5; 30 kHz; Average. 1624.5 to 1625.125; -27.5 to -27.2; 30 kHz; Average. 1625.125 to 1625.8; -27.2 to -20; 30 kHz; Average. 1625.8 to 1626; -20 to -17; 30 kHz; Average. 1626 to 1626.2; -17 to -10; 30 kHz; Average. 1626.2 to 1626.5; -10; 30 kHz; Average. 1660.5 to 1660.525; 30 to 15; 3 kHz; Average. 1660.525 to 1660.625; 15 to - 20; 3 kHz; Average. 1660.625 to 1660.925; -20; 3 kHz; Average. 1660.925 to 1662; -20 to -35; 3 kHz; Average. 1662 to 1677; -25; 30 kHz; Average. 1677 to 1680; -30; 30 kHz; Average. 1680 to 1685; -30; 100 kHz; Average. 1685 to 1695; -30; 300 kHz; Average. 1695 to 1705; -30; 1 MHz; Average. 1705 to 2250; -30; 3 MHz; Average. 2250 to 12750 (Note 4); -30; 3 MHz; Peak hold. NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz NOTE 4: In the bands 3 253.0 MHz to 3 321.0 MHz and 3 336.0 MHz to 3 350.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -8 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 4 879.5 MHz to 4 981.5 MHz and 5 004.0 MHz to 5 025.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 6 506.0 MHz to 6 642.0 MHz and 6 672.0 MHz to 6 700.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 8 132.5 MHz to 8 302.5 MHz and 8 340.0 MHz to 8 375.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied.. |
| R4-2600647 | NTT DOCOMO, INC. | CR for TS 38.101-1 Correction of the DL Fc on MSD table for PC1.5 CA_n1-n78 |
| R4-2601045 | ZTE Corporation, Sanechips | CR for TS 38.101-3 to correct the test frequency for DC_8_n40-n71 and notes for DC_2_n77 |
| R4-2601046 | ZTE Corporation, Sanechips | CR for TR 38.719-03-01 to correct UL transmission bandwidth symbol LCRB in MSD tables |
| R4-2601129 | Ericsson | CR for TS 36.101 correcting LTE CA configuration: Correcting so that CA_4A-4A-7A refers to CA_4A-4A Moving 71 to correct column for CA_40A-71A Correcting rows between CA_1A-1A-3C-38A and CA_1A-3A-3A-38A Correcting rows between CA_7A-32A-46E and CA_7A-38C-66A Correcting rows between CA_3A-3A-7A-8B and CA_3A-3A-7A-7A-8A |
| R4-2601499 | Rakuten Mobile, Inc | CR for TS 38.101-1 for n28 3MHz UL/5MHz DL Asymmetric BW change from Optional to Mandatory |
| R4-2601670 | Nokia, BT Plc | CR to TS 38.101-1 Corrections to BT band combinations with band n1, n3, n7, n28, n75 and n78 |
| R4-2601863 | Rakuten Mobile, Inc | Proposal 1: Rakuten Mobile request RAN4 to approve CR R4-2601499 to define the n28 asymmetric 3 MHz UL / 5 MHz DL case under BCS0 in TS 38.101-1 Table 5.3.6-1, so that UE support becomes mandatory in Release-19 through the existing asymmetric BCS0 rule in TS 38.306. |
| R4-2602028 | T-Mobile USA | CR for TS 38.101-1 Corrections for HPUE notes Adds the superscript notes 3,5 for n41 and n77 for downlink CA_n25(2A)-n41A-n66A-n71A-n77A, and corrects 3,4 to 3,5 in several other places |
| R4-2602029 | T-Mobile USA | CR for TS 38.101-1 Corrections for HPUE MOP table Adds 29 dBm for CA_n25A-n41A to Table 6.2A.1.3-1 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: PC1 in inter-band CA

Issue 1-1-1: Signaling support for PC1 in inter-band CA and Release independence

- Proposals

  - Proposal 1: Send LS to RAN2 informing them that PC1 for inter-band CA should be enabled for both NR and NR-DC (Qualcomn).

  - Proposal 2: Enable PC1 support for inter-band CA for both NR and NR-DC from Release-17(Qualcomn).

- Recommended WF

  - Check whether proposals 1 and 2 are agreeable

Issue 1-1-2: LS to RAN2 on the signalling(Qualcomn)

ACTION: RAN4 kindly requests RAN2 to take the following actions: - Define signaling to enable UE Power Class 1 for inter-band UL CA for both NR and NR-DC configurations. - Enable PC1 support for inter-band UL CA for both NR and NR-DC from Release-17

- Recommended WF

  - Check whether send and revise the LS R4-2600582 based on agreements in issue 1-1-1.

Sub-topic 1-2: Correcting emission requirements for NS_10N for NR NTN band n251

Issue 1-2-1: Correcting emission requirements for NS_10N for NR NTN band n251

- Proposals

  - Proposal 1: To align with EN 301 681 recommendation, update NS_10N emission table in TS 38.101-5 (Table 6.5.3.3.9-1) with changes (highlighted in blue) as shown in Table below and update the associated A-MPR values as part of Rel-19 maintenance work related to n251 at the RAN4 next meeting (Viasat).

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 (Note 4) | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz NOTE 4: In the bands 3 253.0 MHz to 3 321.0 MHz and 3 336.0 MHz to 3 350.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -8 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 4 879.5 MHz to 4 981.5 MHz and 5 004.0 MHz to 5 025.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 6 506.0 MHz to 6 642.0 MHz and 6 672.0 MHz to 6 700.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. In the bands 8 132.5 MHz to 8 302.5 MHz and 8 340.0 MHz to 8 375.0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -18 dBm. Elsewhere in this band the power limit in this table shall be applied. |  |  |  |

- Recommended WF

  - Firstly, check proposal 1 for TS 38.101-5 (Table 6.5.3.3.9-1) with changes (highlighted in blue) can be agreeable, then discuss whether to update AMPR in RAN4#118bis meeting.

Sub-topic 1-3: Change the n28 3 MHz UL/5MHz DL Asymmetric BW from optional to mandatory in Release 19

Sub-topic description

In RAN4 #117 meeting, CR (R4-2522167) to 38.101 for 3MHz BW change from optional to Mandatory is postpone due to the flags by other companies. There is no consensus and agreement for this issue.

Issue 1-3-1: Change the n28 3 MHz UL/5MHz DL Asymmetric BW from optional (BCS1) to mandatory (BCS0) in Release 19

- Proposals

  - Proposal 1: Rakuten Mobile request RAN4 to approve CR R4-2601499 to define the n28 asymmetric 3 MHz UL / 5 MHz DL case under BCS0 in TS 38.101-1 Table 5.3.6-1, so that UE support becomes mandatory in Release-19 through the existing asymmetric BCS0 rule in TS 38.306 (Rakuten Mobile).

- Recommended WF

  - Firstly, to check whether companies can accept 3MHz for n28 from optional to mandatory in Release 19.

Related CRs

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2601499 (R19 38.101-1) | Rakuten Mobile, Inc |  |  |

Sub-topic 1-4: n71 with UL CBW 25MHz

Sub-topic description

In RAN4 #117 meeting, CR was agreed (R4-2522388) for PC3 and PC2 MSD CA_n29-n71 with UL 25MHz CBW at n71. Therefore, there is no need to discuss Apple (R4-2600142) in this meeting.

CRs for TS 38.101-1 (6)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600140 | Apple |  |  |
| R4-2600336 | CATT |  |  |
| R4-2600647 | NTT DOCOMO, INC |  |  |
| R4-2601670 | Nokia, BT Plc |  |  |
| R4-2602028 | T-Mobile USA |  |  |
| R4-2602029 | T-Mobile USA |  |  |

CRs for TS 38.101-3 (2)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600141 | Apple |  |  |
| R4-2601045 | ZTE Corporation, Sanechips |  |  |

CRs for TS 38.101-5 (2)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600127 | Apple,Qualcomm |  |  |
| R4-2600584 | Viasat |  |  |

CRs for TS 36.101 (2)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600139 | Apple |  |  |
| R4-2601129 | Ericsson |  |  |

CRs for TS 38.104 (1)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600337 | CATT |  |  |

CRs for TS 38.307 (1)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600339 | CATT |  |  |

CRs for TR 38.719-03-01 (1)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2601046 | ZTE Corporation, Sanechips |  |  |

CRs for TR 38.863 (2)

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2600126 | Apple |  |  |
| R4-2600128 | Apple |  |  |