3GPP TSG RAN WG4 Meeting #64                                      	      R4-124664

Qingdao, China

August 13th-17th, 2012

Agenda Item:	5.1.2

Source:	Motorola Mobility

Title:	MPR for CA_1C coexistence with Band 34 and PHS

Document for:   Discussion

# Introduction

Simulation results are presented of the MPR needed for CA_1C UE-to-UE co-existence with Band 34 and with the revised PHS frequency range 1884.5 – 1915.7 MHz.  In this contribution, only contiguous allocations are considered.

# Protection Requirements

When NS_01 is signalled for CA_1C, the following protection limits apply:

- Band 34 (2010 – 2025 MHz) is protected at a level of -50 dBm/MHz

- PHS (1884.5 – 1915.7 MHz)  is protected at a level of -41 dBm/300 kHz

It is assumed that the Band 1 duplexer has a selectivity of 11 dB with respect to Band 34, so that the PA is only required to meet an emissions limit of -39 dBm/MHz for Band 34.  No duplexer selectivity is assumed for the PHS frequency range.

The following ACLR limits were applied:

- UTRAACLR1 = 33dB

- UTRAACLR2 = 36dB

- CA E-UTRAACRL = 30dB

As Band 1 is above 1 GHz, the spurious emissions limit applied was -30dBm/MHz.

# Simulations

## Assumptions

Consistent with the requirements in TS 36.101 for bands above 1 GHz, the following modulator impairments were used:

- Modulator IQ – image = 25 dB

- Modulator carrier leakage = 25 dBc

- Modulator C_IM3 = 60 dBc

The PA operating point was defined using a fully allocated 20 MHz (100 RB) LTE signal that is QPSK modulated.  The ouput power at which UTRAACLR1 = 33 dBc is defined as Pout = 22 dBm.  The MPR is defined relative to 23 dBm.

Simulation results are provided for two scenarios:

- 20 + 20 MHz with the component carriers centered at 1950 and 1970 MHz, respectively.

- 15 + 15 MHz with the component carriers centered at 1947.5 and 1962.5 MHz, respectively.

For both scenarios, simulation results are presented for both QPSK and 16QAM modulation.

## CA_1C 20+20 MHz

In Figures 1 and 2, the MPR needed for co-existence with both Band 34 and PHS is shown for CA_1C for two adjacent 20 MHz component carriers centered at 1950 and 1970 MHz, respectively, when using contiguous allocations.  The results in Figures 1 and 2 are for QPSK and 16QAM modulation, respectively.

Figure 1:  MPR for CA-1C 20 MHz + 20 MHz at 1960 MHz with QPSK

## CA_1C 15+15 MHz centered at 1955 MHz

In Figures 3 and 4, the MPR needed for co-existence with both Band 34 and PHS is shown for CA_1C for two adjacent 15 MHz component carriers centered at 1947.5 and 1962.5 MHz, respectively, when using contiguous allocations.  The results in Figures 1 and 2 are for QPSK and 16QAM modulation, respectively.

Figure 2:  MPR for CA-1C 20 + 20 MHz at 1960 MHz with 16QAM

Figure 3:  MPR for CA-1C 15 + 15 MHz at 1955 MHz with QPSK

Figure 4:  MPR for CA-1C 15 + 15 MHz at 1955 MHz with 16 QAM

# Conclusion

In this contribution, simulation results are presented indicating the MPR needed for CA_1C aggregation of 20+20 MHz and 15+15 MHz in order to protect both Band 34 and PHS.  It is recommended that these simulation results be considered when setting the allowed MPR for CA_1C with contiguous allocations.

# References

- R4-120440, CA_1C coexistence with PHS and Band 34, Nokia Corporation

- R4-122805 CA_1C coexistence with PHS and Band 34, Nokia Corporation