TSG-RAN Working Group 4 (Radio) meeting #66	R4-130168

Malta, 28 January - 1 February 2013

Source:	TruePosition

Title:	UTDOA Simulation Results (TS 36.111 Section 5 LMU RF Req)

Agenda item:	6.9.1

Document for:	Discussion

1	Introduction

During RAN 4 #64 in Qingdao, China, a way forward was agreed upon that included running UE-LMU link level simulations for the LMU’s RF characteristics.  This contribution shows the results of those simulations[4].

2	Discussion

The purpose of having requirements for the receiver characteristics as in section 5 of TS 36.111[1] is to ensure that the performance of the LMU RF receiver is adequate to support the successful measurements of an UE’s UL RTOA.   The following simulations have been run to investigate the LMU’s RF characteristics:

- Sensitivity

- Dynamic Range

- Adjacent channel selectivity

- Blocking characteristics

- Intermodulation

The simulation parameters are discussed in R4-130165.  For the purpose of discussion the metric chosen to illustrate the LMU’s characteristics is the 90% probability of detection with the detection threshold set to yield a 0.5% false alarm rate with noise only inputs.

2.1	Received SINR

Figure 2.1 below shows the LMU’s probability of detection versus SINR for 100 transmissions of a full-band SRS for each of the supported channel bandwidths.  The supported channel bandwidths are 1.4, 3, 5, 10, 15 and 20 MHz.  The 90% probability of detection for each of the channel bandwidths are listed in Table 2.1.

| Table 2.1 Required SINR for 90% detection rate |  |  |  |
| --- | --- | --- | --- |
| Channel BW (MHz) | Allocated RBs for SRS | Number of SRS transmissions | SINR [dB] |
| 1.4 | 4 | 100 | -21.1 |
| 3 | 12 | 100 | -24.7 |
| 5 | 24 | 100 | -26.7 |
| 10 | 48 | 100 | -29.5 |
| 15 | 72 | 100 | -31.2 |
| 20 | 96 | 100 | -32.5 |

Figure 2.1 Probability of Detection of 100 Transmissions of SRS

2.2	Reference Sensitivity

Figure 2.2 below shows the LMU’s reference sensitivity for 100 transmissions of a full-band SRS for each of the supported channel bandwidths.  Table 2.2 lists the reference sensitivity in dBm at 90% probability of detection for each of the channel bandwidths.  The values for the reference sensitivity for the supported channel bandwidths are within 1 dB of each other because the wider channel bandwidths have higher processing gain that compensates for the increase in thermal noise.

| Table 2.2 Reference Sensitivity at 90% detection rate |  |  |  |
| --- | --- | --- | --- |
| Channel BW (MHz) | Allocated RBs for SRS | Number of SRS transmissions | Reference Sensitivity (REFSENS) [dBm] |
| 1.4 | 4 | 100 | -129.6 |
| 3 | 12 | 100 | -129.4 |
| 5 | 24 | 100 | -129.2 |
| 10 | 48 | 100 | -129.0 |
| 15 | 72 | 100 | -128.8 |
| 20 | 96 | 100 | -129.1 |

Figure 2.2 Reference Sensitivity with 100 Transmissions of SRS

2.3 	Dynamic Range

Figure 2.3 below shows the LMU’s dynamic range for 100 transmissions of a full-band SRS for each of the supported channel bandwidths.  Table 2.2 lists the mean power in dBm of the wanted signal and interferer for 90% probability of detection for each of the channel bandwidths.  The input power of the wanted signal was varied while holding the mean power of the interfering signal constant at the values shown.

| Table 2.3 Dynamic Range at 90% detection rate |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel BW (MHz) | Allocated RBs for SRS | Number of SRS transmissions | Wanted signal mean power [dBm] | Interferer signal mean power [dBm] |
| 1.4 | 4 | 100 | -94.7 | -73.7 |
| 3 | 12 | 100 | -94.6 | -69.7 |
| 5 | 24 | 100 | -94.3 | -67.5 |
| 10 | 48 | 100 | -94.1 | -64.5 |
| 15 | 72 | 100 | -94.0 | -62.7 |
| 20 | 96 | 100 | -93.9 | -61.4 |

Figure 2.3 Dynamic Range with 100 Transmissions of SRS

2.4 	Adjacent channel selectivity

Figure 2.4 below shows the LMU’s adjacent channel selectivity for 100 transmissions of a full-band SRS for each of the supported channel bandwidths.  Table 2.4 lists the mean power in dBm of the wanted signal and interferer for 90% probability of detection for each of the channel bandwidths.  The input power of the wanted signal was varied while holding the mean power of the interfering signal constant at the values shown.

| Table 2.4 Adjacent Channel Selectivity at 90% detection rate |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel BW (MHz) | Allocated RBs for SRS | Number of SRS transmissions | Wanted signal mean power [dBm] | Interferer signal mean power [dBm] |
| 1.4 | 4 | 100 | -124.5 | -52 |
| 3 | 12 | 100 | -125.5 | -52 |
| 5 | 24 | 100 | -126.1 | -52 |
| 10 | 48 | 100 | -128.3 | -52 |
| 15 | 72 | 100 | -128.3 | -52 |
| 20 | 96 | 100 | -129.6 | -52 |

Figure 2.5 Adjacent Channel Selectivity with 100 Transmissions of SRS

2.5 	Blocking

2.5.1	General Blocking

With the simulation’s current assumptions [4], the results of the general blocking simulation would be identical to the results from the adjacent channel simulation.

2.5.2	Co-location with other base stations

Figure 2.5 below shows the LMU’s capability to reject downlink transmissions from co-located base stations for 100 transmissions of a full-band SRS for each of the supported channel bandwidths.  The other base stations transmissions were modelled as a CW tone that was attenuated by the LMU’s RF filter sufficiently (90 dB) to so as to not activate the LMU’s AGC.  The third-order term of the input CW which was assumed to fall within the passband of the receiver had minimum impact on the LMU’s probability of detection.  Table 2.5 lists the mean power in dBm of the wanted signal and interferer for 90% probability of detection for each of the channel bandwidths.  The input power of the wanted signal was varied while holding the mean power of the interfering signal constant at the values shown.

| Table 2.5 Co-location with other BS at 90% detection rate |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel BW (MHz) | Allocated RBs for SRS | Number of SRS transmissions | Wanted signal mean power [dBm] | Interferer signal mean power [dBm] |
| 1.4 | 4 | 100 | -128.9 | +16 |
| 3 | 12 | 100 | -129.0 | +16 |
| 5 | 24 | 100 | -128.6 | +16 |
| 10 | 48 | 100 | -129.3 | +16 |
| 15 | 72 | 100 | -129.0 | +16 |
| 20 | 96 | 100 | -129.4 | +16 |

Figure 2.5 Probability of Detection with co-located Base Stations

2.6 	Receiver Intermodulation

Figure 2.6 below shows the LMU’s third-order intermodulation results for 100 transmissions of a full-band SRS for each of the supported channel bandwidths.  Table 2.6 lists the mean power in dBm of the wanted signal and interferer for 90% probability of detection for each of the channel bandwidths.  The input power of the wanted signal was varied while holding the mean power of the interfering signal constant at the values shown.  The 90% detection points are within about 2 dB of each other and about 6 dB greater than the reference sensitivity.

Figure 2.6 Third-order Intermodulation with 100 Transmissions of SRS

| Table 2.6 Intermodulation at 90% detection rate |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel BW (MHz) | Allocated RBs for SRS | Number of SRS transmissions | Wanted signal mean power [dBm] | Interferer signal mean power [dBm] |
| 1.4 | 4 | 100 | -123.3 | -52 |
| 3 | 12 | 100 | -123.5 | -52 |
| 5 | 24 | 100 | -124.3 | -52 |
| 10 | 48 | 100 | -124.4 | -52 |
| 15 | 72 | 100 | -122.5 | -52 |
| 20 | 96 | 100 | -124.5 | -52 |

3	Summary

This contribution presents the results of simulation studies conducted

- Sensitivity

- Dynamic Range

- Adjacent channel selectivity

- Blocking characteristics

- Inter-modulation

- False alarm rate

4	References

1.	3GPP TS 36.111 “Location Measurement Unit (LMU) performance specification”

2.	3GPP TS 36.211 “Physical Channel and Modulation”

3.	3GPP TS 36.104 “Base Station (BS) radio transmission and reception”

4.	R4-130165 “UTDOA Simulation Assumptions and Metrics (TS 36.111 Section 5 LMU RF Req)”