3GPP TSG-RAN4 Meeting #66-BIS	 R4-131541

# Chicago, USA, 15-19 April, 2013

Source: 	SONY Mobile Communications Japan, Inc

Title: 	In harmonizing LTE OTA performance testing conditions

Agenda Item: 	7.3.1.2

Document for:	Discussion

# 1		Introduction R4-125719

# Some 10 operators around the world have proposed LTE terminals OTA performance requirements for their suppliers, TRP and TRS. It was recently however discovered that testing conditions specified differs among the operators for a total of 12 parameters.

Since OTA measurements in general are very time consuming to conduct, especially TRS measurements, it ought therefore to be of common interest throughout handset manufacturers as well as of network providers to minimize this burden, hence to agree on a common minimum set of testing conditions.

This document indicates parameters which could be subject for a harmonizing action.

# 2	LTE OTA performance testing conditions

| Test condition | Operator preference (ratio) | Industry counter proposal |
| --- | --- | --- |
| TRP/TRS at Free Space | Yes (9/10) | Keep requirement |
| TRP/TRS at Beside Head only | No (9/10) | Delete requirement |
| TRP/TRS at Beside Head and Hand | Yes (7/10) | Keep requirement |
| TRP/TRS at Hand only | No (6/10) | Optional |
| Individual RX-chains TRS or Gain Imbalance | Yes (8/10) | Keep requirement |
| Combined RX-chains TRS at Free Space | No (7/10) | Optional |
| Combined RX-chains TRS at Beside Head and Hand | No (9/10) | Optional |
| All bands TRS at 10 MHz BW | Yes (8/10) | Keep requirement |
| TRS at other BW:s than 10 MHz | No (7/10) | Delete requirement |
| ECC (Envelope Correlation) | Yes (6/10) | Optional |
| TRP/TRS for phones wider than 72 mm at Free Space | No (9/10) | Keep requirement |
| TRP/TRS for phones wider than 72 mm at Beside Head only | No (9/10) | Delete requirement |

# 3	Conclusions

Counter proposals on which LTE terminals OTA performance testing condition parameters to keep and to delete respectively have been given. This material will therefore serve as input to standardization of LTE OTA testing conditions.