3GPP TSG-RAN4 Meeting #67	 R4-132117

# Fukuoka, Japan, 20-24 May, 2013

Source: 	SONY Mobile Communications Japan, Inc

Title: 	E-UTRA FDD UE TRP/TRS requirement data proposal

Agenda Item: 	7.3.1

Document for:	Discussion

# 1		Introduction R4-125719

This document presents a proposal for E-ULTRA test values for relevant usage positions and is meant as a starting point in further discussions on this topic.

See also document R4-132116 “E-UTRA FDD TRP/TRS Specification proposal justification”.

# 2	TRP

| Band | TRP, dBm |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Beside Head and Hand |  |  | Free Space |  |  | Hand |  |  |
|  | Minimum |  | Recommended | Minimum |  | Recommended | Minimum |  | Recommended |
|  | Average | Min | Average | Average | Min | Average | Average | Min | Average |
| 1 | 11 | 9 | 14 | 16 | 14 | 19 | 13 | 11 | 16 |
| 2 | 10 | 8 | 13 | 16 | 14 | 19 | 12 | 10 | 15 |
| 3 | 10 | 8 | 13 | 16 | 14 | 19 | 12 | 10 | 15 |
| 4 | 10 | 8 | 13 | 16 | 14 | 19 | 12 | 10 | 15 |
| 5 | 6 | 4 | 9 | 14 | 12 | 17 | 9 | 7 | 12 |
| 7 | 9 | 7 | 12 | 15 | 13 | 18 | 9 | 7 | 12 |
| 8 | 6 | 4 | 9 | 14 | 12 | 17 | 9 | 7 | 12 |
| 13 | 5 | 3 | 8 | 14 | 12 | 17 | 8 | 6 | 11 |
| 17 | 5 | 3 | 8 | 14 | 12 | 17 | 8 | 6 | 11 |
| 20 | 6 | 4 | 9 | 14 | 12 | 17 | 9 | 7 | 12 |

3	TRS, primary receiver, (secondary receiver disabled)

| Band | TRS, dBm |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Beside Head and Hand |  |  | Free Space |  |  | Hand |  |  |
|  | Minimum |  | Recommended | Minimum |  | Recommended | Minimum |  | Recommended |
|  | Average | Min | Average | Average | Min | Average | Average | Min | Average |
| 1 | -86 | -84 | -89 | -88 | -86 | -91 | -86 | -84 | -89 |
| 2 | -85 | -83 | -88 | -88 | -86 | -91 | -85 | -83 | -88 |
| 3 | -85 | -83 | -88 | -88 | -86 | -91 | -85 | -83 | -88 |
| 4 | -85 | -83 | -88 | -88 | -86 | -91 | -85 | -83 | -88 |
| 5 | -78 | -76 | -81 | -86 | -84 | -89 | -81 | -79 | -84 |
| 7 | -84 | -82 | -87 | -86 | -84 | -89 | -84 | -82 | -87 |
| 8 | -80 | -78 | -83 | -86 | -84 | -89 | -83 | -81 | -86 |
| 13 | -77 | -75 | -80 | -87 | -85 | -90 | -80 | -78 | -83 |
| 17 | -77 | -75 | -80 | -85 | -83 | -88 | -80 | -78 | -83 |
| 20 | -80 | -78 | -83 | -86 | -84 | -89 | -83 | -81 | -86 |

# 4	TRS, secondary receiver, (primary receiver disabled)

TRS requirements for the secondary receiver are +4 dB to the figures above, i.e.: allow for 4 dB relaxed requirements.

# 5	Conclusion

Together with the document R4-132116 we have proposed testing conditions and TRP/TRS requirement figures for E-UTRA FDD UE:s.