3GPP TSG RAN WG4 Meeting #67	R4-132508

- Fukuoka, JAPAN, 20th – 24th May, 2013

- Agenda Item:	7.19

- Source:	LG Electronics

- Title:	TP for Harmonics and IMD analysis for inter-band CA_1A-5A UE with 2ULs

Document for:	Approval

- Introduction

In the RAN Plenary #59 meetings, the study for the LTE-A dual uplink inter-band CA is approved. And the skeleton of technical report 36.8xx for Rel-12 was discussed in e-mail reflectors [1].

The objective of work item on dual uplink inter-band CA is to gather the relevant background information and studies in order to address Dual Uplink Inter-band Carrier Aggregation requirements according to the CA class1 to 5.

In this contribution, we provide the harmonics and IMD analysis for CA_1A-5A UE with 2ULs.

- References

[1] R4-131929, “TR skeleton for inter-band CA with 2UL v.0.0.2,” Huawei

[2] R4-131416, “Protected Band for B1+B5 2 UP LINK CA,” LG U+

[3] TS 36.101 v11.4.0

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 Chapter 6.2.1 ***********************

- 6.2.1	Inter-region OOBE requirements for Carrier Aggregation of Band 1 and Band 5

Table 6.2.1-1: 2UL inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FUL_low – FUL_high |  |  |  |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 10 | 2110 MHz | – | 2170 MHz | 10 | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 10 | 869 MHz | – | 894 MHz | 10 |  |

      - List of specific combination issues

        - Channel bandwidths per operating band for CA

Table 6.2.1.1.1-1: 2UL inter-band CA operating bands

| E-UTRA band / channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Band | E-UTRA Bands | 1.4MHz | 3MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Bandwidth combination set |
| CA_1A-5A | 1 |  |  |  | Yes |  |  | 0 |
|  | 5 |  |  |  | Yes |  |  |  |

        - Co-existence studies for CA_1A-5A with 2ULs

For the analysis of 2UL inter-band CA 1A-5A UE, we should study and analyze the inter-modulation products and harmonics products when both Band 1 and Band 5 with 2ULs are transmitting.

The combination of Band 1 and Band 5 has harmonics frequencies that are far away from the components receive and transmit frequencies, in the UL and in the DL of the CA combination, as shown in table 6.2.1.1.2-1. So there was no harmonic issue except Band 41. In the conventional SE for UE-to-UE coexistence requirements, the 3rd harmonics of Band 5 UE already fall into B41 DL band, but exceptions for SE measurements are permitted if there is at least one individual RB within the transmission bandwidth for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW) [3].  And also, Band 41 is not defined as protected band when both Band1 and Band5 are operating.

Table 6.2.1.1.2-1: 2 UL B1 + B5 harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 824 | 849 |
| 2nd order harmonics frequency range (MHz) | 3840 to 3960 |  | 1648 to 1698 |  |
| 3rd order harmonics frequency range (MHz) | 5760 to 5940 |  | 2472 to 2547 |  |

Table 6.2.1.1.2-2 gives the inter-modulation products for CA_1A-5A UE with 2 ULs. None of the IMD products fall into the own CA_1A-5A UE receive bands.  But, the 3-tone 3rd IMD products may fall into the Band 2/20/25/27/33/39 DL bands, and also 2-tone 3rd IMD products may have impact on the Band 22/43 DL bands. So we need to study the IMD level from CA 1A-5A UE to interested operating Bands, but Band 22/43 are far away from Band1 and Band 5, so we can assume that the UE front-end filter can effectively attenuate these IMD products.

Table 6.2.1.1.2-2: 2 ULs B1 + B5 IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 824 | 849 |
| Two-tone 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency range (MHz) | 1071 to 1156 |  | 2744 to 2829 |  |
| Two-tone 3rd order IMD products | ¦2*f1_low – f2_high¦ | ¦2*f1_high – f2_low¦ | ¦2*f2_low – f1_high¦ | ¦2* f2_high – f1_low¦ |
| IMD frequency range (MHz) | 2991 to 3136 |  | 222 to 332 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 1910 to 1990 |  | 814 to 859 |  |
| Two-tone 3rd order IMD products | (2*f1_low +f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2* f2_high + f1_high) |
| IMD frequency range (MHz) | 4664 to 4809 |  | 3568 to 3678 |  |

        - ∆TIB and ∆RIB values

FFS

********************* End of text proposal to TR 36.860 Chapter 6.2.1 ************************