3GPP TSG RAN WG4 Meeting #68BIS	R4-135762

- Riga, Latvia, 7th – 11th October, 2013

- Agenda Item:	10.1

- Source:	LG Electronics

- Title:	TP for UE coexistence analysis to protect Band 34

Document for:	Approval

- Introduction

In the last RAN4 meeting in Barcelona, some interested companies would like to see the required A-MPR values to protect Band 34 when S-band UE and Band 34 UE were deployed in same region and/or country.

In this contribution, the RF simulation results are provided to protect band 34 according to variable UE coexistence requirements.

- References

- R4-132400, “TR 36.861 v0.2.0,” SKT

- R4-133145, “Coexistence with Band 34,” KT, SK Telecom, LGU+, ETRI

- TS 36.101 v12.1.0

- R4-135186, “UE coexistence analysis to protect Band 34,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 chapter 7.2.2 ***********************

7.2.2 RF simulation based approach for UE-to-UE coexistence

7.2.2.1   UE co-existence analysis to protect Band 1

The UL emission spectrum into Band1 DL was simulated for single and full RB allocations for 10, 15, and 20 MHz CBW. The simulation assumptions are as follows

  - Tx and Rx architecture : Conventional LTE rel-8/9 UE

  - Channel Bandwidth

    - 10MHz, 15MHz and 20MHz

    - No consideration of the CA scenarios

  - Modulator impairments

    - I/Q imbalance	: 25 dBc

    - Carrier leakage: 25 dBc

    - Counter IM3 : 60dBc

  - PA operating point : Pout = 22dBm when full RBs allocated in REL-8 100RB QPSK

  - Noise Floor : -140dBm/Hz at the PA output

  - Insertion loss : 3dB

  - MPR/A-MPR: same value as used in Band1

  - Measurement BW : dBm/1MHz

leakage

Figure 7.2.2.1-1(a) Transmitter leakage at 10MHz channel BW

leakage

Figure 7.2.2.1-1(b) Transmitter leakage at 15MHz channel BW

leakage

Figure 7.2.2.1-1(c) Transmitter leakage at 20MHz channel BW

Figure 7.2.2.1-1 Transmitter leakage levels at (a) 10MHz (b) 15MHz and (c) 20MHz channel BW

Figure 7.2.2.1-1 shows the S-band UE Tx leakage levels into Band1 Rx UE according to the channel bandwidth.

From the Figure 7.2.2.1-1(c), we can see that the Tx leakage level is maximum -65dBm/1MHz which is the worst case in Band 1 UE. Since this value is below -50dBm/MHz, there is no co-existence problem between S-band UE and Band 1 UE without any discussion for duplexers.

7.2.2.2   UE co-existence analysis to protect Band 34

When Band 34 and S-band are deployed in the same region, the UE-to-UE coexistence needs to be analyzed to protect Band 34 from S-band. RF simulations are performed on this issue considering 4 different protection levels from -50dBm/MHz to -20dBm/MHz with 10dB step.

For the A-MPR simulation, we assume the Duplexer filter attenuation is 0dB to protect Band 34 regardless standalone bands and superset band with Band 1. And additional simulation assumptions are as follow

- Modulation schemes : 16-QAM

- ACLR requirements for E-UTRA and UTRA in TS36.101

- General SE/SEM for E-UTRA in TS36.101.

- UE-to-UE coexistence requirements

  - General UE coexistence : -50 ~ -20dBm/MHz

Figure 7.2.2.2-1 shows the required A-MPR values according to the RB start position and the number of contiguous RB size. We can see that maximum required A-MPR value is about 31dB in (RB_Start, RB_Length = 75, 25) case.

- (RB_Start, RB_Length = 0,100) Required A-MPR= 27.65dB to protect B34 with -50dBm/MHz

- (RB_Start, RB_Length = 50,50), Required A-MPR = 29.14dB to protect B34 with -50dBm/MHz

- (RB_Start, RB_Length = 75,25), Required A-MPR = 30.64dB to protect B34 with -50dBm/MHz

- (RB_Start, RB_Length = 99,1), Required A-MPR = 19.16dB to protect B34 with -50dBm/MHz

Figure 7.2.2.2-1 Some cases of RF simulation results for S-band UE to protect Band 34 with -50dBm/MHz as a co-existence requirement

From the simulation results, we derived 3-Dimendional A-MPR simulation results according to RB start position and the number of contiguous RB size as shown in figure7.2.2.2-2.

Figure 7.2.2.2-2 RF simulation results for S-band UE to protect Band 34 with -50dBm/MHz as a co-existence requirement

And also we provided some additional simulation results to protect band 34 according to the UE coexistence requirements with -20~-40 dBm/MHz in figures 7.2.2.2-3~7.2.2.2-5.

Figure 7.2.2.2-3 RF simulation results for S-band UE to protect Band 34 with -40dBm/MHz as a co-existence requirement

Figure 7.2.2.2-4 RF simulation results for S-band UE to protect Band 34 with -30dBm/MHz as a co-existence requirement

Figure 7.2.2.2-5 RF simulation results for S-band UE to protect Band 34 with -20dBm/MHz as a co-existence requirement

********************** End of text proposal to TR 36.861 Chapter 7.2.2 ***************************