3GPP TSG-RAN4 Meeting #69	 R4-136136

# San Fransisco, USA, 11-15 Novembre, 2013

Source:           SONY Mobile Communications Japan, Inc.

Title: 	Text Proposal to spec. TS37.144 on handheld UE E-UTRA FDD TRP/TRS.

Agenda Item: 	8.2.1

Document for:	Approval

# 1.		Introduction R4-125719

The present contribution provides the text and requirement proposal for inclusion in the specification TS 37.144 chapters 5.4, 6.1.2.3, 6.1.3.3, 7.1.2.3, 7.1.3.3, B.2.1.2.3, B.2.1.3.3, B.3.1.2.3 and B.3.1.3.3 on handset E-UTRA FDD frequency bands, min and recommended TRP/TRS.

<< unchanged sections omitted >>

2.		5.4 E-UTRA frequency bands.

Table 5.4-1: E-UTRA FDD frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| 61 | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | FDD |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
| 15 | Reserved |  |  | Reserved |  |  | FDD |
| 16 | Reserved |  |  | Reserved |  |  | FDD |
| 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | FDD |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
| 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | FDD |
| 23 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| 24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD2 |
| 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
| 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz | FDD |
| NOTE 1: Band 6 is not applicable NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |

_________________________________________________________________________________

3.		6.1.2.3 Transmitter TRP Minimum Performance at Beside Head and Hand E-UTRA FDD

Handheld UE TRP minimum performance requirements for E-UTRA FDD in beside the head and hand phantom position and the primary mechanical mode are defined in Table 6.1.2.3-1.

Table 6.1.2.3-1: Handheld UE TRP minimum performance requirements for E-UTRA FDD roaming bands in beside the head and hand phantom position and at the primary mechanical mode.

| Operating band | Power class 1 | Power class 2 | Power class 3 |  | Power class 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) |  | Power (dBm) |  |
|  |  |  | Average | Min | Average | Min |
| 1 | - | - | 9 | 6 |  |  |
| 2 | - | - | 9 | 6 |  |  |
| 3 | - | - | 9 | 6 |  |  |
| 4 | - | - | 9 | 6 |  |  |
| 5 | - | - | 6 | 3 |  |  |
| 6 | - | - |  |  |  |  |
| 7 | - | - | 9 | 6 |  |  |
| 8 | - | - | 6 | 3 |  |  |
| 9 | - | - |  |  |  |  |
| 10 | - | - |  |  |  |  |
| 11 | - | - | 6 | 3 |  |  |
| 12 | - | - |  |  |  |  |
| 13 | - | - | 5 | 2 |  |  |
| 14 | - | - |  |  |  |  |
| 17 | - | - | 5 | 2 |  |  |
| 18 | - | - |  |  |  |  |
| 19 | - | - |  |  |  |  |
| 20 | - | - | 6 | 3 |  |  |
| 21 | - | - |  |  |  |  |
| 22 | - | - |  |  |  |  |
| 23 | - | - |  |  |  |  |
| 24 | - | - |  |  |  |  |
| 25 | - | - |  |  |  |  |
| 26 | - | - |  |  |  |  |
| 27 | - | - |  |  |  |  |
| 28 | - | - | 5 | 2 |  |  |
| 29 | - | - |  |  |  |  |
| 30 | - | - |  |  |  |  |
| 31 | - | - |  |  |  |  |
| NOTE: Applicable for multi-mode GSM/UMTS/LTE |  |  |  |  |  |  |

_________________________________________________________________________________________

4.	6.1.3.3 Transmitter TRP Minimum Performance at Hand Browsing E-UTRA FDD.

UE TRP minimum performance requirements for E-UTRA FDD in hand phantom browsing position and the primary mechanical mode are defined in Table 6.1.3.3-1.

Table 6.1.3.3-1: Handheld UE TRP minimum performance requirements for E-UTRA FDD roaming bands in hand phantom browsing position and at the primary mechanical mode.

| Operating band | Power class 1 | Power class 2 | Power class 3 |  | Power class 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) |  | Power (dBm) |  |
|  |  |  | Average | Min | Average | Min |
| 1 | - | - | 11 | 8 |  |  |
| 2 | - | - | 11 | 8 |  |  |
| 3 | - | - | 11 | 8 |  |  |
| 4 | - | - | 11 | 8 |  |  |
| 5 | - | - | 9 | 6 |  |  |
| 6 | - | - |  |  |  |  |
| 7 | - | - | 11 | 8 |  |  |
| 8 | - | - | 9 | 6 |  |  |
| 9 | - | - |  |  |  |  |
| 10 | - | - |  |  |  |  |
| 11 | - | - |  |  |  |  |
| 12 | - | - |  |  |  |  |
| 13 | - | - | 8 | 5 |  |  |
| 14 | - | - |  |  |  |  |
| 17 | - | - | 8 | 5 |  |  |
| 18 | - | - |  |  |  |  |
| 19 | - | - |  |  |  |  |
| 20 | - | - | 9 | 6 |  |  |
| 21 | - | - |  |  |  |  |
| 22 | - | - |  |  |  |  |
| 23 | - | - |  |  |  |  |
| 24 | - | - |  |  |  |  |
| 25 | - | - |  |  |  |  |
| 26 | - | - |  |  |  |  |
| 27 | - | - |  |  |  |  |
| 28 | - | - | 8 | 5 |  |  |
| 29 | - | - |  |  |  |  |
| 30 | - | - |  |  |  |  |
| 31 | - | - |  |  |  |  |
| NOTE: Applicable for multi-mode GSM/UMTS/LTE |  |  |  |  |  |  |

5.	7.1.2.3 Receiver TRS Minimum Performance at Beside Head and Hand E-UTRA FDD.

Handheld UE TRS minimum performance requirements for E-UTRA FDD in beside the head and hand phantom position and the primary mechanical mode are defined in Table 7.1.2.3-1.

Table 6.1.2.3-1: Handheld UE TRP minimum performance requirements for E-UTRA FDD roaming bands in beside the head and hand phantom position and at the primary mechanical mode.

| Operating band | Test conditions | Power class 1-4 |  |
| --- | --- | --- | --- |
|  |  | Power (dBm) |  |
|  |  | Average | Min |
| 1 | RX-chains combined, 10 MHz BW | -84 | -81 |
| 2 | RX-chains combined, 10 MHz BW | -84 | -81 |
| 3 | RX-chains combined, 10 MHz BW | -84 | -81 |
| 4 | RX-chains combined, 10 MHz BW | -84 | -81 |
| 5 | RX-chains combined, 10 MHz BW | -80 | -77 |
| 6 |  |  |  |
| 7 | RX-chains combined, 10 MHz BW | -84 | -81 |
| 8 | RX-chains combined, 10 MHz BW | -80 | -77 |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 | RX-chains combined, 10 MHz BW | -78 | -75 |
| 14 |  |  |  |
| 17 | RX-chains combined, 10 MHz BW | -78 | -75 |
| 18 |  |  |  |
| 19 |  |  |  |
| 20 | RX-chains combined, 10 MHz BW | -80 | -77 |
| 21 |  |  |  |
| 22 |  |  |  |
| 23 |  |  |  |
| 24 |  |  |  |
| 25 |  |  |  |
| 26 |  |  |  |
| 27 |  |  |  |
| 28 | RX-chains combined, 10 MHz BW | -78 | -75 |
| 29 |  |  |  |
| 30 |  |  |  |
| 31 |  |  |  |
| NOTE: Applicable for multi-mode GSM/UMTS/LTE |  |  |  |

6.	7.1.3.3 Receiver TRS Minimum Performance at Hand browsing E-UTRA FDD.

Handheld UE TRS minimum performance requirements for E-UTRA FDD in hand phantom browsing position and the primary mechanical mode are defined in Table 7.1.3.3-1.

Table 7.1.3.3-1: Handheld UE TRP minimum performance requirements for E-UTRA FDD roaming bands in hand phantom position and at the primary mechanical mode.

| Operating band | Test conditions | Power class 1-4 |  |
| --- | --- | --- | --- |
|  |  | Power (dBm) |  |
|  |  | Average | Min |
| 1 | RX-chains combined, 10 MHz BW | -86 | -83 |
| 2 | RX-chains combined, 10 MHz BW | -86 | -83 |
| 3 | RX-chains combined, 10 MHz BW | -86 | -83 |
| 4 | RX-chains combined, 10 MHz BW | -86 | -83 |
| 5 | RX-chains combined, 10 MHz BW | -83 | -80 |
| 6 |  |  |  |
| 7 | RX-chains combined, 10 MHz BW | -86 | -83 |
| 8 | RX-chains combined, 10 MHz BW | -83 | -80 |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 | RX-chains combined, 10 MHz BW | -81 | -78 |
| 14 |  |  |  |
| 17 | RX-chains combined, 10 MHz BW | -81 | -78 |
| 18 |  |  |  |
| 19 |  |  |  |
| 20 | RX-chains combined, 10 MHz BW | -83 | -80 |
| 21 |  |  |  |
| 22 |  |  |  |
| 23 |  |  |  |
| 24 |  |  |  |
| 25 |  |  |  |
| 26 |  |  |  |
| 27 |  |  |  |
| 28 | RX-chains combined, 10 MHz BW | -81 | -78 |
| 29 |  |  |  |
| 30 |  |  |  |
| 31 |  |  |  |
| NOTE: Applicable for multi-mode GSM/UMTS/LTE |  |  |  |

7.	B.2.1.2.3 Transmitter TRP Recommended Performance at Beside Head and Hand Position E-UTRA FDD.

UE TRP recommended performance requirements for E-UTRA FDD in head and hand phantom position and the primary mechanical mode are defined in Table B.2.1.2.3-1.

Table B.2.1.2.3-1: Handheld UE TRP recommended performance requirements for E-UTRA FDD roaming bands in beside the head and hand phantom position and at the primary mechanical mode.

| Operating band | Power class 1 | Power class 2 | Power class 3 | Power class 4 |
| --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) |
|  | Average | Average | Average | Average |
| 1 | - | - | 12 | - |
| 2 | - | - | 12 | - |
| 3 | - | - | 12 | - |
| 4 | - | - | 12 | - |
| 5 | - | - | 9 | - |
| 6 | - | - |  | - |
| 7 | - | - | 12 | - |
| 8 | - | - | 9 | - |
| 9 | - | - |  | - |
| 10 | - | - |  | - |
| 11 | - | - | 9 | - |
| 12 | - | - |  | - |
| 13 | - | - | 8 | - |
| 14 | - | - |  | - |
| 17 | - | - | 8 | - |
| 18 | - | - |  | - |
| 19 | - | - |  | - |
| 20 | - | - | 9 | - |
| 21 | - | - |  | - |
| 22 | - | - |  | - |
| 23 | - | - |  | - |
| 24 | - | - |  | - |
| 25 | - | - |  | - |
| 26 | - | - |  | - |
| 27 | - | - |  | - |
| 28 | - | - | 8 | - |
| 29 | - | - |  | - |
| 30 | - | - |  | - |
| 31 | - | - |  | - |

8.	B.2.1.3.3 Transmitter TRP Recommended Performance at Hand Browsing Position E-UTRA FDD.

UE TRP recommended performance requirements for E-UTRA FDD in hand browsing phantom position and the primary mechanical mode are defined in Table B.2.1.3.3-1.

Table B.2.1.3.3-1: Handheld UE TRP recommended performance requirements for E-UTRA FDD roaming bands in beside the head and hand phantom position and at the primary mechanical mode.

| Operating band | Power class 1 | Power class 2 | Power class 3 | Power class 4 |
| --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) |
|  | Average | Average | Average | Average |
| 1 | - | - | 14 | - |
| 2 | - | - | 14 | - |
| 3 | - | - | 14 | - |
| 4 | - | - | 14 | - |
| 5 | - | - | 12 | - |
| 6 | - | - |  | - |
| 7 | - | - | 14 | - |
| 8 | - | - | 12 | - |
| 9 | - | - |  | - |
| 10 | - | - |  | - |
| 11 | - | - | 11 | - |
| 12 | - | - |  | - |
| 13 | - | - | 11 | - |
| 14 | - | - |  | - |
| 17 | - | - | 11 | - |
| 18 | - | - |  | - |
| 19 | - | - |  | - |
| 20 | - | - | 12 | - |
| 21 | - | - |  | - |
| 22 | - | - |  | - |
| 23 | - | - |  | - |
| 24 | - | - |  | - |
| 25 | - | - |  | - |
| 26 | - | - |  | - |
| 27 | - | - |  | - |
| 28 | - | - | 11 | - |
| 29 | - | - |  | - |
| 30 | - | - |  | - |
| 31 | - | - |  | - |

9.	B.3.1.2.3 Receiver TRS Recommended Performance at Beside Head and Hand E-UTRA FDD.

Handheld UE TRS recommended performance requirements for E-UTRA FDD in beside the head and hand phantom position and the primary mechanical mode are defined in Table B.3.1.2.3-1.

Table B.3.1.2.3-1: Handheld UE TRP recommended performance requirements for E-UTRA FDD roaming bands in beside the head and hand phantom position and at the primary mechanical mode.

| Operating band | Test conditions | Power class 1-4 |
| --- | --- | --- |
|  |  | Power (dBm) |
|  |  | Average |
| 1 | RX-chains combined, 10 MHz BW | -87 |
| 2 | RX-chains combined, 10 MHz BW | -87 |
| 3 | RX-chains combined, 10 MHz BW | -87 |
| 4 | RX-chains combined, 10 MHz BW | -87 |
| 5 | RX-chains combined, 10 MHz BW | -83 |
| 6 |  |  |
| 7 | RX-chains combined, 10 MHz BW | -87 |
| 8 | RX-chains combined, 10 MHz BW | -83 |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 | RX-chains combined, 10 MHz BW | -81 |
| 14 |  |  |
| 17 | RX-chains combined, 10 MHz BW | -81 |
| 18 |  |  |
| 19 |  |  |
| 20 | RX-chains combined, 10 MHz BW | -83 |
| 21 |  |  |
| 22 |  |  |
| 23 |  |  |
| 24 |  |  |
| 25 |  |  |
| 26 |  |  |
| 27 |  |  |
| 28 | RX-chains combined, 10 MHz BW | -81 |
| 29 |  |  |
| 30 |  |  |
| 31 |  |  |

10.	B.3.1.3.3 Receiver TRS Recommended Performance at  Hand Browsing E-UTRA FDD.

Handheld UE TRS recommended performance requirements for E-UTRA FDD in hand browsing phantom position and the primary mechanical mode are defined in Table B.3.1.3.3-1.

Table B.3.1.3.3-1: Handheld UE TRP recommended performance requirements for E-UTRA FDD roaming band in hand phantom browsing position and at the primary mechanical mode.

| Operating band | Test conditions | Power class 1-4 |
| --- | --- | --- |
|  |  | Power (dBm) |
|  |  | Average |
| 1 | RX-chains combined, 10 MHz BW | -89 |
| 2 | RX-chains combined, 10 MHz BW | -89 |
| 3 | RX-chains combined, 10 MHz BW | -89 |
| 4 | RX-chains combined, 10 MHz BW | -89 |
| 5 | RX-chains combined, 10 MHz BW | -86 |
| 6 |  |  |
| 7 | RX-chains combined, 10 MHz BW | -89 |
| 8 | RX-chains combined, 10 MHz BW | -86 |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 | RX-chains combined, 10 MHz BW | -84 |
| 14 |  |  |
| 17 | RX-chains combined, 10 MHz BW | -84 |
| 18 |  |  |
| 19 |  |  |
| 20 | RX-chains combined, 10 MHz BW | -86 |
| 21 |  |  |
| 22 |  |  |
| 23 |  |  |
| 24 |  |  |
| 25 |  |  |
| 26 |  |  |
| 27 |  |  |
| 28 | RX-chains combined, 10 MHz BW | -84 |
| 29 |  |  |
| 30 |  |  |
| 31 |  |  |

<< unchanged sections omitted >>

11.	Conclusions.

A Text Proposal for handheld UE E-UTRA FDD bands TRP/TRS performance requirements of TS 37.144 has been given.

12.	References.

3GPP TS 37.144

R4-136125