3GPP TSG RAN WG4 Meeting #69	R4-136350

San Francisco, CA, U.S.A., 11th – 15th Nov., 2013

- Agenda Item:	10.1

- Source:	LG Electronics

- Title:	TP for S-band UE Tx/Rx RF requirements for the extended Band

Document for:	Approval

- Introduction

In the RAN Plenary #57 meetings, the study for the LTE S-band in Korea was approved. And the UE-to-UE coexistence simulation results to protect band 34 was captured [1] for TR 36.861 v0.4.0 in last RAN4 meeting.

In this contribution, we provide the UE transmitter/Receiver RF requirements based on the deployments scenarios assuming new extended S-band.

- References

- R4-133145, “Coexistence with Band 34,” KT, SK Telecom, LGU+, ETRI

- TS 36.101 v11.6.0

- R4-135186, “UE coexistence analysis to protect Band 34,” LG Electronics

- R4-131390, “TP for TR ab.cde : General issue for FS_LTE_1980_2170_Korea,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 Chapter 8 ***********************

- 8.1    UE Tx RF Requirements

- 8.1.1	New separate band (1980-2010 MHz and 2170-2200 MHz)

< <text will be added>.

- 8.1.2	New Extended band

- 8.1.2.2 	Spurious emission for UE co-existence

For the coexistence requirements, a single/dual duplexer approaches can be considered, but the requirements should be defined as single requirements and should be met regardless of duplexer configuration.

When RAN4 can consider dual duplexer approach to protect band 34 because dual duplexer approach has more emission rejection capability than the single duplexer approach when we consider protecting Band1 based on the current UE coexistence requirement of -50dBm/MHz. Therefore RAN4 can assume dual-duplexer filter for the extended S-bands.

For the partial extended band (1950-2010MHz and 2140-2200MHz) with Band 1, RAN4 need further study to verify duplexer filter which has more benefits according to the deployment scenarios.

When Band 34 is allocated in the same country and region where S-band UE is deployed, RAN4 can define network signalling such as NS_XX to apply A-MPR mask with guard band to protect band 34 for S-band UE. SE requirements for UE-to-UE coexistence can be defined as in Table 8.1.2.2-1. The SE requirements level and detail test methodology to protect band 34 are for further study.

Table 8.1.2.2-1: Spurious emission band UE co-existence of the extended S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | 2010 | - | 2025 | TBD | TBD | 1 |
|  |  |  | - |  |  |  |  |
| NOTE 1: This requirement is only applicable for S-band UE, when Band34 and S-band deployed in same region. |  |  |  |  |  |  |  |

- 8.1.2.3	Other expected UE Tx RF requirements

For the other Tx requirements, a single/dual duplexer approaches can be considered. In this section, the detailed descriptions on UE Tx RF requirements are proposed to describe any potential requirements changes needed for the extended S-band UE

When RAN4 decides to protect the band 34 for operating S-band UE in the same geographical area, the other expected UE transmitter RF requirement changes are estimated and summarized in Table 8.1.2.3-1.

Table 8.1.2.3-1: Expected UE Tx RF requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| UE Maximum Output Power | Additional insertion loss will be added to calculate the UE MOP due to the decreased duplex gap and increased Pass band. But for the partial extended band, not allow additional insertion loss. |
| UE Maximum Output Power with additional requirements (A-MPR) | Additional-MPR for NS_XX can be defined to protect Band 34. |
| Spurious emission/ additional Spurious emission | Additional SE can be defined to protect Band 34. |
| ACLR/SEM | No changes are needed in OOB ranges for S-band UE. |
| Additional Spectrum Emission Mask | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics [1]. |

In this case RAN4 can define a network signalling such as NS_XX to apply A-MPR mask and Guard band to protect Band 34.

  - UE Rx RF Requirements

    - New separate band (1980-2010 MHz and 2170-2200 MHz)

< <text will be added>.

    - New Extended band

- 8.2.2.1 REFSENS requirements for S-band UE

Extended band has same Tx-Rx frequency separation but total bandwidth of S-band is larger than that of Band 1. Single duplexer approach is not sufficient to protect band 1 and band 34 based on general UE coexistence requirements of -50dBm/MHz [6] in extended S-band. This is because the new filter does not guarantee the same performances compare to band1 filter due to the decreased duplex gap and increased pass band, and also there is no guard band to protect band 34.

But the partial extended band(1950-2010MHz, 2140-2200MHz) has similar band characteristics compare to Band 1. Hence, single duplexer approach also can be used for partial extended band. RF simulations are provided to verify the REFSENS requirements. However RAN4 need further study to verify the pros and cons in both extended bands.

Figure 8.2.2.1-1 show the simulation results of emission levels in S-band DL lower bands. The red line indicates the leakage level with full RBs allocation in extended S-band DL lower band.

- 20MHz S-band UE (fc=2000MHz)                      (b) 10MHz S-band UE (fc=2005MHz)

Figure 8.2.2.1-1 Simulation results of the Tx leakage level into S-band DL

The leakage level in DL band is about -65dBm/MHz, which would be further attenuated by Duplexer Tx/Rx isolation level, which is assumed to be 50dB. So the leakage level in DL band is estimated as -115dBm/MHz. Eq. 8-4 shows the impact of REFSENS on S-band DL induced by Tx leakage.

- REFSENS for S-band UE = Conventional REFSENS + Tx_leakage levels                          (Eq. 8-4)

= -94dBm/CH_BW +  -115dBm/MHz

= -107dBm/MHz + -115dBm/MHz = - 106.36dBm/MHz

= -93.36dBm/CH_BW

From Eq. 8-4, we can see that the REFSENS in own Rx band by Tx leakage is negligible.

However, the insertion loss may be increased for the single duplexer compared to conventional Band 1 Duplexer, while we can keep the same insertion loss for dual duplexer approach or single duplexer approach for partial extended band.

As a conclusion, REFSENS requirements for S-band UE can be relaxed by the factor of additional insertion loss due to the introduction of new single duplexer of the extended S-band UE. But, as shown in section 8.2.1.1, the REFSENS of Band 34 will be degraded by 36.5dB by S-band UE emissions. Therefore RAN4 should consider the network signalling message as NS_XX to apply A-MPR and guard band to protect Band 34.

- 8.2.2.2 Other UE Rx RF requirements

For the other receiver requirements for S-band UE, requirement changes are not expected since there are no critical impacts to determine the UE reception requirements.

Table 8.2.2.2-1: Expected UE Rx RF requirements for S-band UE

| Description | Comments and Comparison to 2-band Solution |
| --- | --- |
| Adjacent Channel Selectivity | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements that can be larger than or equal to 95% with same condition in section 7.5.1 [2] |
| Blocking characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.6 [2] |
| Receiver inter-modulation characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.8.1 [2] |

********************** End of text proposal to TR 36.861 Chapter 8 ***************************