3GPP TSG-RAN WG4 #69	R1-136428

San francisco, USA, 11th – 15th, Nov. 2013

Source: 	Ericsson

Title:	Link level simulation results and discussion for RI test

Agenda Item:	7.5.1.2

Document for:	Discussion

# Introduction

In RAN4#68bis meeting, the group has the following agreements [1]:

- Introduce Test 3 for FeICIC RI test;

    - Correlation matrix: high correlation for serving cell and low correlation for aggressor cells;

    - SNR test points: Es/Noc1=20dB;

  - Values of RI requirements:

      - Test 3: use γ as test metric;

        - In the next meeting decide the value.

In this paper, we provide link level simulation results for Test 3 under high correlation.

# Preliminary link level simulation for RI test

In the link level simulation, simulation parameters are shown in Table 1 of Appendix A. In 36.101, 1  and   2 are defined as:

  - The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1

  - The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2

In Figure 1, both γ1 and γ2 are provided for 2x2 high correlation channels.

Figure 1: γ1 and γ2 value for 2x2 low correlation

Based on Rel-10/Rel-11 RI test for high correlation, the requirement is γ1=0.9 as shown in Table 1.  From Figure 1, the requirements are met with the current setup.

Table 1: Minimum requirement (FDD)

|  | Test 3 |
| --- | --- |
| 1 | 0.9 |
| 2 | N/A |
| UE Category | 2-8 |

Therefore, we propose:

Proposal: γ1=0.9 can be set as the final requirements for RI test 3 when high correlation channel is used

# Conclusion

In this paper, link level simulation for RI test 3 is provided. We propose

Proposal: γ1=0.9 can be set as the final requirements for RI test 3 when high correlation channel is used

# Appendix A

Table 1: RI Test (FDD)

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2&Cell3 |  |
| Bandwidth |  | MHz | 10 |  |  |
| PDSCH transmission mode |  |  | 3 | Note 10 |  |
| Downlink power allocation |  | dB | -3 |  |  |
|  |  | dB | -3 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | 2 x 2 EPA5 |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 01 for fixed RI = 1 10 for fixed RI = 2 11 for UE reported RI | N/A |  |
| Antenna correlation |  |  | High | Low |  |
| RI configuration |  |  | Fixed RI=1 and follow RI | N/A |  |
|  |  | dB | 0 | 12 dB for cell 2 10 dB for cell 3 |  |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |  |
| Cell Id |  |  | 0 | 126 for cell 2 1 for cell 3 |  |
| Time Offset between Cells |  | s | 3 us for cell 2 -1 us for cell 3 |  |  |
| Frequency offset |  | Hz | 300 Hz for cell 2 -100 Hz for cell 3 |  |  |
| ABS Pattern (Note 6) |  |  | N/A | 10000000 10000000 10000000 10000000 10000000 |  |
| RLM/RRM Measurement Subframe Pattern (Note 7) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A |  |
| CSI Subframe Sets (Note 8) | CCSI,0 |  | 10000000 10000000 10000000 10000000 10000000 | N/A |  |
|  | CCSI,1 |  | 01111111 01111111 01111111 01111111 01111111 |  |  |
| Number of control OFDM Symbols |  |  | 2 | 2 |  |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |
| Reporting mode |  |  | PUCCH 1-0 |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type for CQI |  |  | 4 |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |
| Reporting periodicity |  | ms | Npd= 10 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 11 |  |  |
| ri-ConfigurationInd |  |  | 5 |  |  |
| cqi-pmi-ConfigurationIndex2 |  |  | 10 |  |  |
| ri-ConfigurationInd2 |  |  | 2 |  |  |
| Cyclic prefix |  |  | Normal |  | Normal |

# Reference

R4-135604, Meeting minutes for FeICIC ad hoc, Oct 2013