TSG-RAN Working Group 4 (Radio) meeting #69		R4-136492

San Francisco, US, 11-15 November 2013

Source:	ETRI

Title:	TP for TR 36.861 regarding FS_LTE_2980_2170_Korea

Agenda item:	10.1

Document for:	Approval

Introduction

In the RAN Plenary #57 meetings, the study for the LTE Mobile S-band in Korea are approved. And the skeleton of technical report 36.861 for Rel-12 was approved in RAN4 #64bis meeting.

The objective of study item on LTE FDD in the bands 1980-2010 MHz and 2170-2200 MHz is to facilitate and harmonize the efficient use of these bands for terrestrial IMT, especially LTE, for Korea.

In this contribution, we provide the TP of TR 36.861 regarding coexistence studies between a new S-band and Band 34.

References

- R4-135737, “2 GHz MSS band coexistence for Region 3”. Qualcomm Incorporated. 3GPP RAN4 #68bis

- R4-135152, “Coexistence with Band 34”. KT, LG U+ and ETRI. 3GPP RAN4 #68bis

- R4-135408, “Co-existence between MSS and band 34”. Ericsson. 3GPP RAN4 #68bis

- R4-135121, “Coexistence with Band 34 and a band of 1980-2010 for UL and 2170-2200 for DL”. NTT DOCOMO. 3GPP RAN4 #68bis

- R4-135186, “UE coexistence analysis to protect Band 34”. LG Electronics. 3GPP RAN4 #68bis

- R4-135762, “TP for UE coexistence analysis to protect Band 34”. LG Electronics. 3GPP RAN4 #68bis

- R4-135235, “Discussion on the bands 1980-2010 MHz and 2170-2200 MHz in Regions 1 and 3. NSN. 3GPP RAN4 #68bis

- R4-136490, “Coexistence between a new S-band and Band 34”. ETRI and TTA. 3GPP RAN4 #69

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 Chapter 7.2.2 **********************

7.2.2 RF simulation based approach for UE-to-UE coexistence

7.2.2.1   UE co-existence analysis to protect Band 1

The UL emission spectrum into Band1 DL was simulated for single and full RB allocations for 10, 15, and 20 MHz CBW. The simulation assumptions are as follows

  - Tx and Rx architecture : Conventional LTE rel-8/9 UE

  - Channel Bandwidth

    - 10MHz, 15MHz and 20MHz

    - No consideration of the CA scenarios

  - Modulator impairments

    - I/Q imbalance : 25 dBc

    - Carrier leakage : 25 dBc

    - Counter IM3 : 60dBc

  - PA operating point : Pout = 22dBm when full RBs allocated in REL-8 100RB QPSK

  - Noise Floor : -140dBm/Hz at the PA output

  - Insertion loss : 3dB

  - MPR/A-MPR: same value as used in Band1

  - Measurement BW : dBm/1MHz

leakage

Figure 7.2.2.1-1(a) Transmitter leakage at 10MHz channel BW

leakage

Figure 7.2.2.1-1(b) Transmitter leakage at 15MHz channel BW

leakage

Figure 7.2.2.1-1(c) Transmitter leakage at 20MHz channel BW

Figure 7.2.2.1-1 Transmitter leakage levels at (a) 10MHz (b) 15MHz and (c) 20MHz channel BW

Figure 7.2.2.1-1 shows the S-band UE Tx leakage levels into Band1 Rx UE according to the channel bandwidth.

From the Figure 7.2.2.1-1(c), we can see that the Tx leakage level is maximum -65dBm/1MHz which is the worst case in Band 1 UE. Since this value is below -50dBm/MHz, there is no co-existence problem between S-band UE and Band 1 UE without any discussion for duplexers.

7.2.2.2   UE co-existence analysis to protect Band 34

7.2.2.2.1   General case

In this section, the coexistence studies are done on the coexistence emission limits -50/-40/-30 dBm/MHz. In addition, A-MPR (with NS) or RB restriction (without NS) as a countermeasure can be adopted with possibility to impose guard band (GB). When GB needs to be defined together with A-MPR or RB restriction, 5MHz of GB is considered as a baseline.

Based on those, UE co-existence studies between the new S-band and Band 34 are summarized in Tables 7.2.2.2.1-1 and 7.2.2.2.1-2.

Table 7.2.2.2.1-1. Summary of coexistence studies with Band 34 (A-MPR case with NS)

| Case | Channel Bandwidth (MHz) | Guard Band (MHz) | Coexistence emission limit (dBm/MHz) | Maximum Required A-MPR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 20 | 0 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |
| 2 | 20 | 5 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |
| 3 | 20 | 10 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |

Table 7.2.2.2.1-2. Summary of coexistence studies with Band 34 (RB Restriction case without NS)

| Case | Channel Bandwidth (MHz) | Guard Band (MHz) | Coexistence emission limit (dBm/MHz) | Maximum Required RB Restrictions |
| --- | --- | --- | --- | --- |
| 4 | 20 | 0 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |
| 5 | 20 | 5 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |
| 6 | 20 | 10 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |

7.2.2.2.2   Detailed analysis

When Band 34 and S-band are deployed in the same region, the UE-to-UE coexistence needs to be analyzed to protect Band 34 from S-band. RF simulations are performed on this issue considering 4 different protection levels from -50dBm/MHz to -20dBm/MHz with 10dB step.

For the A-MPR simulation, we assume the Duplexer filter attenuation is 0dB to protect Band 34 regardless standalone bands and superset band with Band 1. And additional simulation assumptions are as follow

- Modulation schemes : 16-QAM

- ACLR requirements for E-UTRA and UTRA in TS36.101

- General SE/SEM for E-UTRA in TS36.101.

- UE-to-UE coexistence requirements

  - General UE coexistence : -50 ~ -20dBm/MHz

Figure 7.2.2.2-1 shows the required A-MPR values according to the RB start position and the number of contiguous RB size. We can see that maximum required A-MPR value is about 31dB in (RB_Start, RB_Length = 75, 25) case.

- (RB_Start, RB_Length = 0,100) Required A-MPR= 27.65dB to protect B34 with -50dBm/MHz

- (RB_Start, RB_Length = 50,50), Required A-MPR = 29.14dB to protect B34 with -50dBm/MHz

- (RB_Start, RB_Length = 75,25), Required A-MPR = 30.64dB to protect B34 with -50dBm/MHz

- (RB_Start, RB_Length = 99,1), Required A-MPR = 19.16dB to protect B34 with -50dBm/MHz

Figure 7.2.2.2.2-1 Some cases of RF simulation results for S-band UE to protect Band 34 with -50dBm/MHz as a co-existence requirement

From the simulation results, we derived 3-Dimendional A-MPR simulation results according to RB start position and the number of contiguous RB size as shown in figure7.2.2.2.2-2.

Figure 7.2.2.2.2-2 RF simulation results for S-band UE to protect Band 34 with -50dBm/MHz as a co-existence requirement

And also we provided some additional simulation results to protect band 34 according to the UE coexistence requirements with -20~-40 dBm/MHz in figures 7.2.2.2.2-3~7.2.2.2.2-5.

Figure 7.2.2.2.2-3 RF simulation results for S-band UE to protect Band 34 with -40dBm/MHz as a co-existence requirement

Figure 7.2.2.2.2-4 RF simulation results for S-band UE to protect Band 34 with -30dBm/MHz as a co-existence requirement

Figure 7.2.2.2.2-5 RF simulation results for S-band UE to protect Band 34 with -20dBm/MHz as a co-existence requirement

********************** End of text proposal to TR 36.861 Chapter 7.2.2 ***************************