3GPP TSG-RAN4 Meeting #72	 R4-144091

# Dresden, Germany, 18-22 August, 2014

| Title: | Measurement Uncertainty Analysis for a Near Field System setup for AAS BS OTA testing |
| --- | --- |
| Source: | SATIMO Industries |
| Agenda Item: | 7.2.4 |
| Work Item | AAS_BS_LTE_UTRA-Perf |
| Release | 12 |
| Document for: | Discussion |
| Abstract | This contribution highlights the uncertainty contributors for a Near Field Measurement System Setup when perfroming AAS BS OTA testing. The measurement uncertainty budget is reported based on the TS 34.114 Annex E. The uncertainty contributors for a Far Field system setup are also reported. |

# 1	Introduction R4-125719

During the 3GPP RAN4#71, preliminary results of an AAS BS OTA testing were provided when using a Near Field Measurement Technique [1]. In the WF [2], the AAS BS group agreed on the need of providing a full characterization of the measurement uncertainty associated with each testing methology such as Far Field, Compact Range, and Near Field. The measurement uncertainty budget should have been based on the existing uncertainty budget in [3]. Each uncertainty contributors shall be highlighted and the measurement uncertainty budget calculated afterwards. This contribtion presents the uncertainty contributors for each testing methodology being investigated such as Far Field, Compact Range and Near Field. The uncertainty contributors for the Near Field technique are also estimated based on a SATIMO test range which is ISO17025 accredited lab.

# 2	Measurementy Uncertainty

Uncertainty is by definition, a parameter associated with the measurement result that characterizes the spread that could be reasonably attributed to the quatity being measured [4-5].

In [6] a detailed explaintaion of the uncertainty contributors associated with Near Field and Far Field testing techniques is given. Some of the uncertainty contributors are common to each testing methodology. To facilitate the reading, the uncertainty contributors have been divided in sub groups. In the following sections the uncertainty contributors associated with each testing methodology are reported.

2.1 	Far Field Methodology

The uncertainty contributors are for an antenna gain measurement performed in Far Field and Compact Test Ranges. The scope of the gain measurement is to measure the absolute gain of the antenna under test and to be able to normalize the radiation pattern to the absolute gain. The antenna under test is illuminated by a plane wave which is polarized with the same polarization as the AUT. The received power PAUT  into a matched load is measured. Then the same measurement is performed when an antenna with know gain GSGHis used for, PSGH. The gain of the AUT is then given by:

GAUTdB=GSGHdB- PSGHdB+ PAUTdB- MSGH[dB]

This definition is not conformal with IEEE definition [4] since the mismatch between test object/gain horn and test equipment is not included. For real measurements those uncertainties must be taken into account. The uncertainty contributions can be divided in category, Mechanical, Electrical, and Stray Signals. Bellow is a list of them with little explaination of those:

Category Mechanical:

Mechanical System Setup -> this quantity is negligible if a true Far Field system and gain substitution is used

Category Electrical:

Dynamic Range -> System Dynamic Range

SGH Measurement Uncertainty -> Meauserement Uncertainty of the lab in which the SGH has been calibrated

Receiver Amplitude Non Linearity -> Non – Linearirty of the system receiver

Rotary Joints and Flexible Cable -> Systematic Amplitude andPhase Change with angle of rotary joint and /or cable movement

Leakage and Cross talk -> Random Amplitude and Phase Change at still AUT

Drift -> Random Ampitude and Phase Change at still AUT

Category Stray Signals

Multiple Reflections between AUT/SGH and range feed -> Contribution to the received signal between AUT/SGH and the range feed/probe

Multiple Reflections between setup receiver and AUT/SGH -> Contribution to the receiver signal due to multiple reflections between setup/receiver and AUT/SGH. Residual if compensated

Room Scattering -> Contribution to the received signal due to finite reflectivity of the anechoic chamber

A detailed way to estimate the above uncertainty contributors is reported in [5].

2.2 	Near Field Methodology

The measurement uncertaintyfor a Near Field technique can be divided in two main sub groups

- Near field measurement uncertainty – pattern (directivity) measurement uncertainty (it does include the uncertainty for the Near Field to Far Field Transformation too)

- EIRP (EIRS) measurement uncertainty – total power measurement

In the Near Field measurements we can obtain an estimate of the magnitude of the various errors but for many errors we cannot obtain an estimate of the phase of the error. Thus, for many Near Field uncertainties the magnitude of the uncertainty equals to the estimated magnitude of the corresponding error. Bellow is a lsit of the main uncertainty contributors for a Spherical Near Field Measurement setup when performing pattern measurements (directivity). As for the Far Field technique, the uncertainyty contributors have been divided in category and are based on [6]:

Category Mechanical

Axes Intersection -> Lateral Displacement between the Horizontal and Vertical Axes

Axes Orthogonality -> Difference from 90° of the angle between Horizontal and Vertical Axes

Horizontal Pointing -> For Theta=0°, horizontal mispointing of the horizontal axis to the probe reference point

Probe Vertical Position -> Vertical displacement of the probe reference point from the horizontal axis

Probe Horizontal/Vertical Pointing -> Horizontal or vertical mispointing of the probe, z-axis from the intersection point of the Vertical and Horizontal axes

Measurement Distance -> Knowlodge of the distance between the intersection point of the Vertical and Horizontal axes and theprobe reference point

Category Electrical

Amplitude and Phase Drift -> Systematic Amplitude and Phase change at still AUT

Amplitude and Phase Noise -> Random Amplitude and Phase change at still AUT

Leakage and Cross Talk -> Extraneous signal dependent on angle

Amplitude Non linearity -> Difference from Linear dependence of measured value versus input signal level

Amplitude and Phase shift in Rotary Joints -> Systematic and Phase change with angle of rotary joints

Category Probe Related

Channel Balance Amplitude and Phase -> Knowledge of the amplitude and phase difference between two signal channels of the receiver

Probe polarization Amplitude and Phase -> Knowledge of the amplitude and Phase of the probe polarization coefficients

Probe pattern knowledge -> Deviation from the known or assumed probe pattern to the true one

Category Stray Signals

Multiple Reflections -> Contribution to the received signal due to the interaction between the AUT and the probes

Room Scattering -> Contribution to the received signal due to the finite reflectivity of the anechoic chamber

AUT support scattering -> Contribution to the received signal due to scattering from the AUT support structure

Category Acquisition

Scan Area Truncation -> Error due to partial sphere acquisition

Sampling point offset -> Error due to continuous postioner rotation

Category Processing

Spherical Mode Truncation -> Changes of the Far Field due to different number of spherical modes included in the field calculation

Total radiated Power -> Error in the calculation of the total power

Category Probe Array

Scattering -> Scattered fields by the probe array

Positioning -> Error due to non ideal positioning and orientation of the probes

Probe Uniformity -> Error due to different probe response

The attached spreadsheets do highlight the uncertainty contributors when a gain measurement (EIRP) is performed.

# 3	Conclusions and next steps

The measurement uncertainty associated with a Near Field Measurement System has been provided. It includes the uncertainty contributors for the Near Field measurement, NF to FF transformation, and then the EIRP (or EIRS) measurement. This measurement uncertainty budget is based on the measurement uncertainty budget reported in [3] for the UE. The Measurement Uncertainty budget for the Near Field Setup system is 1.24dB, and 1.1dB respectively for the  EIRP FF pattern measurement in the range of 0.8-1GHz and 1-6GHz.

# 8		      References

[1] R4-142684, “Near Field Measurement Technique and Preliminary Results for Radiated AAS BS testing”, SATIMO, May 2014

[2] R4 – 143972 “WF for AAS BS conformance testing”, Ericsson, May 2014

[3] 3GPP TS 34.114

[4] IEEE Std 145-1993, IEEE Standard Definitions of Terms for Antennas.

[5] IEEE Standard Test Procedures for Antennas, ANSI/IEEE Std 149-1979 (R2003).

[6] ACE (Antenna Centre for Excellence Project ,“Deliverable A1.2D2 Recommendations and comparative investigations for near-field antenna measurement  techniques and procedures”, December 2007