3GPP TSG-RAN WG4 Meeting #72                           R4-144455

Dresden, Germany, 18 – 22 Aug, 2014

Source: 	CMCC

Title: 	Simulation results of RSRP/RSRQ measurement under high Doppler

Agenda Item:	7.39.4

Document for:	Discussion

- Introduction

RSRP/RSRQ measurement accuracy requirements under high Doppler condition were discussed for a few meeting cycles. In RAN4#71 meeting, a way forward on RSRP/RSRQ requirements under high Doppler condition was approved as follows [1]:

- Definition of ideal RSRP/RSRQ for high Doppler

  - The ideal RSRP and RSRQ are defined as the signal power under AWGN channel. RSRP excludes noise. For RSRQ, RSSI includes noise.

    - Note: The ideal RSRP and RSRQ are calculated using Es1/Noc, Es2/Noc (for 2 cell simulations) and Noc.

  - The definition shall be used for further simulations (HST, EVA300, EVA600), RRM core requirements and corresponding RRM test cases under high Doppler condition.

- RSRP/RSRQ for high Doppler core requirements shall be defined based on one specific channel from HST, EVA300, EVA600. The specific channel to be defined in requirements is FFS.

  - The accuracy tolerance under high Doppler condition shall be determined based on the ideal RSRP/RSRQ defined in this wayforward, compared with the existing legacy RSRP/RSRQ requirements under AWGN channel.

- A new separate RSRP/RSRQ accuracy requirement clause under high Doppler shall be introduced in TS 36.133.

In this contribution, simulation results for RSRP/RSRQ measurement accuracy of 2-cell scenario under high Doppler are provided.

- Simulation Assumptions

Simulation assumptions are listed in Table 1 referred to [2].

Table 1: Simulation Assumptions

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 6 resource blocks | Both RSRP and RSSI measured over 6 RB |
| System bandwidth | 6 resource blocks |  |
| L1 measurement period | 200 ms |  |
| Measurement sampling rate | - | 5 samples/200ms. |
| L3 filtering | disabled |  |
| Transmit antenna | 1 |  |
| Receive antennas | 2 | The receive diversity rule as defined in TS 36.214. Both antennas with equal gain, no correlation between them. |
| DRX/DTX | OFF | DRX/DTX to be considered at later stage |
| Propagation conditions | EVA (EVA600, EVA300), HST |  |
| CP Length | Normal |  |
| TDD Uplink-downlink configuration | 1 |  |
| TDD Special sub-frame configuration | 4 |  |
| Time offset between TDD cells | 0 second |  |
| Frequency band | 2.0 GHz |  |
| Es/Noc (cell 1) | (6dB , -4.7dB , 0dB) |  |
| Es/Noc (cell 2) | (1dB , -4.7dB , 0dB) |  |
| Ês/Iot (cell 1) | (2.5dB , -6dB , -3dB) | Derived from Noc and Es/Noc |
| Ês/Iot (cell 2) | (-6dB , -6dB , -3dB) | Derived from Noc and Es/Noc |
| Target cell | Cell 2 | For absolute accuracy |
| NOTE1 : Encourage companies to provide the details of the measurement sampling rate for interpretation and comparison of the results. |  |  |

- Simulation Results

In this section, simulation results for 2-cell RSRP/RSRQ measurement under different channels are provided in Table 2 and 3. The ideal RSRP and RSRQ are defined as the signal power under AWGN channel.

Table 2: RSRP measurement Accuracy

| Channel Type | Es/Noc(dB) | Es/Iot(dB) | Absolute RSRP bias 5%~95%(dB) | Relative RSRP bias 5%~95%(dB) |
| --- | --- | --- | --- | --- |
| EVA600 | Cell 1:6dB Cell 2:1dB | Cell 1:2.5dB Cell 2:-6dB | [0.53 2.52] | [-0.46 0.48] |
|  | Cell 1:-4.7dB Cell 2:-4.7dB | Cell 1:-6dB Cell 2:-6dB | [-0.41 2.40] | [-1.47 1.57] |
|  | Cell 1:0dB Cell 2:0dB | Cell 1:-3dB Cell 2:-3dB | [0.16 2.29] | [-0.83 0.80] |
| EVA300 | Cell 1:6dB Cell 2:1dB | Cell 1:2.5dB Cell 2:-6dB | [0.63 2.26] | [-0.51 0.49] |
|  | Cell 1:-4.7dB Cell 2:-4.7dB | Cell 1:-6dB Cell 2:-6dB | [-0.82 1.41] | [-1.69 1.64] |
|  | Cell 1:0dB Cell 2:0dB | Cell 1:-3dB Cell 2:-3dB | [-0.15 1.06] | [-0.85 0.87] |
| HST | Cell 1:6dB Cell 2:1dB | Cell 1:2.5dB Cell 2:-6dB | [-0.50 2.00] | [-0.51 0.51] |
|  | Cell 1:-4.7dB Cell 2:-4.7dB | Cell 1:-6dB Cell 2:-6dB | [-1.15 1.86] | [-1.59 1.60] |
|  | Cell 1:0dB Cell 2:0dB | Cell 1:-3dB Cell 2:-3dB | [-0.64 1.78] | [-0.88 0.89] |

Table 3 RSRQ measurement Accuracy

| Channel Type | Es/Noc(dB) | Es/Iot(dB) | Absolute RSRQ bias 5%~95%(dB) | Relative RSRQ bias 5%~95%(dB) |
| --- | --- | --- | --- | --- |
| EVA600 | Cell 1:6dB Cell 2:1dB | Cell 1:2.5dB Cell 2:-6dB | [-0.20 0.78] | [-0.44 0.43] |
|  | Cell 1:-4.7dB Cell 2:-4.7dB | Cell 1:-6dB Cell 2:-6dB | [-0.72 1.60] | [-1.38 1.43] |
|  | Cell 1:0dB Cell 2:0dB | Cell 1:-3dB Cell 2:-3dB | [-0.32 0.98] | [-0.75 0.73] |
| EVA300 | Cell 1:6dB Cell 2:1dB | Cell 1:2.5dB Cell 2:-6dB | [-0.01 0.98] | [-0.45 0.43] |
|  | Cell 1:-4.7dB Cell 2:-4.7dB | Cell 1:-6dB Cell 2:-6dB | [-0.74 1.34] | [-1.60 1.52] |
|  | Cell 1:0dB Cell 2:0dB | Cell 1:-3dB Cell 2:-3dB | [-0.11 0.95] | [-0.75 0.79] |
| HST | Cell 1:6dB Cell 2:1dB | Cell 1:2.5dB Cell 2:-6dB | [-0.36 0.75] | [-0.46 0.48] |
|  | Cell 1:-4.7dB Cell 2:-4.7dB | Cell 1:-6dB Cell 2:-6dB | [-1.16 1.34] | [-1.48 1.50] |
|  | Cell 1:0dB Cell 2:0dB | Cell 1:-3dB Cell 2:-3dB | [-0.60 0.85] | [-0.80 0.77] |

Based on the simulation results, the following observations can be obtained:

Observation 1: For absolute RSRP measurement accuracy, the maximum accuracy is 2.52dB, 2.26dB and 2dB under EVA600, EVA300 and HST respectively.

Observation 2: For relative RSRP measurement accuracy, the maximum accuracy is 1.57dB, 1.69dB and 1.6dB under EVA600, EVA300 and HST respectively.

Observation 3: For absolute RSRQ measurement accuracy, the maximum accuracy is 1.6dB, 1.34dB and 1.34dB under EVA600, EVA300 and HST respectively.

Observation 4: For relative RSRQ measurement accuracy, the maximum accuracy is 1.43dB, 1.6dB and 1.5dB under EVA600, EVA300 and HST respectively.

For absolute RSRP accuracy (+/-6dB), +/-2dB is the baseband accuracy and +/-4dB is the RF margin [4]. Based on the above observations, absolute RSRP accuracy under EVA600/ EVA300 is out of range. Regarding the other cases, the results show that the accuracy under high Doppler condition could satisfy the existing requirements.

Proposal 1: The new separate RSRP/RSRQ accuracy requirements should be specified based on EVA600.

Proposal 2: For absolute RSRP accuracy under high Doppler condition, new requirement need to be specified based on EVA600.

Proposal 2: For relative RSRP accuracy under high Doppler condition, the existing requirements could be reused.

Proposal 3: For absolute/relative RSRQ accuracy under high Doppler condition, the existing requirements could be reused.

- Conclusion

In this contribution, we provide the simulation results for RSRP/RSRQ measurement accuracy of 2 cells scenario under high Doppler condition. According to the results, several observations and proposals are listed as follows:

Observation 1: For absolute RSRP measurement accuracy, the maximum accuracy is 2.52dB, 2.26dB and 2dB under EVA600, EVA300 and HST respectively.

Observation 2: For relative RSRP measurement accuracy, the maximum accuracy is 1.57dB, 1.69dB and 1.6dB under EVA600, EVA300 and HST respectively.

Observation 3: For absolute RSRQ measurement accuracy, the maximum accuracy is 1.6dB, 1.34dB and 1.34dB under EVA600, EVA300 and HST respectively.

Observation 4: For relative RSRQ measurement accuracy, the maximum accuracy is 1.43dB, 1.6dB and 1.5dB under EVA600, EVA300 and HST respectively.

Proposal 1: The new separate RSRP/RSRQ accuracy requirements should be specified based on EVA600.

Proposal 2: For absolute RSRP accuracy under high Doppler condition, new requirement need to be specified based on EVA600.

Proposal 2: For relative RSRP accuracy under high Doppler condition, the existing requirement could be reused.

Proposal 3: For absolute/relative RSRQ accuracy under high Doppler condition, the existing requirements could be reused.

- Reference

- R4-143894, Wayforward on RSRP/RSRQ requirements under high Doppler condition, Huawei, HiSilicon.

- R4-142350, High Doppler measurement accuracy simulation assumptions, Ericsson, RAN4 #70bis.

- R4-142340, Wayforward on RSRP/RSRQ requirements under high Doppler condition, Huawei, HiSilicon, CATT, Intel, NTT DOCOMO, CMCC, Ericsson, Anritsu, LG Electronics, Samsung, ZTE, MTK

- R4-143905, WF on tightening absolute RSRP requirements, CMCC.