3GPP TSG-RAN Working Group 4 Meeting #72                    R4-144512

Dresden, Germany, 18-22 Aug, 2014

Source:	Samsung

Title:	Text Proposal for TR 36.851: TDD-FDD CA for CA_1A-41A

Agenda item:	7.7.5.2

Document for:	Approval

- 1	Introduction

In RAN4 #70, inter-band CA operation on Band 1 and Band 41(CA_1A-41A) was approved as one of the example bands for TDD-FDD CA WI in Rel-12[1]. And in RAN#64, new WI of CA_1A_41A_41A with UL requirements only for Band 1was approved[2], so it is necessary to start research CA_1A-41A firstly. In this contribution, a relevant text proposal for TR36.851 was provided, which included channel bandwidth combination, harmonic analysis results and transmitter/receiver insertion losses, etc. Due to relaxation valuesΔTIB,c and ΔRIB,c dominate depend on reference architecture which still in discussion, so are left for FFS.

- 2	Discussion

Table 1 and 2 show E-UTRA operating bands and supported channel bandwidth for this combination.

Table 1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| NOTE 1: The WI only define the uplink requirements for Band 1 |  |  |  |  |  |  |  |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| CA_1A-41A | 1 |  |  | Yes | Yes | Yes | Yes |
|  | 41 |  |  | Yes | Yes | Yes | Yes |

Due to this WI only define the uplink requirements for Band 1, then the harmonic products for Band 1+Band 41 CA are summarized as table 3. It can be seen that none of the harmonic products fall into the B1 or B41 UE receiving bands..

Table 3: B1 + B41 harmonic products with B1 UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |
| 41 | 2496 | 2690 | 2496 | 2690 |  |  |  |  |

The relaxation for CA_1A-41A is mostly depend on the reference RF architecture, and it is still in discussion, thus relaxation values are left FFS as below.

Table 4: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-41A | 1 | FFS |
|  | 41 | FFS |

Table 5: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-41A | 1 | FFS |
|  | 41 | FFS |

- 3	Conclusion

Above discussion to be captured in TR36.851[3] and text proposal is provided in annex.

- References

- RP-140465, " Revised WID: LTE TDD-FDD joint operation including Carrier Aggregation " Nokia Corporation

- RP-140963 “New WI: LTE Advanced 3 Band Carrier Aggregation(3DL/1UL) of Band 1, Band 41 and Band 41”

- TR36.851 v0.10.0

Annex: Text proposal for the TR36.851(Inter-band CA Rel-12)

Text Proposal for TR36.851

<Start of change section>

7 TDD-FDD Inter-band Carrier Aggregation

7.1 General Part

<unchanged section omitted>

7.2 Band Combination Specific Part

7.2.1 LTE Advanced Carrier Aggregation of Band 1 and Band 41(1UL)

Table 7.2.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| NOTE 1: The WI only define the uplink requirements for Band 1 |  |  |  |  |  |  |  |  |

7.2.1.1 List of specific combination issues

7.2.1.1.1 channel bandwidth per operating band for CA

Table 7.2.1.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| CA_1A-41A | 1 |  |  | Yes | Yes | Yes | Yes |
|  | 41 |  |  | Yes | Yes | Yes | Yes |

NOTE 1: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [4] when operating in single carrier mode.

NOTE 2: This WI only define the uplink requirements for Band 1.

7.2.1.1.2 Co-existence studies for CA_1-41

- 7.2.1.1.2.1 For the BS

<Test will be added>

- 7.2.1.1.2.2 For the UE

Due to this WI only define the uplink requirements for Band 1, then the harmonic products for Band 1+Band 41 CA are summarized as Table 7.2.1.1.2-1. It can be seen that one of the harmonic products fall into the B1 or B41 UE receiving bands..

Table 7.2.1.1.2-1: B1 + B41 harmonic products with B1 UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |
| 41 | 2496 | 2690 | 2496 | 2690 |  |  |  |  |

7.2.1.1.3 ∆TIB and ∆RIB values

The relaxation for CA_1A-41A is mostly depend on the reference RF architecture, and it is still in discussion, thus relaxation values are left FFS as shown in Table 7.2.1.1.3-1 and Table 7.2.1.1.3-2.

Table 7.2.1.1.3-1 : ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-41A | 1 | FFS |
|  | 41 | FFS |

Table 7.2.1.1.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-41A | 1 | FFS |
|  | 41 | FFS |

<End of change section>