3GPP TSG-RAN WG4 Meeting #72	    R4-144534

Dresden, Germany, 18 - 22 Aug, 2014

Agenda Item:	7.10.4

Source: 	Samsung

Title: 	System level simulation results for CRS/CSI-RS based SINR

Document for:	Discussion

Introduction

In RAN4 #71AH meeting, system level simulation assumptions for RRM measurements based on CSI-RS/CRS were approved in [1], to study the SINR side condition for CRS/CSI-RS based SCE RRM.

In this contribution, we provide the simulation results for CRS/CSI-RS based SINR according to the agreed assumptions.

Methodology for Interference Level Evaluation

The methodology for interference level evaluation is describled as below:

- Step 1: Each UE connect to a serving cell (macro cell or small cell) based on the evaluated geometry.

  - Macro cells and small cells are assumed on different frequency layer.

  - Geometry is evaluated assuming full load traffic model.

- Step 2: Identify cell edge UE, cell medium UE, and cell centre UE simiar as the methodology in NAICS SI:

  - Cell edge UE: geometry is in the range of 5%~25% -tile;

  - Cell medium UE: geometry is in the range of 40% ~60% -tile;

  - Cell centre UE: geometry is in the range of 75%~95% -tile;

- Step 3: For each UE, calculate the wideband SINR of top 3 strongest interference small cells.

Assuming SINRi,j is the wideband SINR from UE i to Cell j, it is calculated as

SINRi,j=Pi,jm,m≠jαi,m∙Pi,m+N

where Pi,j is the received power from Cell j at UE i and N is the noise power.

- For CRS based SINR, αi,m=1 if CRS of interfernece cell m is colliding with targeted cell j. Otherwise, αi,m=cell loading level.

- For CSI-RS based SINR, it is assumed CSI-RS muting is applied within each small cell cluster. Therefore, αi,m=0 if CRS of interfernece cell m is in the same cluster with targeted cell j. Otherwise, αi,m=cell loading level.

- Step 4: For each set of UE, the average SINR of top 3 interfence cells is calculated.

Smulation Results

The detailed simulation assumptions are listed in Table A.1 in Annex A.

In this paper, we mainly focus on dense small cell deployment, i.e. 10 LPNs per macro cell. The results under sparse deployment are listed in Annex B.

Results for cell edge UE

In this sub-section, the interference level results of top 3 strongest interference small cells are presetned for cell edge UE.  Furthermore, the results are presented for SCE UE and macro UE in Table 1 and Table 2 separately.

Table 1: Interference Level of Top 3 Strongest Interfernece Small Cells for SCE UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 0.28 | -4.92 | -9.79 |
| CRS based, 50% load | 2.30 | -2.88 | -7.76 |
| CSI-RS based, 100% load | 11.54 | 7.65 | 2.83 |
| CSI-RS based, 50% load | 14.49 | 10.60 | 5.79 |

Table 2: Interference Level of Top 3 Strongest Interfernece Small Cells for Macro UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | -1.06 | -4.79 | -8.77 |
| CRS based, 50% load | 0.93 | -2.80 | -6.82 |
| CSI-RS based, 100% load | 9.69 | 6.90 | 2.96 |
| CSI-RS based, 50% load | 12.63 | 9.85 | 5.92 |

Results for cell medium UE

In this sub-section, the interference level results of top 3 strongest interference small cells are presetned for cell medium UE.  Furthermore, the results are presented for SCE UE and macro UE in Table 3 and Table 4 separately.

Table 3: Interference Level of Top 3 Strongest Interfernece Small Cells for SCE UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 4.42 | -8.03 | -12.82 |
| CRS based, 50% load | 6.48 | -5.87 | -10.78 |
| CSI-RS based, 100% load | 16.87 | 8.62 | 3.22 |
| CSI-RS based, 50% load | 19.80 | 11.56 | 6.16 |

Table 4: Interference Level of Top 3 Strongest Interfernece Small Cells for Macro UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 0.92 | -5.98 | -10.21 |
| CRS based, 50% load | 2.90 | -3.96 | -8.18 |
| CSI-RS based, 100% load | 12.37 | 7.63 | 3.08 |
| CSI-RS based, 50% load | 15.31 | 10.57 | 6.03 |

Results for cell centre UE

In this sub-section, the interference level results of top 3 strongest interference small cells are presetned for cell centre UE.  Furthermore, the results are presented for SCE UE and macro UE in Table 5 and Table 6 separately.

Table 5: Interference Level of Top 3 Strongest Interfernece Small Cells for SCE UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 9.56 | -12.81 | -17.28 |
| CRS based, 50% load | 11.53 | -10.68 | -15.20 |
| CSI-RS based, 100% load | 21.50 | 7.44 | 1.46 |
| CSI-RS based, 50% load | 24.40 | 10.36 | 4.38 |

Table 6: Interference Level of Top 3 Strongest Interfernece Small Cells for Macro UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 2.19 | -6.78 | -11.14 |
| CRS based, 50% load | 4.24 | -4.66 | -9.07 |
| CSI-RS based, 100% load | 15.24 | 9.28 | 4.56 |
| CSI-RS based, 50% load | 18.15 | 12.20 | 7.49 |

Discussion

Based on the above simulation results, it can be observed:

- For all interference level of the top 3 strongest cells, CSI-RS based interference level shows more than 10 dB gain over CRS based, if CSI-RS muting is applied within each small cell cluster.

- For CRS based RRM measurement, the side condition of 2nd and 3rd interference cell is expected to be much worse than the legacy side condition. Thus, it may be a big chanllege for UE implementation if the similar performance requirement is applied.

Conclusion

In this contribution, we provide the simulation results of the interference level of top 3 srongest interfernece small cells. Our observations are,

- For all interference level of the top 3 strongest cells, CSI-RS based interference level shows more than 10 dB gain over CRS based, if CSI-RS muting is applied within each small cell cluster.

- For CRS based RRM measurement, the side condition of 2nd and 3rd interference cell is expected to be much worse than the legacy side condition. Thus, it may be a big chanllege for UE implementation if the similar performance requirement is applied.

Furthermore, it is proposed to take those results into account when RAN4 group define the side condition for RRM measurement based on CSI-RS and CRS.

Reference

- R4-71AH-0095, System level simulation assumptions for RRM measurements based on CSI-RS/CRS, Ericsson.

Annex A: Simulation Assumptions

Table A.1 Assumptions for system level evaluation

| Parameter |  |  | Values used for evaluation |  |
| --- | --- | --- | --- | --- |
| Deployment scenarios |  | - Scenario 2a according to 36.872: Outdoor small cell deployment, with small cells deployed in the presence of an overlaid macro network. Separate frequency deployment of the macro cell and small cells. Cluster deployment model should be referred. (LPNs are randomly dropped within 50 m radius cluster) |  |
| Number of macro sites |  | 7 |  |
| Number of sectors per macro site |  | 3 |  |
| ISD |  | 500 m |  |
| Number of LPNs per macro cell | Sparse scenario | 4 |  |
|  | Dense scenario | 10 |  |
| Number of UEs |  | 40 |  |
| UE dropping |  | UEs randomly and uniformly dropped within the clusters. 20% UEs are outdoor and 80% UEs are indoor. |  |
| LPN TX power |  | 1 w |  |
| System BW |  | 10 MHz |  |
| Carrier frequency of small cells |  | f1=3.5 GHz |  |
| Carrier frequency of macro cells |  | f2 ≠ f1 |  |
| Minimum distance UE to LPN |  | 5 m |  |
| Minimum distance LPN toLPN |  | 20 m |  |
| LPN antenna gain |  | 5 dBi |  |
| UE Antenna gain |  | 0 dBi |  |
| UE Noise Figure |  | 9 dB |  |
| Network synchronization |  | Synchronized |  |
| Antenna height |  | 10m for LPN/Hotzone Node |  |
| Pathloss model |  | - ITU UMi - Carrier Frequency : 3.5GHz - 3D distance UMi penetration, pathloss, and shadowing generation methodology is used for LPN to UE |  |
| Load per LPN |  | 50% and 100% |  |
| CSI-RS muting pattern |  | Muting within a cluster and no muting within cluster |  |

Annex B: Simulation Results for Sparse Deployment

In sparse small cell deployment, there are 4 LPNs per macro cell.

Results for cell edge UE

In this sub-section, the interference level results of top 3 strongest interference small cells are presetned for cell edge UE.  Furthermore, the results are presented for SCE UE and macro UE in Table 7 and Table 8 separately.

Table 7: Interference Level of Top 3 Strongest Interfernece Small Cells for SCE UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 2.61 | -5.01 | -14.36 |
| CRS based, 50% load | 4.81 | -2.76 | -12.23 |
| CSI-RS based, 100% load | 12.88 | 7.25 | -5.18 |
| CSI-RS based, 50% load | 15.57 | 10.00 | -2.38 |

Table 8: Interference Level of Top 3 Strongest Interfernece Small Cells for Macro UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 0.15 | -4.20 | -11.68 |
| CRS based, 50% load | 2.24 | -2.06 | -9.73 |
| CSI-RS based, 100% load | 8.80 | 5.41 | -4.00 |
| CSI-RS based, 50% load | 11.44 | 8.09 | -1.27 |

Results for cell medium UE

In this sub-section, the interference level results of top 3 strongest interference small cells are presetned for cell medium UE.  Furthermore, the results are presented for SCE UE and macro UE in Table 9 and Table 10 separately.

Table 9: Interference Level of Top 3 Strongest Interfernece Small Cells for SCE UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 9.02 | -11.16 | -19.50 |
| CRS based, 50% load | 11.16 | -8.90 | -17.27 |
| CSI-RS based, 100% load | 18.07 | 3.37 | -9.37 |
| CSI-RS based, 50% load | 20.68 | 6.08 | -6.60 |

Table 10: Interference Level of Top 3 Strongest Interfernece Small Cells for Macro UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 3.02 | -5.96 | -14.56 |
| CRS based, 50% load | 5.16 | -3.74 | -12.48 |
| CSI-RS based, 100% load | 13.53 | 7.16 | -4.71 |
| CSI-RS based, 50% load | 16.15 | 9.83 | -1.97 |

Results for cell centre UE

In this sub-section, the interference level results of top 3 strongest interference small cells are presetned for cell centre UE.  Furthermore, the results are presented for SCE UE and macro UE in Table 11 and Table 12 separately.

Table 11: Interference Level of Top 3 Strongest Interfernece Small Cells for SCE UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 17.99 | -20.64 | -27.44 |
| CRS based, 50% load | 20.04 | -18.31 | -25.18 |
| CSI-RS based, 100% load | 23.86 | -5.80 | -15.43 |
| CSI-RS based, 50% load | 26.35 | -3.15 | -12.71 |

Table 12: Interference Level of Top 3 Strongest Interfernece Small Cells for Macro UE

|  | 1st cell(dB) | 2nd cell(dB) | 3rd cell(dB) |
| --- | --- | --- | --- |
| CRS based, 100% load | 4.95 | -7.77 | -16.56 |
| CRS based, 50% load | 7.05 | -5.56 | -14.43 |
| CSI-RS based, 100% load | 15.02 | 5.58 | -6.59 |
| CSI-RS based, 50% load | 17.61 | 8.26 | -3.86 |