3GPP TSG RAN WG4 Meeting #72	        R4-144552

# Dresden, Germany, 18th  to 22th, August 2014

- Agenda Item:	7.35

- Source:	LG Electronics

- Title:	TP for Self desense analysis for 2ULs CA_5A-7A UE

Document for:	Approval

- Introduction

In RAN4 #70 meetings, RAN4 agreed that high order IMD3 ~ IMD5 shall be considered for self-desensitization analysis in 2ULs inter-band CA UE case-by-case [1][2], which shall not modify the inter-band CA classes.

In this contribution, we performed measurements of high order IMD3 to IMD5 level for 2ULs CA_5A-7A UE as a general aspect based on the agreed WF [3] and provide our view on this result.

- Measurements on the high order IMD levels for 2ULs CA_5A-7A

Table 1 shows the frequency range of high order IMD products in CA_5A-7A combinations.

Table 1: Band 5 and Band 7 UL IMD product

| UE UL Carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| Two-tone 3rd order IMD products | ¦2*fx_low - fy_high¦ | ¦2*fx_high - fy_low¦ | ¦2*fy_low - fx_high¦ | 2*fy_high - fx_low |
| IMD frequency limits (MHz) | 802 – 922 |  | 4151 - 4316 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4148 – 4268 |  | 5824 – 5989 |  |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 47 – 98 |  | 6651 – 6886 |  |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 – 5117 |  | 8324 – 8559 |  |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ |  | ¦2*fy_low - 2*fx_high¦ |  |
| IMD frequency limits (MHz) | 3302 – 3492 |  |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ |  | ¦2*fy_high + 2*fx_high¦ |  |
| IMD frequency limits (MHz) | 6648 – 6838 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9151 – 9456 |  | 726 – 896 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 – 11129 |  | 5796 – 5966 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5802 – 6062 |  | 2453 – 2668 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 – 9408 |  | 7472 – 7687 |  |

Based on table 1, IMD3 and IMD5 may fall into own Band 5 RX frequency. And IMD5 may fall into own Band 7 Rx frequency. However IMD4 does not fall into own Rx frequencies, thus IMD4 has no impact on the self desense for CA_5A-7A UE.

To analyze the IMD products at PA output, we assumed the typical RF components values in CA_5A-7A as follows

  - RB Allocation : Band 5 (5MHz CBW, Full RB), Band 7 (10MHz CBW, Full RB)

  - Aggregated maximum output power : 25dBm with 2dB tolerance

  - Tx/Rx RF component IL : 3.5dB

    - 3.5dB = sum(ANT SW: 0.5dB,  Diplexer :0.5dB, Duplexer: 2.5dB)

  - FE coupling loss : 40~70dB

  - Antenna loss :10dB

  - Diplexer Characteristics: 20dB for Band 5- Band 7 isolation

  - Duplexer Characteristics: Tx-Rx isolation: 50dB, Tx-Ant isolation : 35 dB

For 2UL inter-band CA_5A-7A UE, the leaked signal from Band 5 may fall into Band 7 own DL frequency and the leakage of Band 7 may also have impact on the Band 5 own DL frequency as shown in figure1.

Figure1. Example of input signal level to generate the high order IMD at RF components

In the previous analysis [4] of active components, we knew that the coupling loss in PA is more critical than the general interference signal by reversed RF path (blue line). If coupling loss is less than 50dB, and then the estimated leaked input signal level is higher than these values. So we can assume only coupling loss from 40~70dB ignoring the reversed RF path’s signal levels.

- Case1: Band 5 aggressor and Band 7 victim

- PA Active IMD (P1 point):

  - Signal level from B7 : 0dBm

  - Signal level from B5: -15, -25, -35 or -45dBm with coupling loss 40, 50, 60 or 70dB

The same approaches are applied for the opposite case that is the leaked signal from Band 7 may also have impact on the Band 5 own DL frequency.

- Case2: Band 7 aggressor and Band 5 victim

- PA Active IMD (P2 point):

  - Signal level from B5 : 0dBm

  - Signal level from B7: -15, -25, -35 or -45dBm with coupling loss 40, 50, 60 or 70dB

From the observation, we can measure the generated IMD products in P1 and P2 points as shown in figure2 and figure3.

Figure2. IMD measurement setup at RF components output in Case1

Figure3. IMD measurement setup at RF component output in Case2

The measurement results of IMD products of PA output are shown in Table 2 and 3 according two test cases.

Table 2: IMD3 and IMD5 measurements results in PA of 2ULs CA_5A-7A UE in Case1

| Band 5 (MHz) | Band 7 (MHz) | Coupling Loss | B5 (aggressor) Power | B7 (victim) Power | Level of IM3 | Level at 2640 MHz (IMD5) |
| --- | --- | --- | --- | --- | --- | --- |
| 830 | 2565 | 40dB | -15dBm | 0 dBm | N/A | -64.74 |
| 830 | 2565 | 50dB | -25dBm | 0 dBm | N/A | -65.21 |
| 830 | 2565 | 60dB | -35dBm | 0 dBm | N/A | -65.67 |
| 830 | 2565 | 70dB | -45dBm | 0 dBm | N/A | - |

Table 3: IMD3 and IMD5 measurements results in PA of 2ULs CA_5A-7A UE in Case2

| Band 5 (MHz) | Band 7 (MHz) | Coupling loss | B5 (Victim) Power | B7 (aggressor) Power | Level at 879 MHz (IMD3) | Level at 891.5 MHz (IMD5) |
| --- | --- | --- | --- | --- | --- | --- |
| 834 | 2547 | 40dB | 0 dBm | -15dBm | -19.72-2.8 | N/A |
| 834 | 2547 | 50dB | 0 dBm | -25dBm | -29.83-2.8 | N/A |
| 834 | 2547 | 60dB | 0 dBm | -35dBm | -38.91-2.8 | N/A |
| 834 | 2547 | 70dB | 0 dBm | -45dBm | -48.07-2.8 | N/A |
| 846.5 | 2505 | 40dB | 0 dBm | -15dBm | N/A | -23.58-3.4 |
| 846.5 | 2505 | 50dB | 0 dBm | -25dBm | N/A | -33.44-3.4 |
| 846.5 | 2505 | 60dB | 0 dBm | -35dBm | N/A | -43.27-3.4 |
| 846.5 | 2505 | 70dB | 0 dBm | -45dBm | N/A | -53.03-3.4 |

The Correction Factor (CF) is proposed in [3] due to the power difference between the total IMD power and the IMD power within DL carrier modulation bandwidth. So the CF was considered to analyze the MSD levels in equation 1 and 2.

From the Table 2 and 3, we can see that the 3rd IMD level has quite an impact on the Band 5 DL frequency range (6th column) considering coupling loss of equal or less than 50 dB as shown in Eq.1.  And the 5th IMD level is also impacted on the self desense on Band 5, which level are analyzed as shown in Eq.2. For this analysis, we consider the Band 5 duplexer attenuation level is 50dB.

- Desense level for 2ULs UE = Conventional thermal noise level – (Thermal noise + Interference level of 3rd IMD products- CF)

= -104dBm – (10^(-10.4) +10^( -8.263)) = -104dBm – (-82.60 dBm)

= 21.40dB										  (Eq. 1)

- Desense level for 2ULs UE = Conventional thermal noise level – (Thermal noise + Interference level of 5th IMD products-CF)

= -104dBm – (10^(-10.4) +10^( -8.684)) = -104dBm – (86.76 dBm)

= 17.24dB										 (Eq. 2)

The 3rd IMD at 879MHz is impacted on self desense as 21.4dB and 5th IMD products at 891.5MHz has desensitized about 17.24dB on Band 5 DL frequency.

So, we have the following observation for PA component.

Observation1: When considering 50dB coupling loss in RF design, the generated IMD3 and IMD5 products in PA are not negligible on REFSENS of Band 5.

From the IMD measurements results and REFSENS analyses in active RF components, we can summarize the self desensitisation levels in Table 4 according to the coupling loss.

Table 4: Self-desensitization in Band 5 by coupling loss for 2ULs CA_5A-7A UE

| Coupling factor (dB) | Desensitization by PA(dB) |  |
| --- | --- | --- |
|  | IMD3 at 879MHz | IMD5 at 891.5MHz |
| 40 | 31.48 | 27.03 |
| 50 | 21.40 | 17.24 |
| 60 | 12.54 | 8.07 |
| 70 | 4.85 | 1.96 |

From the measurements results, we need to define MSD level for Band 5, but REFSENS of Band 7 did not impacted by high order IMD products. Therefore, we can provide additional observation as follow.

Observation2: When considering 50dB coupling loss in RF design, the generated IMD3 and IMD5 products by 2ULs CA_5A-7A do not impact on the REFSENS of Band 7.

- Conclusions

In this contribution we measured the high order IMD level on RF component to analyze the impact the self desense problems. The followings are the observations in this paper.

Observation1: When considering 50dB coupling loss in RF design, the generated IMD3 and IMD5 products in PA are not negligible on REFSENS of Band 5.

Observation2: When considering 50dB coupling loss in RF design, the generated IMD3 and IMD5 products by 2ULs CA_5A-7A do not impact on the REFSENS of Band 7.

The corresponding text proposal for TR36.860 is provided below.

- Reference

- R4-141259, “Higher order IMD and harmonic table for UE self-desensitization analysis,” Intel

- R4-140605, “More on IMD measurements for 2UL inter-band CA,” Media Tek Inc.

- R4-144031, “WF on 2UL inter-band CA MSD test configurations,” MediaTek

- R4-142096, “Measurements results of IMD4 level for 2ULs CA_1A-5A UE,” LG Electronics

- R4-143227, “TP for self-desense analysis for CA_5A-7A UE,” LG Electronics

- R4-141544, “Spectrum study on intermodulation analysis for 2 UL CA,” Intel

- TR 36.860 v0.9.0

- TS 36.101 v12.4.0

Text proposal for TR 36.860

********************* Start of text proposal to TR 36.860 Chapter 9.7.1.2 *****************

9.7.1.2	Co-existence studies for CA_5A-7A with 2ULs

Harmonic and intermodulation frequency ranges up to 3rd order are presented in Table 9.7.1.2-1

Table 9.7.1.2-1: CA_5A-7A UL harmonics and IMD products

| UE UL Carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 - 1698 |  | 5000 - 5140 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 - 2547 |  | 7500 - 7710 |  |
| 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 - 1746 |  | 3324 - 3419 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low - fy_high¦ | ¦2*fx_high - fy_low¦ | ¦2*fy_low - fx_high¦ | 2*fy_high - fx_low |
| IMD frequency limits (MHz) | 802 - 922 |  | 4151 - 4316 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4148 - 4268 |  | 5824 - 5989 |  |

Frequency limits for 5th order IMD are presented in Table 9.7.1.2-2

Table 9.7.1.2-2: CA_5A-7A 5th order UL IMD product frequency ranges

| ¦2*fx - 3*fy¦ | 5802 – 6062 MHz |
| --- | --- |
| ¦2*fx + 3*fy¦ | 9148 – 9408 MHz |
| ¦2*fy - 3*fx¦ | 2453 – 2668 MHz |
| ¦2*fy + 3*fx¦ | 7472 – 7687 MHz |
| ¦fx - 4*fy¦ | 9151 – 9456 MHz |
| ¦fx + 4*fy¦ | 10824 – 11129 MHz |
| ¦fy – 4*fx¦ | 726 – 896 MHz |
| ¦fy + 4*fx¦ | 5796 – 5966 MHz |

Based on table 9.7-1 and table 9.7.1.2-1, IMD3 may fall into own RX of band 5. 2nd and 3rd order IMD and harmonics may also fall into RX frequencies of bands 6, 18, 19, 26, 27, 28, 41, 42 and 44.

Based on table 9.7-1 and table 9.7.1.2-2, 5th order IMD and harmonics may also fall into own RX of bands 5 and 7 and additionally on Rx frequencies of bands 6, 12, 13, 14, 17, 18, 19, 38, 41 and 44.

In addition to E-UTRA bands also co-existence with GNSS and ISM frequencies was studied. GNSS and ISM frequencies which were included in the study are captured in Table 9.7.1.2-3.

Table 9.7.1.2-3: B5+B7 general IMD/Harmonics analysis to protect ISM bands and GNSS

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  |  |
| Galileo | 1559 | - | 1591 | Yes |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  |  |
| GPS | 1563 | - | 1587 | Yes |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe | H3, IMD5 |
|  | 2400 | - | 2494 | No | Asia | H3, IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe |  |
|  | 5470 | - | 5725 | Yes |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD5 |

      - Analysis of self desense due to high order IMD products

Based on table 9.7.1.2-1 and 9.7.1.2-2, IMD3 and IMD5 may fall into own Band 5 RX frequency. And IMD5 may fall into own Band 7 Rx frequency. However IMD4 does not fall into own Rx frequencies, thus has no impact on the self desense for CA_5A-7A UE.

To analyze the IMD products at PA output, we assumed the typical RF components values in CA_5A-7A as follows

  - Band 5 (5MHz CBW, Full RB), Band 7 (10MHz CBW, Full RB)

  - Aggregated maximum output power : 25dBm with 2dB tolerance

  - Tx/Rx RF component IL : 3.5dB

    - 3.5dB = sum(ANT SW: 0.5dB,  Diplexer :0.5dB, Duplexer: 2.5dB)

  - FE coupling loss : 40~70dB

  - Antenna loss :10dB

  - Diplexer Characteristics: 20dB for Band 5- Band 7 isolation

  - Duplexer Characteristics: Tx-Rx isolation: 50dB, Tx-Ant isolation : 35 dB

For 2UL inter-band CA_5A-7A, the leaked signal from Band 5 may fall into Band 7 own DL frequency and the leakage of Band 7 also impacted on the Band 5 own DL frequency as shown in Figure 9.7.1.3-1.

Figure 9.7.1.3-1. Example of input signal level to generate the high order IMD at RF components

In the previous analysis [4] of active components, we knew that the coupling loss in PA is more critical than the general interference signal by reversed RF path (blue line). If coupling loss is less than 50dB, and then the estimated leaked input signal level is higher than these values. So we can assume only coupling loss from 40~70dB ignoring the reversed RF path’s signal levels.

- Case1: Band 5 aggressor and Band 7 victim

- PA Active IMD (P1 point):

  - Signal level from B7 : 0dBm

  - Signal level from B5: -15, -25, -35 or -45dBm with coupling loss 40, 50, 60 or 70dB

The same approaches are applied for the opposite case that is the leaked signal from Band 7 may also have impact on the Band 5 own DL frequency.

- Case2: Band 7 aggressor and Band 5 victim

- PA Active IMD (P2 point):

  - Signal level from B5 : 0dBm

  - Signal level from B7: -15, -25, -35 or -45dBm with coupling loss 40, 50, 60 or 70dB

From the observation, we can measure the generated IMD products in P1 and P2 points as shown in figure 9.7.1.3-2 and Figure 9.7.1.3-3.

Figure 9.7.1.3-2. IMD measurement setup at RF components output in Case1

Figure 9.7.1.3-3. IMD measurement setup at RF component output in Case2

The measurement results of IMD products of PA output are shown in Table 9.7.1.3-1 and 9.7.1.3-2 according two test cases.

Table 9.7.1.3-1: IMD3 and IMD5 measurements results in PA of 2ULs CA_5A-7A UE in Case1

| Band 5 (MHz) | Band 7 (MHz) | Coupling Loss | B5 (aggressor) Power | B7 (victim) Power | Level of IM3 | Level at 2640 MHz (IMD5) |
| --- | --- | --- | --- | --- | --- | --- |
| 830 | 2565 | 40dB | -15dBm | 0 dBm | N/A | -64.74 |
| 830 | 2565 | 50dB | -25dBm | 0 dBm | N/A | -65.21 |
| 830 | 2565 | 60dB | -35dBm | 0 dBm | N/A | -65.67 |
| 830 | 2565 | 70dB | -45dBm | 0 dBm | N/A | - |

Table 9.7.1.3-2: IMD3 and IMD5 measurements results in PA of 2ULs CA_5A-7A UE in Case2

| Band 5 (MHz) | Band 7 (MHz) | Coupling loss | B5 (Victim) Power | B7 (aggressor) Power | Level at 879 MHz (IMD3) | Level at 891.5 MHz (IMD5) |
| --- | --- | --- | --- | --- | --- | --- |
| 834 | 2547 | 40dB | 0 dBm | -15dBm | -19.72-2.8 | N/A |
| 834 | 2547 | 50dB | 0 dBm | -25dBm | -29.83-2.8 | N/A |
| 834 | 2547 | 60dB | 0 dBm | -35dBm | -38.91-2.8 | N/A |
| 834 | 2547 | 70dB | 0 dBm | -45dBm | -48.07-2.8 | N/A |
| 846.5 | 2505 | 40dB | 0 dBm | -15dBm | N/A | -23.58-3.4 |
| 846.5 | 2505 | 50dB | 0 dBm | -25dBm | N/A | -33.44-3.4 |
| 846.5 | 2505 | 60dB | 0 dBm | -35dBm | N/A | -43.27-3.4 |
| 846.5 | 2505 | 70dB | 0 dBm | -45dBm | N/A | -53.03-3.4 |

The Correction Factor (CF) is proposed in [3] due to the power difference between the total IMD power and the IMD power within DL carrier modulation bandwidth. So the CF was considered to analyze the MSD levels in equation 9.7.1.3-1 and 9.7.1.3-2.

From the Table 9.7.1.3-1 and 9.7.1.3-2, we can see that the 3rd IMD level has quite an impact on the Band 5 DL frequency range (6th column) considering coupling loss of equal or less than 50 dB as shown in Eq. 9.7.1.3-1.  And the 5th IMD level is also impacted on the self desense on Band 5, which level are analyzed as shown in Eq. 9.7.1.3-2. For this analysis, we consider the Band 5 duplexer attenuation level is 50dB.

- Desense level for 2ULs UE = Conventional thermal noise level – (Thermal noise + Interference level of 3rd IMD products- CF)

= -104dBm – (10^(-10.4) +10^( -8.263)) = -104dBm – (-82.60 dBm)

= 21.40dB								     (Eq. 9.7.1.3-1)

- Desense level for 2ULs UE = Conventional thermal noise level – (Thermal noise + Interference level of 5th IMD products-CF)

= -104dBm – (10^(-10.4) +10^( -8.684)) = -104dBm – (86.76 dBm)

= 17.24dB								     (Eq. 9.7.1.3-2)

The 3rd IMD at 879MHz is impacted on self desense as 21.4dB and 5th IMD products at 891.5MHz has desensitized about 17.24dB on Band 5 DL frequency.

So, we have the following observation for PA component.

Observation1: When considering 50dB coupling loss in RF design, the generated IMD3 and IMD5 products in PA are not negligible on REFSENS of Band 5.

From the IMD measurements results and REFSENS analyses in active RF components, we can summarize the self desensitisation levels in Table 9.7.1.3-3 according to the coupling loss.

Table 9.7.1.3-3: Self-desensitization in Band 5 by coupling loss for 2ULs CA_5A-7A UE

| Coupling factor (dB) | Desensitization by PA(dB) |  |
| --- | --- | --- |
|  | IMD3 at 879MHz | IMD5 at 891.5MHz |
| 40 | 31.48 | 27.03 |
| 50 | 21.40 | 17.24 |
| 60 | 12.54 | 8.07 |
| 70 | 4.85 | 1.96 |

From the measurements results, we need to define MSD level for Band 5, but REFSENS of Band 7 did not impacted by high order IMD products. Therefore, we can provide additional observation as follow.

Observation2: When considering 50dB coupling loss in RF design, the generated IMD3 and IMD5 products by 2ULs CA_5A-7A do not impact on the REFSENS of Band 7.

******************** End of text proposal to TR 36.860 chapter 9.7.1.2 **********************