3GPP TSG RAN WG4 Meeting #72BIS	        R4-145792

# Singapore, Singapore, 6th  to 10th, October 2014

- Agenda Item:	7.11.1

- Source:	LG Electronics

- Title:	D2D-WAN capability for simultaneous Transmission and reception in intra-frequency and inter-frequency

Document for:	Approval

- Introduction

In the previous RAN4 meeting, UE coexistence analysis are studied and sent LS to RAN WG1 for the D2D co-existence results according to discovery signals and communication signal [1]. But the coexistence study was for interference analysis between inter-device UEs, the D2D-WAN capability has not consensus for simultaneous transmission and reception in intra-device UE.

In this contribution, we show our view for the D2D-WAN capability for simultaneous transmission and reception in intra-device based on two candidate D2D UE RF architectures. One of RF architecture for CA capable UE and the other RF architecture is single RF chain architecture such as LTE rel-8/9 UE.

- D2D UE candidate RF architecture

In UE-to-UE coexistence based on TR36.942, RAN WG4 only consider D2D UE’s impact to the legacy LTE system and LTE UE in inter-frequency into inter-device. However RAN WG4 did not consider that the legacy cellular UE/NW signal impacted to D2D UE receiving signal. So we should verify D2D-WAN UE capability in scenarios 3 and 4 in inter-device. And also all scenarios should be considered for determining D2D-WAN UE capability in intra-device.

Table 1: Coexistence Scenarios in inter-device

| Scenario | Aggressor | Victim | LTE Duplex | Comments |
| --- | --- | --- | --- | --- |
| 1 | D2D Tx | LTE UE Rx | TDD and FDD | Normal operation, may be some impact in narrow band Tx/Rx separation |
| 2 | D2D Tx | LTE UE Tx | TDD and FDD | LS out for co-existence study [1] |
| 3 | LTE UE Rx | D2D Rx | TDD and FDD | Reply LS sent to RAN1[2] (No critical impact to D2D Rx) |
| 4 | LTE UE Tx | D2D Rx | TDD and FDD | No verified in TR36.942, Critical impact to D2D Rx in intra-frequency, but may possible in inter-frequency with appropriate Gap btw. D2D Rx and WAN Tx |

For the D2D-WAN UE capability in intra-device, we consider two example D2D UE RF architectures as shown in Figure1.

In the figure, we show D2D UE RF example architectures based on the D2D-WAN operating frequency. Figure 1-(a) depicted two candidate UE RF architectures to support D2D-WAN operating in same frequency and Figure 1-(b) shows example UE RF architecture for D2D-WAN operation in inter-band frequency.

- D2D-WAN operation in Intra-frequency

(b) D2D-WAN operation in Inter-frequency

Figure 1: UE RF example architectures for D2D UE

For the D2D-WAN UE capability depend on the coexistence impacts on intra-device for all scenarios in intra-frequency. Therefore we can analyze as shown in Table2.

Table 2: Coexistence Scenarios in intra-device

| Scenario | Aggressor | Victim | LTE Duplex | Comments |
| --- | --- | --- | --- | --- |
| 1 | D2D Tx | LTE UE Rx | TDD and FDD | Normal operation, may be some impact in narrow band Tx/Rx separation |
| 2 | D2D Tx | LTE UE Tx | TDD and FDD | Possible to simultaneous D2D Tx and LTE Tx, the operation is similar to intra-NC CA transmission, Need to consider power imbalance. The D2D signal can be under the interference by high Tx WAN power. Need to guard gap or A-MPR to protect D2D signal |
| 3 | LTE UE Rx | D2D Rx | TDD and FDD | No critical impact to D2D Rx from LTE NW Tx. Need SC-FDMA decoding chain in uplink spectrum. |
| 4 | LTE UE Tx | D2D Rx | TDD and FDD | Impossible to simultaneous D2D Rx and LTE Tx, D2D REFSENS is quite impacted by LTE Tx. But may possible in inter-frequency with appropriate Gap btw. D2D Rx and WAN Tx |

Figure2 shows the coexistence scenario 1 of the simultaneous D2D Tx-WAN Rx in intra-frequency. We can see the D2D Tx-WAN Rx separation is less than legacy Tx-Rx separation in LTE system. In this case the WAN REFSENS could not maintain the current requirements in Band X since D2D signals can be leaked into WAN Rx band. However, if Tx-Rx separation is larger than 100MHz, the filter can be degraded the D2D interference levels in the WAN Rx band, then WAN REFSENS can be maintained. Therefore, the UE capability of simultaneous D2D Tx-WAN Rx is depend on each operating bands.

Figure 2: Simultaneous D2D Tx-WAN Rx in intra-frequency

Figure3 depicted the coexistence scenario 2 of the simultaneous D2D Tx-WAN Tx in intra-frequency. We can see the D2D Tx-WAN Tx is similar to intra-band non contiguous CA transmission. But we should consider the power imbalance between D2D signals and WAN signals. The D2D signal can be under the interference levels by high Tx WAN power. Therefore RAN WG4 needs to define guard gap or A-MPR to protect D2D signal as shown in Figure3.

Figure 3: Simultaneous D2D Tx-WAN Tx in intra-frequency

Figure4 displayed the coexistence scenario 3 of the simultaneous D2D Rx-WAN Rx in intra-frequency. The D2D REFSENS can be maintained due to large separation gap between WAN Rx band and D2D Rx band. The scenario 3 is can be supported in the architecture of Figure 1-(a).

The left UE architecture of Figure 1-(a) has additional D2D RF chain in UE, so it is quite complicated and can be increased UE cost. But the UE can support simultaneous D2D Rx and WAN Rx capability. On the other hand, the right UE architecture of Figure 1-(a) has only added switch and filter in UE architecture, so it is simple to implement and can be expected low cost, but the UE do not support simultaneous D2D Rx and WAN Rx capability.

Figure 4: Simultaneous D2D Rx-WAN Rx in intra-frequency

Figure 5 shows the scenario 4 of the simultaneous D2D Rx-WAN Tx in intra-frequency. We can see that D2D Rx signal is significant impacts from the WAN Tx because the Tx and Rx separation is less than 20MHz CBW, the WAN Tx leakages can be affected in D2D Rx band. If guard gap and additional power reduction for D2D protection defined, this operation can be possible. However we need to pros and cons between keep the coverage and reduce the coverage due to D2D operation.

Figure 5: Simultaneous D2D Rx-WAN Tx in intra-frequency

From the D2D-WAN capability analysis in intra-device based on coexistence scenarios, we can propose as follow

Proposal 1: UE capability of simultaneous D2D Tx-WAN Rx is depending on each operating bands.

Proposal 2: UE capability of Simultaneous D2D Tx-WAN Tx is possible, but need to investigate for the in-band emission due to power imbalance.

Proposal 3: UE capability of Simultaneous D2D Rx-WAN Rx is possible, if additional D2D Rx RF chain, but D2D UE only has single Rx RF chain, it is not possible.

Proposal 4: UE capability of Simultaneous D2D Rx-WAN Tx is impossible due to short guard gap between WAN-Tx and D2D Rx.

- Conclusion

In this contribution we analyzed D2D-WAN UE capability in intra-device according to the candidate UE RF architectures. The followings are proposals in this paper.

Proposal 1: UE capability of simultaneous D2D Tx-WAN Rx is depending on each operating bands.

Proposal 2: UE capability of Simultaneous D2D Tx-WAN Tx is possible, but need to investigate for the in-band emission due to power imbalance.

Proposal 3: UE capability of Simultaneous D2D Rx-WAN Rx is possible, if additional D2D Rx RF chain, but D2D UE only has single Rx RF chain, it is not possible.

Proposal 4: UE capability of Simultaneous D2D Rx-WAN Tx is impossible due to short guard gap between WAN-Tx and D2D Rx.

- Reference

- R4-145560, “LS on D2D co-existence in adjacent bands,” Ericsson

- R4-145512, “Reply LS on D2D Multicarrier capabilities,” Qualcomm

- R4-145793, “Consideration of D2D RF requirements,” LG Electronics

- R4-143087, “Impact of D2D In-band emissions on RF requirements,” Ericsson