3GPP TSG-RAN WG4 Meeting #72bis	 R4-146075

# Singapore, Singapore, 6 – 10 Oct, 2014

Agenda Item:	7.10.3

Source: 	Samsung

Title: 	Test cases design for SCE CSI performance

Document for:	Discussion

# 1	Introduction

A new work item “Small cell enhancement” was introduced in Rel-12. 256QAM and small cell on/off were the key features for this WI. From RAN4 point of view, 256QAM is mostly related to UE demodulation performance and CSI performance part.

In RAN4#70bis meeting, work plan on SCE for RAN4 work was agreed in [1] and a way forward was agreed in last meeting to evaluate such options for CQI test:

- Option 1: CQI definition PUCCH 1-0 single layer, CQI definition PUCCH 1-1 dual layer and CQI fading test TM9 PUSCH 3-1;

- Option 2: CQI definition PUCCH 1-0 single layer and CQI fading test TM9 PUSCH 3-1;

- Option 3: CQI definition PUCCH 1-1 dual layer and CQI fading test TM9 PUSCH 3-1;

- Option 4 : PUCCH 1-0 static test for TM1 and PUCCH 1-1 static test for TM9; no CQI fading test

In this contribution, initial simulation results and analysis for 256QAM CSI test were supplied.

# Analysis

### 2.1 Overview

As shown in figure 1 below, new CQI index table was agreed in RAN1 to support 256QAM. New CQI test cases were needed to verify UE supporting new CQI table and CQI calculation for 256QAM.

Figure1: New CQI index table including 256QAM

From UE implementation point of view, this new CQI index table have effect on effective SNR calculation and CQI mapping between effective SNR and CQI index. The purpose of fading CQI test cases was to verify whether UE can effectively track channel variance in time domain and frequency domain or interference variance in frequency domain. Static CQI test cases were introduced to verify UE correctly following CQI definition when implementation CQI calculation. Based above considerations, there is no need to introduce fading CQI test since static CQI test cases have already severed test purpose well. Furthermore, both CRS based and CSI-RS based on feedback modes were needed to be introduced.

Proposal1: Prefer option 4 or option 3 to introduce CQI test cases.

In below sections, we verified the feasibility of reusing the existing test parameters and methodology by evaluation.

### 2.2 PUCCH 1-0 static CQI test (TM1)

Simulation assumption

The existing test parameters and methodology in section 9.2.1.1 (FDD) were reused here, as summarized in table 1 below.

Table 1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |  |
| SNR (Note 2) |  | dB | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| Note 1: Reference measurement channel according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Simulation results

Reporting CQI spread and BLER performance for PUCCH1-0 test were summarized in table 2 below. Based on alignment simulation results, we observed that:

- Without considering Rx impairment loss, reporting CQI values will cover 256QAM under SNR range 17~24dB.

- Both CQI distribution and BLER requirements in existing static CQI test can be satisfied with proper UE implementation.

Table 2 Reporting Spread of the Wideband CQI and BLER performance for PUCCH1-0 test

| SNR | Median CQI Index | Percentage of CQI index in {median CQI1 -1, median CQI1 +1} | BLER Using Reported CQI Median-1 | BLER Using Reported CQI Median | BLER Using Reported CQI Median+1 |
| --- | --- | --- | --- | --- | --- |
| 17 | 12 | 1 | 0.0000 | 0.0908 | 1.0000 |
| 18 | 12 | 1 | 0.0000 | 0.0000 | 1.0000 |
| 19 | 13 | 1 | 0.0000 | 0.0050 | 1.0000 |
| 20 | 13 | 1 | 0.0000 | 0.0000 | 0.1242 |
| 21 | 14 | 1 | 0.0000 | 0.0008 | 1.0000 |
| 22 | 14 | 1 | 0.0000 | 0.0000 | 1.0000 |
| 23 | 14 | 1 | 0.0000 | 0.0000 | 1.0000 |
| 24 | 14 | 1 | 0.0000 | 0.0000 | 1.0000 |
| 25 | 15 | 1 | 0.0000 | 0.0441 | NA |

Proposal2: SNR points {19, 20} dB can be selected for PUCCH 1-0 test as high SNR test points to cover 256QAM CQI index.

### 2.3 PUCCH 1-1 static CQI test (TM4)

Simulation Assumption

The existing test parameters and methodology in section 9.2.2.1 (FDD) were reused here, as summarized in table 3 below.

Table3: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 010000 |  |  |  |
| SNR (Note 2) |  | dB | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| ri-ConfigIndex |  |  | 1 (Note 3) |  |  |  |
| Note 1: Reference measurement channel according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: It is intended to have UL collisions between RI reports and HARQ-ACK, since the RI reports shall not be used by the eNB in this test. |  |  |  |  |  |  |

Simulation results

Reporting CQI spread and BLER performance for both codeword 0 and codeword 1 were summarized in table 4 below. Based on alignment simulation results, we observed that:

- Without considering Rx impairment loss, reporting CQI values will cover 256QAM under SNR range 20~24dB.

- Both CQI distribution and BLER requirements in existing static CQI test can be satisfied with proper UE implementation.

Table 4 Reporting Spread of the Wideband CQI and BLER performance for PUCCH1-1 test

| SNR | Median CQI Index |  | Percentage of CQI index in {median CQI1 -1, median CQI1 +1} |  | BLER Using Reported CQI Median-1 |  | BLER Using Reported CQI Median |  | BLER Using Reported CQI Median+1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 |
| 20 | 12 | 12 | 1 | 1 | 0.0000 | 0.0000 | 0.0012 | 0.0006 | 1.0000 | 1.0000 |
| 21 | 12 | 12 | 1 | 1 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 |
| 22 | 13 | 13 | 1 | 1 | 0.0000 | 0.0000 | 0.0018 | 0.0012 | 1.0000 | 1.0000 |
| 23 | 14 | 14 | 1 | 1 | 0.0000 | 0.0000 | 0.6352 | 0.5905 | 1.0000 | 1.0000 |
| 24 | 14 | 14 | 1 | 1 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.5830 | 0.5790 |

Proposal3: SNR points {22, 23} dB can be selected for PUCCH 1-1 test (TM4) as high SNR test points to cover 256QAM CQI index.

# Conclusion

In this contribution, firstly some initial considerations and analysis for 256QAM CSI test were given. Then we evaluate the feasibility of reusing the existing test parameters and methodology for CRS base static CQI test.

Such observations were given based on the evaluation results:

Observation1: It’s feasible to reuse existing test parameters and test metrics for static CQI test 1-0 (TM1).

Observation2: It’s feasible to reuse existing test parameters and test metrics for dual CWs static CQI test 1-1 (TM4).

Furthermore, such proposals were given:

Proposal1: Prefer option 4 or option 3 to introduce CQI test cases.

Proposal2: If static CQI PUCCH 1-0 test was introduced, SNR points {19, 20} dB can be selected as high SNR test points to cover 256QAM CQI index.

Proposal3: If static CQI PUCCH 1-1 test(TM4) was introduced, SNR points {22, 23} dB can be selected as high SNR test points to cover 256QAM CQI index.

# Reference

- R4-142303, “Work plan for RAN4 work on Small Cell Enhancement”, Huawei

- R4-145377, “WF on 256QAM CSI test”, Huawei