3GPP TSG-RAN WG4 (Radio) Meeting #72bis	R4-146158

Singapore, 6 – 10 October 2013

Agenda Item:	8.36.2.1

Source: 	Alcatel-Lucent, AT&T

Title: 	TP for TR 36.853: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Contiguous Carrier Aggregation of Band 2

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 29 was approved in RAN#65 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for Band 2 intra-band contiguous CA (2DL/1UL) scenario.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting contiguous CA of this band to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the 3 Band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<New section>

##### 6.x.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of contiguous Band 2 DL carriers can be calculated as shown in Table 6.x.1-1 below:

Table 6.x.1-1: Band 2 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 1930 | 1990 |
| 2nd harmonics frequency limits (MHz) | 3860 | 3980 |
| 3rd harmonics frequency limits (MHz) | 5790 | 5970 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -60 | 60 |
| 3rd order IMD products | (2* f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 1870 | 2050 |
| 3rd order IMD products (with maximum channel bandwidth) | (f-low – f-BWmax) | (f-high + f-BWmax) |
| IMD frequency limits (MHz) | 1890 | 2030 |

It can be seen from Table 6.x.1-1 that the 3rd IMD products of BS supporting CA of Band 2 may fall into the BS receive band of Bands 1, 2, 23, 25, 33, 34, 35, 36, 37 and 39. Note that the calculation in Table 6.x.1-1 (except the last row) assumes the BS is transmitting with the whole 60 MHz DL frequency of Band 2. But even if the BS is only transmitting up to 20 MHz (20 + 20 MHz) contiguous carriers in Band 2 as stated in the WIDS, the 3rd IMD products may still fall into the BS receive band of these bands as shown in the last row in Table 6.x.1-1. However, the 3rd IMD products will not fall into the BS own (up to 40 MHz) receive block within Band 2.

Note that Bands 1, 33, 34 and 39 are not intended for use in the same geographical area as Band 2. Moreover, co-location of Band 2 transmitter and Band 35, 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Consequently, the focus here will be on the 3rd order IMD products falling into Bands 2, 23 and 25.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the Band 2 BS transmit filter and the Band 2, 23 or 25 BS receive filter would not be able to provide substantial attenuation to reduce the IMD interference generated within the Band 2, 23 or 25 BS receive band to well below the receiver noise floor to prevent receiver desensitization, as there is not a large frequency gap between Band 2 DL and Band 2, 23 or 25 UL.

Therefore, it is recommended that Band 2 BS transmitters supporting contiguous CA should not share the same antenna with other Band 2, 23 or 25 BS receiver to prevent BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of section>

# References

[1]	RP-141448, “New WI proposal: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 29”, AT&T.

[2]	R4-146157, “Harmonics and Intermodulation Products caused by LTE Advanced Contiguous Carrier Aggregation of Band 2”, Alcatel-Lucent, AT&T.

[3]	RP-141217, “TR 36.853 v2.0.0 on LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)”, AT&T.