3GPP TSG-RAN WG4 Meeting #72bis 	R4-146437

# Singapore, 6th – 10th October, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.133 | CR | 1376 | rev |  | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on measurement requirements change to monitor additional carriers in Idle/CELL_PCH_URA_PCH state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UTRA_Incmon-Core |  |  |  |  |  |  | Date: |  | 2014-08-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | CR to introduce requirements for increased carrier monitoring |  |  |  |  |  |  |  |  |
| Summary of change: |  | CR with changes to idle requirements to introduce increased carrier monitoring |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Increased UE carrier monitoring requirements for idle mode are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

The main general definitions strictly related to the Transmission and Reception characteristics but important also for the present document can be found in [3] for UE FDD, in [4] for BS FDD, in [5] for UE TDD, in [6] for BS TDD.

Node B:A logical node responsible for radio transmission / reception in one or more cells to/from the User Equipment. Terminates the Iub interface towards the RNC

Power Spectral Density: The units of Power Spectral Density (PSD) are extensively used in this document. PSD is a function of power versus frequency and when integrated across a given bandwidth, the function represents the mean power in such a bandwidth. When the mean power is normalised to (divided by) the chip-rate it represents the mean energy per chip. Some signals are directly defined in terms of energy per chip, (DPCH_Ec, Ec, OCNS_Ec and S-CCPCH_Ec) and others defined in terms of PSD (Io, Ioc, Ior and Îor). There also exist quantities that are a ratio of energy per chip to PSD (DPCH_Ec/Ior, Ec/Ior etc.). This is the common practice of relating energy magnitudes in communication systems.
It can be seen that if both energy magnitudes in the ratio are divided by time, the ratio is converted from an energy ratio to a power ratio, which is more useful from a measurement point of view. It follows that an energy per chip of X dBm/3,84 MHz can be expressed as a mean power per chip of X dBm. Similarly, a signal PSD of Y dBm/3,84 MHz can be expressed as a signal power of Y dBm.

MBSFN cluster: Set of cells operating in MBSFN mode providing only MBMS service in PtM mode and seen as one cell by a UE.

Adjacent Frequency: A downlink frequency in the inter-frequency cell info list that is within 5 MHz from the intra frequency. It is explicitly indicated by UTRAN to UE via upper layer signaling.

Primary uplink frequency: If a single uplink frequency is configured for the UE, then it is the primary uplink frequency. In case more than one uplink frequency is configured for the UE, then the primary uplink frequency is the frequency on which the E-DCH corresponding to the serving E-DCH cell associated with the serving HS-DSCH cell is transmitted. The association between a pair of uplink and downlink frequencies is indicated by higher layers.

Secondary uplink frequency: A secondary uplink frequency is a frequency on which an E-DCH corresponding to a serving E-DCH cell associated with a secondary serving HS-DSCH cell is transmitted. The association between a pair of uplink and downlink frequencies is indicated by higher layers.

Activated Uplink Frequency: For a specific UE, an uplink frequency is said to be activated if the UE is allowed to transmit on that frequency. The primary uplink frequency is always activated when configured while a secondary uplink frequency has to be activated by means of an HS-SCCH order in order to become activated.

Normal Performance Group: The group of inter-frequency carriers and inter-RAT carriers is divided into two groups. The group which has a better delay performance compared to the other group is refered to as the normal performance group

Reduced Performance Group: The group of inter-frequency carriers and inter-RAT carriers is divided into two groups. The group which has worse delay performance compared to the other group is refered to as the reduced performance group

## <End of Changes>

## <Start of Changes>

### 4.2.2	Requirements

In the following sections, Thigher_priority_search  is defined as (60 * Nlayers) seconds, where Nlayers is the total number of configured higher priority E-UTRA, UTRA FDD and UTRA TDD carrier frequencies and is additionally increased by one if one or more groups of GSM frequencies is configured as a higher priority.

The reselection performance for different carriers may be configured by higher layers to be either normal or reduced. If higher layer signalling is not present, then a default configuration is used by the UE to determine which layers have normal and reduced performance. The following definitions are used in the performance requirements:

Ncarrier-1 : Total number of interfrequency carriers in the interfrequecny neighbour cell info list

Nnorm,inter : Number of interfrequency carriers to be searched for and measured with normal performance

Nreduced,inter = (Ncarrier-1- Nnorm,inter,)

Kcarrier : Total number of configured E-UTRA carriers in the neighbour cell list

Knorm,E-UTRA : Number of UTRA FDD carriers to be searched for and measured with normal performance

Kreduced,E-UTRA =( Kcarrier - Knorm,E-UTRA)

The requirements in sections 4.2.2.3 and  4.2.2.5a, apply provided that Nnorm,inter ≤2 and, Nnorm,E-UTRA≤4. In case the limits for the number of  normal performance carriers is exceeded considering the broadcast neighbour cell list, and the bands supported by the UE, the UE shall measure at least 2 interfrequency carriers, and 4 E-UTRA carriers with normal performance. UE which do not indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall set Nreduced,inter = 0 and Kreduced,E-UTRA =0.

If no information is signalled on which carriers have normal performance, and which carriers have reduced performance, then the default configuration is that the first 2 UTRA carriers in the interfrequency neighbour cell info list  and the first 4 E-UTRA carriers in the interRAT neighbour cell info list shall meet normal performance requirements, and any additional carriers in the neighbour lists within the UEs measurement capability as defined in section  4.2.2.8 shall meet reduced performance requierments.

## <End of Changes>

## <Start of Changes>

#### 4.2.2.3	Measurements of inter-frequency FDD cells

If priority information for UTRA FDD carrier frequencies is provided in the measurement control systems information and SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 then the UE shall search for any higher priority UTRA inter-frequency cells at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2. If the UE is not in CELL_PCH, URA_PCH or IDLE state then the UE shall search for UTRA FDD layers of higher priority within Thigher_layer_start upon entering into any of these states. If 1 second has not elapsed since the UE camped on the current serving cell when the UE enters into any of these states, Thigher_layer_start is one DRX cycle plus 1 second; otherwise Thigher_layer_start is one DRX cycle. If higher priority UTRA cells are found by the higher priority search, they shall be measured at least every (Ncarrier-1) * TmeasureFDD,.  If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell. If priority information for UTRA FDD carrier frequencies is provided in the measurement control systems information and SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2, the rules defined below apply irrespective of the priority of the inter-frequency layer

The UE shall measure CPICH Ec/Io and CPICH RSCP at least every Nnorm, inter * TmeasureFDD for the normal performance group carriers and Nreduced, inter * 6 * TmeasureFDD (see table 4.1) for the reduced performance group carriers for inter-frequency cells that are identified and measured according to the measurement rules. The parameter Nnorm, inter is the number of carriers in the normal performance group and Nreduced, inter is the number of carriers in the reduced performance group used for FDD cells as defined in section 4.2.2. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least TmeasureFDD/2.

If CPICH Ec/Io is used as measurement quantity for cell reselection, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-frequency cell has met the reselection criteria in [1] within Nnorm, inter * TevaluateFDD for the normal performance group carriers and Nreduced, inter * 6 * TmeasureFDD (see table 4.1) for the reduced performance group carriers from the moment the inter-frequency cell met the reselection criteria by at least 3 dB provided that Treselection timer is set to zero. For non-identified inter-frequency cells, the filtering shall be such that the UE shall be capable of evaluating that inter-frequency cell belonging to the normal performance group has met the reselection criteria in [1] within 30 s and inter-frequency cell belonging to the reduced performance group has met the reselection criteria in [1] within 180 s from the moment the inter-frequency cell met the reselection criteria by at least 3 dB provided that Treselection timer is set to zero.

If CPICH RSCP is used as measurement quantity for cell reselection, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-frequency cell has met the reselection criteria in [1] within Nnorm, inter * TevaluateFDD for the normal performance group carriers and Nreduced, inter * 6 * TevaluateFDD for the reduced performance group carriers from the moment the inter-frequency cell met the reselection criteria by at least 5 dB for reselection based on ranking, or [6]dB for reselection based on absolute priorities provided that Treselection timer is set to zero. For non-identified inter-frequency cells, the filtering shall be such that the UE shall be capable of evaluating that inter-frequency cell belonging to the normal performance group has met the reselection criteria in [1] within 30 s and inter-frequency cell belonging to the reduced performance group has met the reselection criteria in [1] within 180 s from the moment the inter-frequency cell met the reselection criteria by at least 5 dB dB for reselection based on ranking, or [6]dB for reselection based on absolute priorities provided that Treselection timer is set to zero.

If Treselection timer has a non zero value and the inter-frequency cell meets the reselection criteria in [1], the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell fulfils the reselection criteria within this duration, then the UE shall reselect that cell. UE which do not indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall set Nreduced,inter = 0.

## <End of Changes>

## <Start of Changes>

#### 4.2.2.5a	Measurements of inter-RAT E-UTRA cells

The UE shall be able to identify new E-UTRA cells and perform RSRP and RSRQ measurements of identified E-UTRA cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 then the UE shall search for E-UTRA layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 4.2.2. If the UE is not in CELL_PCH, URA_PCH or IDLE state then the UE shall search for E-UTRA layers of higher priority within Thigher_layer_start upon entering into any of these states. If 1 second has not elapsed since the UE camped on the current serving cell when the UE enters into any of these states, Thigher_layer_start is one DRX cycle plus 1 second; otherwise Thigher_layer_start is one DRX cycle. The minimum rate at which the UE is required to search for and measure such layers may be reduced in this scenario to maintain UE battery life.

If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 then the UE shall search for and measure E-UTRA frequency layers of higher or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below for a lower priority layers.

The UE shall be able to evaluate whether a new detectable lower priority inter-RAT E-UTRA cell meets the reselection criteria defined in [1] within Knorm, EUTRA * TdetectE-UTRA for the normal performance group and Kreduced, EUTRA * 6 * TdetectE-UTRA for the reduced performance group, where TdetectE-UTRA is given in Table 4.2, if E-UTRA carrier frequency information is provided in the inter-RAT measurement control system information when Treselection=0 provided that the reselection criteria is met by at least 6dB for RSRP reselections or 4dB for RSRQ reselections. The parameter Knorm, EUTRA is the number of E-UTRA carrier frequencies in the normal performance group and the parameter Kreduced, EUTRA is the number of E-UTRA carrier frequencies in the reduced performance group indicated in the inter-RAT measurement control system information as defined in section 4.2.2. An inter RAT E-UTRAN cell is considered to be detectable if:

-	RSRP|dBm according to Annex B.1.1 for a corresponding Band

The UE shall measure RSRP and RSRQ at least every Knorm, EUTRA * TmeasureE-UTRA for normal performance group and Kreduced, EUTRA * TmeasureE-UTRA for reduced performance group as defined in table 4.2 for identified E-UTRA cells.

When higher priority cells are found by the higher priority search, they shall be measured at least every TmeasureE-UTRA. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a E-UTRA carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

If the UE detects on a E-UTRA carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least TmeasureE-UTRA/2.

RSRP and RSRQ measurements of E-UTRA cells shall not be filtered over a longer period than that specified in Table 4.2.

The UE shall not consider an E-UTRA neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-RAT E-UTRA cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the E-UTRA cell has met reselection criterion defined TS 36.304 within Knorm, EUTRA * TevaluateEUTRA for normal performance group and Kreduced, EUTRA * TevaluateE-UTRA for reduced performance group as specified in table 4.2 provided that the reselection criteria is met by at least 6dB for RSRP reselections or 4dB for RSRQ reselections and the Treselection timer is set to zero. UE which do not indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall set Kreduced,E-UTRA =0.

## <End of Changes>

## <Start of Changes>

#### 4.2.2.8	Number of cells in cell lists

For idle mode cell re-selection purposes, the UE shall be capable of monitoring:

-	32 intra-frequency cells (including serving cell), and

-	32 inter-frequency cells, including

-	FDD cells on maximum 2 additional carriers, and

-	Depending on UE capability, TDD cells distributed on up to 3 TDD carriers, and

-	Depending on UE capability, 32 GSM cells distributed on up to 32 GSM carriers, and

-	Depending on UE capability, up to 4 E-UTRA FDD carriers, and

-	Depending on UE capability up to 4 E-UTRA TDD carriers

as indicated in cell information lists sent in system information (BCCH).

In addition to the requirements defined above, a UE supporting E-UTRA measurements in RRC_IDLE state shall be capable of monitoring a minimum total of at least 8 carrier frequency layers, including the intra-frequency serving layer and comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM layers (one GSM layer corresponds to 32 cells).

UE which indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall be capable of monitoring at least

80 inter-frequency cells, including

-	5 FDD E-UTRA inter-frequency carriers with up to 32 cells per carrier.

UE which indicate support for increased UE carrier monitoring E-UTRA (either default or signalled) according to capabilities in [16] shall be capable of monitoring at least

-	Depending on UE capability, 8 FDD E-UTRA carriers, and

-	Depending on UE capability, 8 TDD E-UTRA carriers, and

In addition to the requirements defined above, a UE supporting E-UTRA measurements in RRC_IDLE state and supporting Increased UE carrier monitoring UTRA (either default or signalled) or increased UE carrier monitoring E-UTRA(either default or signalled) ) according to capabilities in [16] shall be capable of monitoring a total of at least 13 carrier frequency layers, which includes serving layer, comprising of any above defined combination of of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM layers (one GSM layer corresponds to 32 cells).

## <End of Changes>