3GPP TSG-RAN WG4 Meeting #73	 R4-147010

# San Francisco, CA, USA, 17 – 21 Nov, 2014

Agenda Item:	7.9.4.1

Source: 	Samsung

Title: 	Simulation results for DRS-RSRQ measurement

Document for:	Discussion

1	Introduction

Based on RAN1 agreements in RAN1#78[1], DRS based RSRQ measurement was introduced for small cell discovery with definition as below:

- For DRS-based RSSI measurement,

  - Discovery RSSI (DRSSI) is measured over all OFDM symbols in DL part of measurement subframes within DRS occasion

- For DRS-based RSRQ measurement,

  - DRSRQ = N×DRSRP/DRSSI, where N is the number of RB’s of the DRSSI measurement bandwidth.

  - CSI-RS-based RSRQ measurement is not specified

In last RAN4 meeting, it was agreed that for CRS based RSRP measurement [2]:

- CRS based RSRP measurement assuming keeping the Rel-8 RSRP measurement accuracy unchanged when Es/Iot≥-6dB

- CRS based RRM measurement time:

| Measurement BWs | Number of DRS occasions (i×160ms) |
| --- | --- |
| ≥6PRB | 5 |
| ≥25PRB | 3 |

In this contribution, we provide evaluation results for CRS based RSRQ measurement and verify the feasibility of applying above agreements for DRS-RSRQ measurement.

- Analysis

2.1 Simulation assumption

The detailed simulation assumption was agreed in [3].

Several cases with different combination of parameters as listed below are evaluated.

- SNR: {-6,-3,0}dB

- Measurement bandwidths: {6RB,25RB}

- Fading channel:AWGN,EPA5Hz,ETU30Hz

- Measure period: i*160ms (i=1,3,5)

- DRS occasion:1SF

2.2 Simulation results

Absolute DRS-RSRQ

The detailed simulation results with the 5-percentile, 50-percentile, 95-percentile distribution and 90% absolute DRS-RSRQ accuracy are given in table 1 and table 2 for 6RB and 25RB cases.

Table 1: Absolute DRS-RSRQ accuracy results under 6RB

|  | AWGN |  |  |  |  | EPA5Hz |  |  |  |  | ETU30Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 5% | 50% | 95% | 5%~95% | 90% | 5% | 50% | 95% | 5%~95% | 90% | 5% | 50% | 95% | 5%~95% | 90% |
| Number of samples =1(160ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -1.4 | 1.8 | 3.5 | 4.9 | 3.5 | -2.4 | 0.8 | 5.3 | 7.7 | 5.3 | -2.8 | 0.5 | 3.6 | 6.4 | 3.6 |
| -3 | -1.1 | 1.1 | 2.4 | 3.5 | 2.4 | -2.1 | 0.3 | 3.3 | 5.4 | 3.3 | -2.7 | -0.1 | 2.1 | 4.8 | 2.7 |
| 0 | -0.7 | 0.7 | 1.7 | 2.4 | 1.7 | -1.7 | 0.1 | 2 | 3.6 | 2 | -2.4 | -0.3 | 1.2 | 3.5 | 2.4 |
| Number of samples =3(480ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -0.5 | 1.3 | 2.7 | 3.2 | 2.7 | -2.3 | 0.3 | 2.1 | 4.4 | 2.3 | -2.8 | 0 | 1.8 | 4.6 | 2.8 |
| -3 | -0.4 | 0.8 | 1.8 | 2.1 | 1.8 | -1.3 | 0.1 | 1.3 | 2.6 | 1.3 | -1.8 | -0.2 | 1 | 2.8 | 1.8 |
| 0 | -0.2 | 0.5 | 1.2 | 1.4 | 1.2 | -0.8 | 0.1 | 0.8 | 1.6 | 0.8 | -1.4 | -0.3 | 0.5 | 1.9 | 1.4 |
| Number of samples =5(800ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -0.1 | 1.4 | 2.4 | 2.5 | 2.4 | -1.7 | 0.2 | 1.6 | 3.3 | 1.7 | -2 | -0.1 | 1.3 | 3.3 | 2 |
| -3 | 0 | 0.9 | 1.6 | 1.6 | 1.6 | -0.9 | 0.1 | 1 | 1.9 | 1 | -1.4 | -0.3 | 0.7 | 2.1 | 1.4 |
| 0 | -0.1 | 0.4 | 1.1 | 1.2 | 1.1 | -0.6 | 0 | 0.6 | 1.2 | 0.6 | -1.1 | -0.3 | 0.3 | 1.4 | 1.1 |

Table 2: Absolute DRS- RSRQ accuracy results under 25RB

|  | AWGN |  |  |  |  | EPA5Hz |  |  |  |  | ETU30Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 5% | 50% | 95% | 5%~95% | 90% | 5% | 50% | 95% | 5%~95% | 90% | 5% | 50% | 95% | 5%~95% | 90% |
| Number of samples =1(160ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -0.9 | 0.7 | 2 | 2.9 | 2 | -1.7 | 0.2 | 2.2 | 3.9 | 2.2 | -2.3 | -0.1 | 1.5 | 3.8 | 2.3 |
| -3 | -0.6 | 0.4 | 1.3 | 1.9 | 1.3 | -1.2 | 0.1 | 1.3 | 2.5 | 1.3 | -1.8 | -0.3 | 0.8 | 2.6 | 1.8 |
| 0 | -0.4 | 0.3 | 0.9 | 1.2 | 0.9 | -0.8 | 0 | 0.8 | 1.6 | 0.8 | -1.5 | -0.4 | 0.4 | 1.9 | 1.5 |
| Number of samples =3(480ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -0.2 | 0.7 | 1.5 | 1.7 | 1.5 | -0.9 | 0.1 | 1.1 | 2 | 1.1 | -1.4 | -0.2 | 0.7 | 2.1 | 1.4 |
| -3 | -0.2 | 0.4 | 0.9 | 1.1 | 0.9 | -0.6 | 0.1 | 0.6 | 1.2 | 0.6 | -1.1 | -0.4 | 0.3 | 1.4 | 1.1 |
| 0 | -0.1 | 0.3 | 0.6 | 0.7 | 0.6 | -0.4 | 0 | 0.4 | 0.8 | 0.4 | -1 | -0.4 | 0.1 | 1 | 1 |
| Number of samples =5(800ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | 0 | 0.7 | 1.3 | 1.3 | 1.3 | 0.1 | 0.8 | 1.5 | 0.8 | -0.7 | -1.1 | -0.2 | 0.5 | 1.6 | 1.1 |
| -3 | 0 | 0.4 | 0.8 | 0.9 | 0.8 | 0.1 | 0.5 | 0.9 | 0.5 | -0.4 | -1 | -0.4 | 0.2 | 1.1 | 1 |
| 0 | 0 | 0.3 | 0.5 | 0.6 | 0.5 | 0 | 0.3 | 0.6 | 0.3 | -0.3 | -0.8 | -0.4 | -0.1 | 0.8 | 0.8 |

Relative DRS-RSRQ

The detailed simulation results with the 5-percentile, 50-percentile, 95-percentile distribution and 90% relative DRS-RSRQ accuracy are given in table 3 and table 4 for 6RB and 25RB cases.

Table 3: Relative DRS-RSRQ accuracy results under 6RB

|  | AWGN |  |  |  |  | EPA5Hz |  |  |  |  | ETU30Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 5% | 50% | 95% | 5%~95% | 90% | 5% | 50% | 95% | 5%~95% | 90% | 5% | 50% | 95% | 5%~95% | 90% |
| Number of samples =1(160ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -1.7 | 0.2 | 2.2 | 3.9 | 2.2 | -5.4 | 0 | 5.5 | 10.9 | 5.5 | -4.6 | 0 | 4.5 | 9 | 4.6 |
| -3 | -1.2 | 0.1 | 1.3 | 2.5 | 1.3 | -3.9 | 0 | 3.9 | 7.9 | 3.9 | -3.4 | 0 | 3.4 | 6.8 | 3.4 |
| 0 | -0.8 | 0 | 0.8 | 1.6 | 0.8 | -2.7 | 0 | 2.7 | 5.4 | 2.7 | -2.5 | 0 | 2.5 | 5 | 2.5 |
| Number of samples =3(480ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -0.9 | 0.1 | 1.1 | 2 | 1.1 | -3.2 | 0 | 3.2 | 6.4 | 3.2 | -3.3 | 0 | 3.2 | 6.5 | 3.3 |
| -3 | -0.6 | 0.1 | 0.6 | 1.2 | 0.6 | -1.9 | 0 | 1.8 | 3.7 | 1.9 | -2 | 0 | 2 | 4 | 2 |
| 0 | -0.4 | 0 | 0.4 | 0.8 | 0.4 | -1.2 | 0 | 1.2 | 2.4 | 1.2 | -1.4 | 0 | 1.3 | 2.7 | 1.4 |
| Number of samples =5(800ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -0.7 | 0.1 | 0.8 | 1.5 | 0.8 | -2.4 | 0 | 2.4 | 4.7 | 2.4 | -2.4 | 0 | 2.5 | 4.9 | 2.5 |
| -3 | -0.4 | 0.1 | 0.5 | 0.9 | 0.5 | -1.3 | 0 | 1.3 | 2.6 | 1.3 | -1.5 | 0 | 1.5 | 2.9 | 1.5 |
| 0 | -0.3 | 0 | 0.3 | 0.6 | 0.3 | -0.9 | 0 | 0.9 | 1.7 | 0.9 | -1 | 0 | 1 | 2 | 1 |

Table 4: Relative DRS-RSRQ accuracy results under 25RB

|  | AWGN |  |  |  |  | EPA5Hz |  |  |  |  | ETU30Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 5% | 50% | 95% | 5%~95% | 90% | 5% | 50% | 95% | 5%~95% | 90% | 5% | 50% | 95% | 5%~95% | 90% |
| Number of samples =1(160ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -2.1 | 0 | 2.1 | 4.2 | 2.1 | -1.7 | 0.2 | 2.2 | 3.9 | 2.2 | -2.7 | 0 | 2.7 | 5.4 | 2.7 |
| -3 | -1.4 | 0 | 1.3 | 2.7 | 1.4 | -1.2 | 0.1 | 1.3 | 2.5 | 1.3 | -1.9 | 0 | 1.9 | 3.8 | 1.9 |
| 0 | -0.9 | 0 | 0.9 | 1.7 | 0.9 | -0.8 | 0 | 0.8 | 1.6 | 0.8 | -1.3 | 0 | 1.3 | 2.7 | 1.3 |
| Number of samples =3(480ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -1.2 | 0 | 1.2 | 2.4 | 1.2 | -0.9 | 0.1 | 1.1 | 2 | 1.1 | -1.5 | 0 | 1.5 | 3 | 1.5 |
| -3 | -0.8 | 0 | 0.8 | 1.5 | 0.8 | -0.6 | 0.1 | 0.6 | 1.2 | 0.6 | -1 | 0 | 1 | 2 | 1 |
| 0 | -0.5 | 0 | 0.5 | 1 | 0.5 | -0.4 | 0 | 0.4 | 0.8 | 0.4 | -0.7 | 0 | 0.7 | 1.4 | 0.7 |
| Number of samples =5(800ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 | -0.9 | 0 | 0.9 | 1.9 | 0.9 | -0.7 | 0.1 | 0.8 | 1.5 | 0.8 | -1.2 | 0 | 1.2 | 2.4 | 1.2 |
| -3 | -0.6 | 0 | 0.6 | 1.2 | 0.6 | -0.4 | 0.1 | 0.5 | 0.9 | 0.5 | -0.8 | 0 | 0.8 | 1.6 | 0.8 |
| 0 | -0.4 | 0 | 0.4 | 0.8 | 0.4 | -0.3 | 0 | 0.3 | 0.6 | 0.3 | -0.6 | 0 | 0.6 | 1.1 | 0.6 |

RSRQ accuracy comparison with legacy Rel.8 performance

Table 5 and 6 below summarizes the 90% absolute and relative RSRQ accuracy under SNR point -6dB and measurement BWs {6RB, 25RB} across all channel models.

Table 5: 90% absolute RSRQ accuracy with different configurations

| Measurement BWs | 6 RB |  |  | 25 RB |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | N=1 | N =3 | N =5 | N=1 | N =3 | N =5 |
| -6 | 3.6 | 2.8 | 2.4 | 2.3 | 1.5 | 1.3 |

Table 6: 90% relative RSRQ accuracy with different configurations

| Measurement BWs | 6 RB |  |  | 25 RB |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | N=1 | N =3 | N =5 | N=1 | N =3 | N =5 |
| -6 | 4.6 | 3.3 | 2.5 | 2.7 | 1.5 | 1.2 |

Following the general methodology for CRS-RSRP measurement as agreed in last meeting, DRS-RSRQ measurement accuracy can be defined with a scalable way based on number of DRS samples and measurement bandwidth.

Based on the simulation results and the observation, we can conclude that:

- Under SINR =-6dB with 6RB, Rel.8 requirements can be fulfilled with N =5 DRS samples with 1.1dB margin

- Under SINR =-6dB with 25RB, Rel.8 requirements can be fulfilled with N =3 DRS samples with 2.0dB margin.

Observation 1: For DRS RSRQ measurement, with N=5, 3 DRS samples for 6RB and 25 RB measurement BWs, Rel.8 requirements can be fulfilled with enough margin.

- Conclusion

In this contribution, we provide the simulation results and analysis for DRS-RSRQ measurement accuracy.

Our observation is:

Observation 1: For DRS RSRQ measurement, with N=5, 3 DRS samples for 6RB and 25 RB measurement BWs, Rel.8 requirements can be fulfilled with enough margin.

Based on the observation, it’s feasible to reuse the agreements for DRS-RSRP measurement to defining DRS-RSRQ accuracy.

Proposal1: The Rel-8 RSRQ measurement accuracy can be reused when Es/Iot≥-6dB for DRS based RSRQ measurement with below measurement time

| Measurement BWs | Number of DRS occasions (i×160ms) |
| --- | --- |
| ≥6PRB | 5 |
| ≥25PRB | 3 |

- Reference

- R1-143610, “LS on agreements on small cell discovery”

- R4-146704, “Meeting minutes for SCE RRM ad hoc”, Huawei

- R4-71AH-0103, “Simulation assumptions for DRS based RSRP accuracy for SCE”, Huawei