3GPP TSG-RAN WG4 Meeting #73	R4-147904

# San Francisco, USA, 17 – 21 November, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 2690 | rev |  | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ecat clarification for iRAT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Corporation, Nokia Networks, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12 |  |  |  |  |  |  | Date: |  | 2014-11-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Clarification of parallel reporting criteria in E-UTRAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increase of Ecat for inter-RAT measurement category for UEs supporting increased number of carriers to monitor. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for reporting criteria for inter-RAT measurement category will limit the network possibility to configure inter-RAT events for a UE supporting increased number of carriers to monitor. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged contents have been omitted>

## 8.2	Capabilities for Support of Event Triggering and Reporting Criteria

### 8.2.1	Introduction

This clause contains requirements on UE capabilities for support of event triggering and reporting criteria. As long as the measurement configuration does not exceed the requirements stated in clause 8.2.2, the UE shall meet the performance requirements defined in clause 9.

The UE can be requested to make measurements under different measurement identities defined in TS 36.331 [2]. Each measurement identity corresponds to either event based reporting, periodic reporting or no reporting. In case of event based reporting, each measurement identity is associated with an event. In case of periodic reporting, a measurement identity is associated with one periodic reporting criterion. In case of no reporting, a measurement identity is associated with one no reporting criterion.

The purpose of this clause is to set some limits on the number of different event, periodic and no reporting criteria the UE may be requested to track in parallel.

### 8.2.2	Requirements

In this clause a reporting criterion corresponds to either one event (in the case of event based reporting), or one periodic reporting criterion (in case of periodic reporting), or one no reporting criterion (in case of no reporting). For event based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in table 8.2.2-1.

The UE shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, and inter-RAT per supported RAT, the UE need not support more than the total number of reporting criteria as follows:

-	26 reporting criteria in total if the UE is not configured with any SCell carrier frequency,

-	35 reporting criteria in total if the UE is configured with one SCell carrier frequency and

-	44 reporting criteria in total if the UE is configured with two SCell carrier frequencies.

Table 8.2.2-1: Requirements for reporting criteria per measurement category

| Measurement category |  | Ecat |  | Note |  |
| --- | --- | --- | --- | --- | --- |
| Intra-frequency Note 1 |  | 9 |  | E-UTRA intra-frequency cells |  |
| Intra-frequency UE Rx-Tx time difference |  | 2 |  | Intra-frequency UE Rx-Tx time difference measurements reported to E-UTRAN via RRC and to positioning server via LPP. Applies for UE supporting both LPP and UE Rx-Tx time difference measurement. |  |
| Intra-frequency RSTD Note 2 |  | 1 |  | Intra-frequency RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 cell measurements for the intra-frequency |  |
| Intra-frequency RSRP and RSRQ measurements for E-CID |  | 1 |  | Intra-frequency RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [24]. One report capable of at least in total 9 intra-frequency RSRP and RSRQ measurements. Applicable to UE capable of reporting RSRP and RSRQ to E-SMLC via LPP. |  |
| Inter-frequency |  | 7 |  | E-UTRA inter-frequency cells |  |
| Inter-frequency RSTD Note 2 |  | 1 |  | Inter-frequency RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 cell measurements for at least one inter-frequency. Only applicable as specified in Section 8.1.2.6. |  |
| Inter-RAT (E-UTRAN FDD or TDD, GSM, cdma2000 1 x RTT and HRPD) |  | 5 |  | Only applicable for UE with this (inter-RAT) capability. This requirement (Ecat = 5) is per supported RAT. |  |
| Inter-RAT (UTRAN FDD, UTRAN TDD) |  | 5 or 11 |  | Only applicable for UE with this (inter-RAT) capability. This requirement (Ecat = 5 or 11) is per supported RAT. For UE which indicate support for Increased UE carrier monitoring UTRA Ecat = 11. |  |
| Note 1: When the UE is configured with SCell carrier frequency, Ecat for Intra-frequency is applied per serving frequency. Note 2: When the UE is configured with SCell carrier frequency, the UE shall be capable of supporting at least 2 reporting criteria for all RSTD measurements configured to be performed on PCell carrier frequency, SCell carrier frequency and inter-frequency carrier. This requirement applies when there is a single on-going LPP OTDOA location session. |  |  |  |  |  |