3GPP TSG-RAN4 Meeting #74 	R4-150468

# Athens, Greece, 9 – 13 February 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.853 | CR | 1 | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_B25_B41_B41-Core |  |  |  |  |  |  | Date: |  | 2015-02-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The WI to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 was completed in Rel-12. But the related text proposals were approved into the Rel-13 TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Put the corresponding UE materials into the Rel-12 TR to avoid unnecessary confusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains and unnecessary confusion may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, New clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

# 1	Scope

The present document is a technical report for 3DL Inter-band Carrier Aggregation under Rel-12 time frame. The purpose is to gather the relevant background information and studies in order to address 3DL Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address 3DL Inter-band Carrier Aggregation requirements for the Rel-12 band combinations in table 1-1.

Table 1-1: Release 12 3DL inter-band carrier aggregation combinations

| WI code | WI title |
| --- | --- |
| LTE_CA_B2_B2_B13 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 1 |
| LTE_CA_B2_B4_B13 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 4 and Band 13 |
| LTE_CA_B2_B5_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 5 and Band 30 |
| LTE_CA_B2_B12_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 30 |
| LTE_CA_B2_B29_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 29 and Band 30 |
| LTE_CA_B4_B4_B13 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 4, Band 4 and Band 13 |
| LTE_CA_B4_B5_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 5 and Band 30 |
| LTE_CA_B4_B12_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 12 and Band 30 |
| LTE_CA_B4_B29_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 29 and Band 30 |
| LTE_CA_B1_B3_B8 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 8 |
| LTE_CA_B1_B5_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 5 and Band 7 |
| LTE_CA_B2_B4_B5 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 4 and Band 5 |
| LTE_CA_B2_B4_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 4 and Band 12 |
| LTE_CA_B2_B5_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 5 and Band 12 |
| LTE_CA_B2_B12_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 12 |
| LTE_CA_B3_B7_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 7 and Band 20 |
| LTE_CA_B3_B8_B27 | LTE Advanced 3 band Carrier Aggregation (3DL/1UL) for Band 3, Band 8 and Band 27 |
| LTE_CA_B4_B5_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 5 and Band 12 |
| LTE_CA_B4_B12_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 12 and Band 12 |
| LTE_CA_B1_B3_B5 | LTE Advanced 3 Band Carrier Aggregation(3DL/1UL) for Band 1, Band 3 and Band 5 |
| LTE_CA_B1_B3_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 20 |
| LTE_CA_B1_B7_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 7 and Band 20 |
| LTE_CA_B7_B8_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 7, Band 8 and Band 20 |
| LTE_CA_B3_B7_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 7 and Band 7 |
| LTE_CA_B3_B3_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 3 and Band 7 |
| LTE_CA_B1_B42_B42 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 42 and Band 42 |
| LTE_CA_B3_B42_B42 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 42 and Band 42 |
| LTE_CA_B1_B3_B19 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 19 |
| LTE_CA_B2_B4_B29 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 4 and Band 29 |
| LTE_CA_B4_B4_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 4 and Band 7 |
| LTE_CA_B4_B7_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 7 and Band 12 |
| LTE_CA_B39_B39_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 39, Band 39 and Band 41 |
| LTE_CA_B39_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 39, Band 41 and Band 41 |
| LTE_CA_B2_B2_B29 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 2 and Band 29 |
| LTE_CA_B1_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 41, and Band 41 |
| LTE_CA_B2_B2_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 2, and Band 12 |
| LTE_CA_B25_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 25, Band 41, and Band 41 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New clause>

## 6.X	LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 25, Band 41, and Band 41

Table 6.X-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_25-41 | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.X-2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_25A-41C | 25 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | CA_41C | See table below |  |  |  |  |  |  |  |

Table 6.X-3: Supported E-UTRA bandwidths of CA_41C configuration for 3DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |
|  | 5, 10 | 20 | 40 | 1 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 5, 10, 15, 20 |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [4] when operating in single carrier mode

##### 6.X.3	ΔTIB,c and ΔRIB,c

For the UE which supports inter-band carrier aggregation in Band 25A and Band 41C with uplink assigned to Band 25 the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Table 6.X.3-1 and 6.X.3-2 respectively.

Table 6.X.3-1: ΔTIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_25A-41C | 25 | 0.5 |
|  | 41 | 0.5 |

Table 6.X.3-2: ΔRIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_25A-41C | 25 | 0 |
|  | 41 | 0 |

<End of change>