3GPP TSG-RAN WG4 (Radio) Meeting #74	R4-150472

Athens, Greece, 9 – 13 February 2015

Agenda Item:	7.23.3

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for Rel-13 4DL TR 36.8xx: LTE Advanced Carrier Aggregation of Band Combination (25 + 41 + 41 + 41)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 4 Band Carrier Aggregation (4DL/1UL) of Band 25, Band 41, Band 41 and Band 41 was approved in RAN#66 [1]. In consequence of this WI approval, RAN4 should define the relevant requirements of the maximum output power relaxation and the reference sensitivity relaxation to support inter-band CA of Band 25, Band 41, Band 41 and Band 41.

In this contribution, both ∆TIB and ∆RIB values are proposed and the relaxation requirements are based on the RAN4 approved UE RF requirements for the inter-band Carrier Aggregation (2DL/1UL) of Band 25 and Band 41 as well as the inter-band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41, and a text proposal is provided to include the values into the Rel-13 4DL CA TR [2].

# 2.	Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for 4DL Inter-band Carrier Aggregation under Rel-13 time frame. The purpose is to gather the relevant background information and studies in order to address 4DL Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address 4DL Inter-band Carrier Aggregation requirements for the Rel-13 band combinations in Table 1-1.

Table 1-1: Release 13 4DL inter-band carrier aggregation combinations

| WI code | WI title |
| --- | --- |
| LTE_CA_B25_B41_B41_B41 | LTE Advanced 4 Band Carrier Aggregation (4DL/1UL) of Band 25, Band 41, Band 41and Band 41 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

<New section>

## 6.X	LTE Advanced Carrier Aggregation of Band 25 and Band 41 and Band 41 and Band 41 (1 UL)

### 6.X.1	List of specific combination issues

#### 6.X.1.1	Channel bandwidths per operating band for CA

Table 6.X.1.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_25-41 | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

NOTE: 	The primary component carrier (PCC) will be in Band 25.

Table 6.X.1.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz |  |  |
| CA_25A-41D | 25 | Yes | Yes | Yes | Yes | 80 | 0 |
|  | CA_41D | Table 5.6A.1-1 of TS 36.101 [2] |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

##### 6.X.1.3	ΔTIB,c and ΔRIB,c

For the UE which supports CA_25A-41D the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Table 6.X.1.3-1 and 6.X.1.3-2 respectively.

Table 6.X.1.3-1: ΔTIB,c for 4DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_25A-41D | 25 | 0.5 |
|  | 41 | 0.5 |

Table 6.X.1.3-2: ΔRIB,c for 4DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_25A-41D | 25 | 0 |
|  | 41 | 0 |

<End of change>

# References

[1]	RP-142204, “New WID: LTE Advanced 4 Band Carrier Aggregation (4DL/1UL) of Band 25, Band 41, Band 41 and Band 41”, Sprint.

[2]	R4-73AH-0080, “Skeleton TR 36.8XX for 4 Band CA (4DL/1UL)”, Nokia Corporation.