3GPP TSG-RAN WG4 Meeting #74	R4-150900

San Francisco, CA, US, 17 – 21 Nov, 2014

Source:	Nokia Networks, Nokia Corporation

Title:	General discussion on Rel-13 high speed SI

Agenda item:	9.3

Document for:	Discussion

### 1	Introduction

A new Rel-13 SI is approved in last RAN plenary meeting to study LTE performance under high speed scenarios [1].  Based on the SID [1], there are two major objectives of the study:

- Comprehensively study enhancing of the requirements for UE RRM, UE demodulation and BS demodulation under existing high speed scenario.

- Identifying other new conditions to affect the system performance for E-UTRA under high speed environment and investigating BS demodulation, UE demodulation, and UE RRM performance under the identified new high speed scenarios.

The first objective includes study performance enhancements with UE velocity at least up to 350km/h (as the 1st priority) and with UE velocity above 350km/h.  The second objective is to study new high speed train scenarios for potential enhancements of performance under various deployments.

In this contribution, we provide an overview on the current RAN4 status of high speed specifications.  Based on the analysis, we also provide our general views on the progress of the SI.

### 2	BS performance for high speed scenarios

The LTE BS specification, up to Rel-12, supports high speed scenarios with the maximum speed as high as 350km/hr, as specified in 36.104.  The parameters for high speed train (HST) conditions are defined in Table B.3-1 of 36.104, as illustrated here (36.104, V12.6.0):

Table B.3-1: Parameters for high speed train conditions

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario 1 | Scenario 3 |
|  | 1000 m | 300 m |
|  | 50 m | 2 m |
|  | 350 km/h | 300 km/h |
|  | 1340 Hz | 1150 Hz |

NOTE1:	Parameters for HST conditions in table B.3-1 including  and Doppler shift trajectories presented on figures B.3-1 and B.3-2 were derived from Band1 and are applied for performance verification in all frequency bands.

As noted in the specification, the maximum speed of 350 km/h is used to calculate the maximum Doppler shift, which was derived from Band 1.  The uplink Doppler shift  takes account both DL Doppler spread and UL Doppler spread, because UE would use DL reference symbols to track.  The calculation of the UL Doppler shift  is based on

,

,

where  is UE velocity,  is the speed of wave propagation (light speed),  and  are UL and DL carrier frequency, respectively, and  and are UL and DL Doppler shifts, respectively.

The Doppler shift defined in 36.104 shall be used to specify BS performance for all frequency bands.  With a fixed Doppler shift, the supported maximum speed becomes band dependent, because the supported speed depends on operating frequency.

Based on the equation of uplink Doppler shift, the supported maximum speed of various LTE FDD and TDD bands are listed in Table 1.

Table 1    Supported maximum speed of LTE bands with fixed Doppler shift 1340Hz for UL, and with fixed Doppler shift 750Hz for DL

|  | UL |  | DL |  | Center f_c |  | UL fd=1340 Hz | DL fd=750Hz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE Band | UL_low | UL_high | DL_low | DL_high | UL fc | DL fc | v_m (km/hr) | v_m (km/hr) |
| 1 | 1920 | 1980 | 2110 | 2170 | 1950 | 2140 | 354 | 379 |
| 2 | 1850 | 1910 | 1930 | 1990 | 1880 | 1960 | 377 | 413 |
| 3 | 1710 | 1785 | 1805 | 1880 | 1747.5 | 1842.5 | 403 | 440 |
| 4 | 1710 | 1755 | 2110 | 2155 | 1732.5 | 2132.5 | 374 | 380 |
| 5 | 824 | 849 | 869 | 894 | 836.5 | 881.5 | 842 | 919 |
| 61 | 830 | 840 | 875 | 885 | 835 | 880 | 844 | 920 |
| 7 | 2500 | 2570 | 2620 | 2690 | 2535 | 2655 | 279 | 305 |
| 8 | 880 | 915 | 925 | 960 | 897.5 | 942.5 | 787 | 859 |
| 9 | 1750 | 1785 | 1845 | 1880 | 1767.4 | 1862.4 | 399 | 435 |
| 10 | 1710 | 1770 | 2110 | 2170 | 1740 | 2140 | 373 | 379 |
| 11 | 1428 | 1448 | 1476 | 1496 | 1437.9 | 1485.9 | 495 | 545 |
| 12 | 699 | 716 | 729 | 746 | 707.5 | 737.5 | 1002 | 1,098 |
| 13 | 777 | 787 | 746 | 756 | 782 | 751 | 944 | 1,079 |
| 14 | 788 | 798 | 758 | 768 | 793 | 763 | 930 | 1,062 |
| 15 |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |
| 17 | 704 | 716 | 734 | 746 | 710 | 740 | 998 | 1,095 |
| 18 | 815 | 830 | 860 | 875 | 822.5 | 867.5 | 856 | 934 |
| 19 | 830 | 845 | 875 | 890 | 837.5 | 882.5 | 841 | 918 |
| 20 | 832 | 862 | 791 | 821 | 847 | 806 | 875 | 1,005 |
| 21 | 1448 | 1463 | 1496 | 1511 | 1455.4 | 1503.4 | 489 | 539 |
| 22 | 3410 | 3490 | 3510 | 3590 | 3450 | 3550 | 207 | 228 |
| 23 | 2000 | 2020 | 2180 | 2200 | 2010 | 2190 | 345 | 370 |
| 24 | 1627 | 1661 | 1525 | 1559 | 1643.5 | 1542 | 454 | 525 |
| 25 | 1850 | 1915 | 1930 | 1995 | 1882.5 | 1962.5 | 376 | 413 |
| 26 | 814 | 849 | 859 | 894 | 831.5 | 876.5 | 847 | 924 |
| 27 | 807 | 824 | 852 | 869 | 815.5 | 860.5 | 863 | 941 |
| 28 | 703 | 748 | 758 | 803 | 725.5 | 780.5 | 961 | 1,038 |
| 29 | n/a |  | 717 | 728 |  | 722.5 |  | 1,121 |
| 30 | 2305 | 2315 | 2350 | 2360 | 2310 | 2355 | 310 | 344 |
| 31 | 453 | 458 | 463 | 468 | 455 | 465 | 1573 | 1,742 |
| 32 | n/a |  | 1452 | 1496 |  | 1474 |  | 550 |
| 33 | 1900 | 1920 | 1900 | 1920 | 1910 | 1910 | 379 | 424 |
| 34 | 2010 | 2025 | 2010 | 2025 | 2017.5 | 2017.5 | 359 | 401 |
| 35 | 1850 | 1910 | 1850 | 1910 | 1880 | 1880 | 385 | 431 |
| 36 | 1930 | 1990 | 1930 | 1990 | 1960 | 1960 | 369 | 413 |
| 37 | 1910 | 1930 | 1910 | 1930 | 1920 | 1920 | 377 | 422 |
| 38 | 2570 | 2620 | 2570 | 2620 | 2595 | 2595 | 279 | 312 |
| 39 | 1880 | 1920 | 1880 | 1920 | 1900 | 1900 | 381 | 426 |
| 40 | 2300 | 2400 | 2300 | 2400 | 2350 | 2350 | 308 | 345 |
| 41 | 2496 | 2690 | 2496 | 2690 | 2593 | 2593 | 279 | 312 |
| 42 | 3400 | 3600 | 3400 | 3600 | 3500 | 3500 | 207 | 231 |
| 43 | 3600 | 3800 | 3600 | 3800 | 3700 | 3700 | 196 | 219 |
| 44 | 703 | 803 | 703 | 803 | 753 | 753 | 961 | 1,076 |

For UL, the Doppler shift uses the fixed f_d = 1340Hz for all LTE FDD bands shown in the table.  The lower the carrier frequency, with fixed Doppler shift, the higher maximum speed the LTE band can support.  For band less than 2GHz, the current specified Doppler shift can support speed greater than 350km/hr.  The exceptions are high bands with carrier frequency greater than 2GHz, such as Band 7, Band 22, Band 30, Band 38, Band 40, Band 41, Band 42, and Band 43.

Based on the table, the motivation to support higher Doppler shift than Rel-12 Doppler shift is not very strong for carrier frequency less than 2GHz, because low LTE bands can support speed greater than 350km/hr.  For high frequency bands, the question to operators is whether there are intentions to extend these high bands to support high speed train scenarios.  Some of these high bands are likely to support small cell coverage, and specification of high speed train support for small cell coverage won’t be necessary.

Observation: Higher Doppler shift is supported for speed greater than 350km/hr in all bands with frequency less than 2GHz.

Observation: The exception bands to support 350km/hr are high bands with frequency greater than 2GHz, such as Band 7, Band 22, Band 30, Band 38, Band 40, Band 41, Band 42 and Band 43.

Proposal 1: Operators shall provide inputs on their intentions to support HST for specific high frequency bands.

### 3	UE performance for high speed scenarios

Similar exercise can be carried out to calculate the band-specific maximum speed with the fixed maximum Doppler shift.  For DL, based on 36.101, the maximum Doppler shift is 750Hz, as listed in Table B.3-1 of 36.101

Table B.3-1: High speed train scenario

| Parameter | Value |
| --- | --- |
|  | 300 m |
|  | 2 m |
|  | 300 km/h |
|  | 750 Hz |

With fixed Doppler shift of f_d = 750Hz, the supported DL maximum speed of all LTE FDD and TDD bands are also listed in Table 1.  Based on the table, all bands with frequency less than 2GHz can support speed as high as 350km/hr.  The exceptions are Band 7, Band 22, Band 30, Band 38, Band 40, Band 41, Band 42, and Band 43.

For UE RRM, there were already some discussions taking place under TEI12 and some agreements have been made. The accuracy requirements of absolute RSRP/RSRQ measurement under high Doppler have been captured in the latest version of 36.133 specification, where a relaxation for fading channel over AWGN channel is introduced with the fading channel modeling (EVA300) as part of the side conditions; the methodology to derive relative accuracy requirements from simulations remains under discussion. It is better to clarify if the remaining work will be carried out still under TEI12 or under this new SI.

It should be noted that many simulation results showed that EVA300 is more demanding for the measurement accuracy then EVA600, which implies that the higher speed may not necessarily lead to additional accuracy requirements. Also no consensus was reached during the early RAN4 discussions whether adaptations/enhancements for other RRM aspects are needed in existing high speed scenarios. Since higher speed (350km/h or beyond) has not been carefully addressed, we think further investigations are needed.

Proposal 2: RAN4 should clarify the work plan for the high Doppler RRM under TEI12 and the new SI.

Proposal 3: Investigations on all RRM aspects including the measurement accuracy are needed for higher speed (350km/h or beyond) in existing high speed scenarios before making decisions on RRM requirement enhancement.

The SID is asking two enhancements for UE requirements:

- UE velocity at least up to 350km/h

- UE velocity above 350km/h

Given that the gap between BS and UE is not quite significant, at current stage it is not clear at which speed and which specific band that UE should support high speed beyond 350km/h.

Proposal 4: Define which speed and which specific band that UE should support high speed.

### 4	New high speed scenarios

It is very interesting that multiple high speed scenarios are provided in the SID.  Although these listed scenarios in the SID are not exclusive, these scenarios shall be considered as basis for further study on new high speed scenarios.

#### 4.1	RRH deployments

When RRH are deployed along the high speed railway, these RRH shall meet the HST requirements.  New high speed train (HST) scenarios other than Scenario 1 and Scenario 3 could be defined to satisfy the new deployments, although the RRH deployments could be similar to HST Scenario 3 defined in 36.104.  We shall decide whether new HST scenario or the existing HST Scenario 3 should be used for RRH deployment in RAN4.

#### 4.2	Separate carriers for simultaneous access of high speed and high data rate

The SID discussed a potential application of utilizing multiple carriers, as

“Separate carriers are utilized for high speed scenario. One carrier with good coverage serves as PCell for mobility management. One carrier at high frequency may provide the good data transmission.”

Even with this design, both carriers shall meet HST requirements to provide reliable transmission links.  Companies should be encouraged to provide more details on the benefits of the scheme.  Besides, it might have specification impact of other 3GPP working groups.

#### 4.3	Public/dedicated network with repeaters installed in carriages

This is a viable solution to solve the high Doppler issue for UEs, which are associated with the repeater without Doppler effect.  The link between repeaters and eNB shall satisfy the performance requirements of HST scenarios.

#### 4.4	Repeaters inside tunnels

If the repeaters are deployed inside tunnels, new HST scenarios have to be defined for tunnel deployment.  With the new scenarios, performance should be studied and specified for new HST scenario of tunnel deployment.

Observation: new HST scenarios have to be defined for tunnel deployment

Proposal 5: Companies should be encouraged to provide more details on needed deployment scenario.

### 5	Conclusion

This contribution provides an overview on general aspects for the Rel-13 high speed SI.  The observations and proposals are summarized here.

Observation: Higher Doppler shift is supported for speed greater than 350km/hr in all bands with frequency less than 2GHz.

Observation: The exception bands to support 350km/hr are high bands with frequency greater than 2GHz, such as Band 7, Band 22, Band 30, Band 38, Band 40, Band 41, Band 42 and Band 43.

Proposal 1: Operators shall provide inputs on their intentions to support HST for specific high frequency bands.

Proposal 2: RAN4 should clarify the work plan for the high Doppler RRM under TEI12 and the new SI.

Proposal 3: Investigations on all RRM aspects including the measurement accuracy are needed for higher speed (350km/h or beyond) in existing high speed scenarios before making decisions on RRM requirement enhancement.

Proposal 4: Define which speed and which specific band that UE should support high speed.

Observation: new HST scenarios have to be defined for tunnel deployment

Proposal 5: Companies should be encouraged to provide more details on needed deployment scenario.

Given the very broad scope of the SI on high speed scenarios, virtually almost every aspect associated with high speed is under further investigation.  Therefore, we need to prioritize our study under the SI:

- The 1st priority should be specification of UE performance following the TEI discussion to support UE up to 350km/hr for major LTE bands.

- The 2nd priority is to decide whether >350km/hr requirements are needed and which bands are needed for both UE and BS performance, with inputs from operators.  Operators shall provide their intentions to support either wide area coverage or small cells for specific bands, especially the high bands.

- New high speed scenarios, as listed in the SID but not limited to, should be investigated after HST enhancements are concluded.

RAN4 shall consider the listed priority order for the SI planning and work.

# Reference

[1]	RP-142307	“New SI: Performance enhancements for high speed scenario”, NTT DOCOMO, Huawei, HiSilicon, Dec 2014

[2]	R4-147300	“Motivation of New SI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO, Huawei, HiSilicon, Nov 2014