3GPP TSG-RAN WG4 Meeting #74bis	R4-152473

Brazil, 20 - 24 April, 2015

Agenda item:	7.26

Source:	SAMSUNG

Title:	TP for dual uplink TR 36.860-13: Introducing additional channel bandwidth combination for LTE_CA_B3_B5

Document for:	Approval

1.  Introduction

In RAN#67, additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 5 was approved [1]. This contribution  just added it into dual uplink TR 36.860-13, to ensure the integrity of CA_B3_B5 in Rel-13, some content repeats with Rel-12.

2. Reference

[1] RP-150425 WID: Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 5

Text Proposal for TR36.860-13 (based on v1.0)

----- Start of TP -----

9	Class A4. Low-low, low-high or high-high band combination with intermodulation problem (low order IM)

[Unchanged sections omitted]

9.X	LTE-Advanced Carrier Aggregation of Band 3 and Band 5

9.X.1		Operating bands for CA

9.X.2	Channel bandwidths per operating band for CA_3A-5A

Table 9.X.2-1: Channel bandwidths per operating band for CA_3A-5A

| CA operating / channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Sets |
| CA_3A-5A | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 3 |  |  |  | Yes |  |  | 20 | 1 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 3 |
|  | 5 |  | Yes | Yes | Yes |  |  |  |  |

9.X.3	Harmonics and IMD study

2nd and 3rd order intermodulation frequencies are considered when studying UE-to-UE co-existence. In other cases, such as Rx desensitization and GNSS/ISM co-existence, also higher order IMD may be considered.

A harmonics and IMD co-existence table for dual uplink inter-band CA within band combination (3+5) is provided in Table 9.X.3-1.

Table 9.X.3-1: Band 3 and band 5 UL IMD products and harmonics

| UE UL Carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 - 1698 |  | 3420 - 3570 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 - 2547 |  | 5130 - 5355 |  |
| 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 - 961 |  | 2534 - 2634 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low - fy_high¦ | ¦2*fx_high - fy_low¦ | ¦2*fy_low - fx_high¦ | 2*fy_high - fx_low |
| IMD frequency limits (MHz) | 12 - 137 |  | 2571 - 2746 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3358 - 3483 |  | 4244 - 4419 |  |

Frequency limits for 5th order IMD is provided in table 9.X.3-2.

Table 9.X.3-2: Band 3 and band 5 5th order UL IMD product frequency ranges.

| ¦2*fx - 3*fy¦ | 3432 - 3707 |
| --- | --- |
| ¦2*fx + 3*fy¦ | 6778 - 7053 |
| ¦2*fy - 3*fx¦ | 873 - 1098 |
| ¦2*fy + 3*fx¦ | 5892 - 6117 |
| ¦fx - 4*fy¦ | 5991 - 6316 |
| ¦fx + 4*fy¦ | 7664 - 7989 |
| ¦fy – 4*fx¦ | 1511 - 1686 |
| ¦fy + 4*fx¦ | 5006 - 5181 |

Based on Table 9.X.3-1 2nd order IMD may fall into own Rx of band 5. In addition to this 5th order IMD may fall into band 5 Rx also, which is seen in Table 9.X.3-2 and may have to be considered in Rx desensitization studies. 2nd and 3rd order IMD and harmonics may also fall into Rx frequencies of bands 5, 6, 7, 8, 18, 19, 22, 26, 27, 38, 41 and 42.

In addition to E-UTRA bands also co-existence with GNSS and ISM frequencies was studied. GNSS and ISM frequencies which were included in the study are captured in Table 9.X.3-3.

Table 9.X.3-3: Frequency bands reserved for GNSS and ISM operation.

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | H3 |
|  | 2400 | - | 2494 | Yes | Asia | H3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | H3, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD5 |

9.X.4	ΔTIB,c and ΔRIB,c

----- End of TP -----