3GPP TSG RAN WG4 #75 			　　    　　　　	                 R4-152695

Fukuoka, Japan, 25 – 29 May, 2015

Source:		Anite Telecoms

Title:	Survey on User Interaction for Data Throughput

Agenda Item:	7.3.3

Document for:	Discussion

# Introduction

A work item of the Radiated requirements for the verification of multi-antenna reception performance of UEs was agreed in RAN #66 meeting [1]. The way forward from RAN4 #74bis is in the cite [2], and the harmonization campaign plan from the same meeting is presented in [3]. Part of the harmonization campaign plan is the discussion of the test time. Some discussion on this topic was already presented in RAN4 #74bis meeting [4] with the proposal to reduce the number of device orientations. A huge user based survey was presented in July 2014 for CTIA Certification Program Working Group meeting [5].

In this contribution it is presented the results of the survey done for the operators and the OEMs. The survey is done anonymous through the email. The operators are here weighted so that their number is greater and are spread geographically to Americas, Asia and Europe. Furthermore, the analysis of the results contains the weighting function for the grades given by the operators but the significance of this weighting is not high because the results between the operators and OEMs are quite much on the same direction. The results will be presented also in a CTIA meeting.

The survey sent tried to be simple and quick to answer. These aspects were selected to raise up the response rate. A detailed discussion on the coordinates and device orientations is found in [5]. In this survey the pictures were presented to make clear what is the intent of the described orientations. The orientations are the same as presented in the current version of the harmonization campaign plan [3].

The hand and head phantoms were not included in the survey.

# Survey Document

The document sent to x operators and y original equipment manufacturers (OEM) is attached here.

There are listed orientations as follows, the same as listed in [3]:

- Portrait, no tilt

- Portrait, 45° tilt down

- Portrait, 45° tilt up

- Face up

- Face down

- Landscape, no tilt

- Landscape, 45° tilt down

- Landscape, 45° tilt up

For the reader’s clarity the 45° tilt up orientation is here separately shown.

Figure 1. Orientation called Portrait, 45° tilt up. No hand phantom included.

The other very important position is the landscape mode of the same tilting degree. There it is not defined whether the device upper end is directed to left or right. In this survey they are equal.

The survey asked to grade the different orientations using the scale from 1 to 5 (1 being the least important and 5 being the most important). Only the smartphones and phablets were considered. Also the free feedback option was reserved.

# Survey Results

The responses are from 8 operators and 5 OEMs.

Table 1. Survey response data.

|  | Portrait | Portrait 45up | Portrait 45down | Face up | Face down | Landscape | Landscape 45up | Landscape 45down |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operator A | 1 | 4 | 2 | 1 | 2 | 1 | 3 | 2 |
| Operator B | 3 | 4 | 1 | 3 | 3 | 3 | 4 | 2 |
| Operator C | 4 | 5 | 1 | 4 | 1 | 4 | 5 | 3 |
| Operator D | 4 | 5 | 3 | 4 | 1 | 4 | 5 | 3 |
| Operator E | 3 | 5 | 1 | 5 | 1 | 3 | 5 | 1 |
| Operator F | 4 | 5 | 4 | 3 | 2 | 5 | 5 | 2 |
| Operator G | 2 | 4 | 3 | 4 | 2 | 1 | 5 | 3 |
| Operator H | 3 | 5 | 1 | 5 | 1 | 3 | 5 | 1 |
| Operator average | 3,0 | 4,6 | 2,0 | 3,6 | 1,6 | 3,0 | 4,6 | 2,1 |
| OEM A | 3 | 5 | 1 | 3 | 1 | 3 | 5 | 1 |
| OEM B | 4 | 5 | 1 | 3 | 3 | 4 | 5 | 1 |
| OEM C | 3 | 5 | 1 | 3 | 1 | 2 | 3 | 2 |
| OEM D | 3 | 5 | 1 | 3 | 1 | 3 | 5 | 1 |
| OEM E | 1 | 5 | 1 | 3 | 1 | 1 | 1 | 1 |
| OEM average | 2,8 | 5,0 | 1,0 | 3,0 | 1,4 | 2,6 | 3,8 | 1,2 |
| Total average | 2,9 | 4,8 | 1,6 | 3,4 | 1,5 | 2,8 | 4,3 | 1,8 |

The average could be also weighted on a party. If, for example, weighting the operators by the weight 60 and OEMs by 40 the total weighted average differs at maximum a tenth from the non–weighted one. In the table above the line OEM E is corrected so that the zero input values are translated to ones.

Using a cut–off of 3 or greater one can see from the table that the weighted average is higher than 3 for three cases. Please, remember that the grade 3 is the median of the grades to be selected so it is neither of highest importance nor of least importance.

- Portrait 45° tilt up (average 4,7)

- Landscape 45° tilt up (average 4,4)

- Face up (average 3,4)

When comparing the results to the user survey in [5] it is seen that the results coincide well. In the user survey the orientation Face up did not get hits but this might be well explained by the phrasing of the question: the user were asked to look at a video or to play a game.

# Future Considerations

The MIMO OTA testing options cover currently only part of the user scenarios. This was seen also in the free text feedback. There were listed some cases that are neither listed in this survey nor are in the test plans. But they might be in the future releases. The feedback proposals are shortly summarized here:

- Phantoms (right hand, left hand, two hands, head) should be used

- Tilt angle should be 30°

- Pocket and headset used case should be added

# Conclusion

Device orientations for the testing purposes were asked. There are three orientations when using the average calculation got the grade higher than 3 and these are

- Tilted positions for portrait and landscape modes

- Face up mode.

# References

- RP-142221, “New WI proposal: Radiated requirements for the verification of multi-antenna reception performance of UEs,” Intel Corporation, 3GPP RAN #66, December 2014

- R4-152539 “MIMO OTA Way Forward”, Intel Corporation, CTTC, Bluetest, Keysight, NTT Docomo, Spirent, SGS, 3GPP RAN4 #74bis, April 2015

- R4-151976, “MIMO OTA Harmonization testing campaign”, Bluetest, CTTC, Keysight, 3GPP RAN4 #74bis, April, 2015

- R4-151400, “Evaluation of MIMO OTA MU caused by UE positioning in anechoic chamber”, NTT Docomo, 3GPP RAN4 #74bis, April, 2015

- MOSG140803, “Proposal of MPAC coordinate system and scaling-down representative test cases based on a holding habit survey”, ABP, Zealer, HWA-Tech, TMC, Sony Mobile Communications, CTIA CPWG, 31st of July, 2014