3GPP TSG-RAN WG4 (Radio) Meeting #76	R4-153990

Beijing, China, 24 – 28 August 2015

Agenda Item:	7.20.1

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for TR 36.833-8-41: Operating bands and channel bandwidths for LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 41 for 4DL

Document for:	Approval

# 1.	Introduction

The LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 41 for 4 DL work item was approved at TSG RAN #66 [1]. This contribution provides a text proposal to include the operating bands and channel bandwidths for this work item into the TR 36.833-8-41 [2] of this work item.

# 2.	Text proposal

<Start of change>

## 5.1	Operating bands and channel bandwidths

### 5.1.1	Operating bands for CA

Table 5.1.1-1 below shows the CA operating band needed for this work item.

Table 5.1.1-1: Intra-band non-contiguous CA operating bands (with two sub-blocks)

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_41-41 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 5.1.2	Channel bandwidths for CA

Table 5.1.2-1 below shows the CA configurations and bandwidth combination sets needed for this work item.

Table 5.1.2-1: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C-41C | See Table 5.6A.1-1 |  | See Table 5.6A.1-1 |  | 80 | 0 |
| CA_41A-41D | 5, 10, 15, 20 | See Table 5.6A.1-1 |  |  | 80 | 0 |
| CA_41D-41A | See Table 5.6A.1-1 |  |  | 5, 10, 15, 20 | 80 | 0 |

<End of change>

# References

[1]	RP-142022, “New WID: LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 41 for 4DL”, Sprint.

[2]	R4-151292, “TR 36.833-8-41 V0.1.0”, Alcatel-Lucent, Sprint.