3GPP TSG-RAN WG4 Meeting #77		R4-158266

Anaheim, USA, 16 – 20 November, 2015

Agenda item:	7.3.3

Source:	Rohde&Schwarz

Title:	Updated Harmonization Analyses

Document for:	Discussion

This contribution is taking some of the proposals for handling of results that do not reach the target throughput [1] and applying them to the harmonization analyses for Bands 13 and 7, based on the informative Excel spreadsheet included in the RAN4#76bis way forward [2].

The focus of these analyses is the residual error calculated for Option G with the following harmonization parameters [2]:

| Opt.G harmonization parameters |  |
| --- | --- |
| Parameter | Value |
| Max TPT per measured curve assumption | Maximum expected per MCS |
| Throughput outage value | Residual error evaluated at 70% and 95% |
| Outage point search method | First intersect search high TPT to low |
| Averaging method across outage points | Linear across mW |
| DUT positions in MPAC | Avg across all x pos. (single test case) |
| RC/RC+CE FoM | avg. across outage points (see labels) |
| Channel Models | UMa, UMi, NIST, LCSD, HCLD |

The number of DUT positions, x, is reduced from 8 to 3 according to the priority of DUT positions published in the operator and OEM survey on user interaction [3]. While the attached spreadsheets are taking the residual errors for RC vs MPAC, RC&CE vs MPAC, and RTS vs MPAC into account, the graphical results in this document are focused only on RC&CE vs MPAC and RTS vs MPAC due to the very large residual errors of RC vs MPAC (only one offset can be used per band; the same offset needs to be applied for each channel model).

The following proposals for handling of results that do not reach 95% target throughput are considered here and the attached harmonization analysis spreadsheets:

- 70% Only: perform the analysis with the 70% throughput outage value only

- Substitution with -80: For throughput curves that do not reach the 95% outage level, an upper value for the DL power of -80dBm/15kHz is used as substitute

- Ignore outliers: Ignore curves that don’t’ reach the target value but note how many do this

- Top 10/100: For AC(RC&CE) methodologies, only take the best 10 (120) curves into account for the linear average

- Ignore outliers + 0.2dB/curve penalty: Ignore curves that don’t’ reach the target value, note how many outliers exist, and penalize the linear average by 0.2dB per curve that does not achieve the target value

The residual errors of RTS with respect to MPAC and RC&CE with respect to MPAC are shown in Figure 1 for Band 13 and in Figure 2 for Band 7.

Figure 1: Residual errors for Band 13

Figure 2: Residual errors for Band 7

References

# [1] 3GPP R4-157290, Keysight Technologies, “Proposals for handling of results that do not reach the target throughput”

[2] 3GPP R4-155596, Intel Corporation, Spirent, Bluetest, CTTC, CATR, Keysight, R&S, “Way Forward on MIMO OTA”

[3] R4-152695, Anite Telecoms, “Survey on User Interaction for Data Throughput”