3GPP TSG-RAN WG4 (Radio) Meeting #77	R4-158314

Anaheim, CA, US, 16 – 20 November 2015

Agenda Item:	7.42.2.4

Source: 	Alcatel-Lucent, AT&T

Title: 	TP for TR 36.854-13: BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands (4 + 4 + 5/12/29 + 30)

Document for:	Approval

# 1.	Introduction

3 WI proposals to support the LTE Advanced (LTE-A) 4DL/1UL Carrier Aggregation (CA) with Bands (4 + 4 + 5/12/29 + 30) were approved in RAN#68 [1-3]. The required BS studies of harmonics and IMD products for the 10 WI proposals were provided in RAN4#76 [4-6].

In this contribution, we provide a text proposal to complete the BS studies in the 4DL/1UL TR 36.854-13 [7].

# 2.	Text Proposal

<New clause>

## 6.X	LTE Advanced Carrier Aggregation of Band 4 and Band 4 and Band 5 and Band 30 (1 UL)

### 6.X.1	List of specific combination issues

#### 6.X.1.1	Channel bandwidths per operating band for CA

Table 6.X.1.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_4-5-30 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |

Table 6.X.1.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_4A_4A_5A_30A | See CA_4A-4A Bandwidth Combination Set 0 in TR 36.101 Table 5.6A.1-3 |  |  |  |  |  |  | 60 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  |  | Yes | Yes |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

#### 6.X.1.2	Co-existence studies for CA_4-4-5-30

The 3rd order IMD products caused in the BS by transmitting CA_4A-4A-5A-30A follow from those derived for the constituent 2DL and 3DL CA fall-back modes, which have been analysed and captured in [3-7].

## 6.Y	LTE Advanced Carrier Aggregation of Band 4 and Band 4 and Band 12 and Band 30 (1 UL)

### 6.Y.1	List of specific combination issues

#### 6.Y.1.1	Channel bandwidths per operating band for CA

Table 6.Y.1.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_4-12-30 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |

Table 6.Y.1.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_4A_4A_12A_30A | 4 | See CA_4A-4A Bandwidth Combination Set 0 in TR 36.101 Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  | 12 |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  |  | Yes | Yes |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

#### 6.Y.1.2	Co-existence studies for CA_4-4-12-30

The 3rd order IMD products caused in the BS by transmitting CA_4A-4A-12A-30A follow from those derived for the constituent 2DL and 3DL CA fall-back modes, which have been analysed and captured in [3-7].

## 6.Z	LTE Advanced Carrier Aggregation of Band 4 and Band 4 and Band 29 and Band 30 (1 UL)

### 6.Z.1	List of specific combination issues

#### 6.Z.1.1	Channel bandwidths per operating band for CA

Table 6.Z.1.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_4-29-30 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 29 | N/A |  |  | 717 MHz | – | 728 MHz |  |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |

Table 6.Z.1.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_4A_4A_29A_30A | See CA_4A-4A Bandwidth Combination Set 0 in TR 36.101 Table 5.6A.1-3 |  |  |  |  |  |  | 60 | 0 |
|  | 29 |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  |  | Yes | Yes |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

#### 6.Z.1.2	Co-existence studies for CA_4-4-29-30

The 3rd order IMD products caused in the BS by transmitting CA_4A-4A-29A-30A follow from those derived for the constituent 2DL and 3DL CA fall-back modes, which have been analysed and captured in [3-7].

<End of change>

# References

[1]	RP-151015, “New WI proposal: LTE Advanced 4 Band Carrier Aggregation (4DL) of Band 4, Band 4, Band 5 and Band 30”, AT&T.

[2]	RP-151016, “New WI proposal: LTE Advanced 4 Band Carrier Aggregation (4DL) of Band 4, Band 4, Band 12 and Band 30”, AT&T.

[3]	RP-151017, “New WI proposal: LTE Advanced 4 Band Carrier Aggregation (4DL) of Band 4, Band 4, Band 29 and Band 30”, AT&T.

[4]	R4-153997, “Required BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands (4 + 4 + 5/12/29 + 30)”, Alcatel-Lucent, AT&T.

[5]	R4-153998, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (4 + 4 + 29)”, Alcatel-Lucent, AT&T.

[6]	R4-155374, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (4 + 4 + 30)”, Alcatel-Lucent, AT&T.

[7]	R4-157452, “TR 36.854-13 v0.5.0”, Nokia Networks.