3GPP TSG RAN4 Meeting #80bis	R4-168352

Ljubljana, Slovenia, 10 – 14 Oct, 2016

Title: 	Intermodulation analysis for UL CA including B46 UL

Source: 	Huawei, HiSilicon

Agenda item:	8.18.2.4

Document for:	Discussion

# 1	Introduction

This contribution provides some example UL CA MSD analysis including B46 UL

# 2	Discussion

All potential 2UL/2DL CA band combinations were listed in [1]. In this paper we analyse a couple of different combinations with common and separate antennas. We note that even some of the specifications assume single or separate TX antennas none of the implementations are prohibited by the standard as long as the requirements are met.

CA_3A-46A has IMD3 (B46-2*B3) and IMD5 relation (4*B3-B46).

CA_7A-46A has IMD2 (B46-B7) and IMD5 relation (2*B46-3*B7).

We used the following architectures to analyse MSD. Please note that for CA_3A-46A only the separate TX-antennas architecture was used in this analysis.

Figure 1 Separate TX-antennas -architecture

Figure 2 Common TX-antenna architecture

The component parameters are as follows.

| Component | IP2 (dBm) | IP3 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 53 |
| Diplexer | 115 | 86 | 53 |
| Duplexer/Filter | 113 | 74 | 53 |
| PA forward mixing | 27 | 30 | 27 |
| PA reverse mixing | 38 | 28 | 32 |
| LNA | 5 | -5 | -10 |

Generic assumptions are:

PA gain							28

Antenna ISO						10/15

PCB ISO PAout-PAin			65

Diplexer ISO						20

B3/B7 BW						5

Component related assumptions are:

B46 filter rejection at B3 RX 						40

B46 filter rejection at B3 TX 						40

B3 Duplexer TX isolation							50

B3 Duplexer RX isolation							46

B3 Duplexer RX rejection at B46					30

B3 Duplexer TX rejection at B3 RX band 			43

B3 Duplexer TX rejection at B46 RX band 		17

B46 filter rejection at B7 RX 						35

B46 filter rejection at B7 TX 						35

Duplexer TX isolation								53

Duplexer RX isolation								48

Duplexer RX rejection at B46						40

B7 Duplexer TX rejection at B7 RX band 			45

B7 Duplexer TX rejection at B46 RX band 		25

These values are based on actual component. We can see there is a lot of room for further optimization, likely both in PA and filter characteristics. For instance B3 duplexer TX rejection at B46 is only 17dB because in the past the has not been any explicit requirement to push down the rejection.

IMD’s are as follows. Please not e that the respective MSD would be a tad smaller due to correction factors that are not included.

| CA configuration | Antenna ISO | IMD order | Separate TX-antennas | Common TX-antennas |
| --- | --- | --- | --- | --- |
| CA_3A-46A | 10 | IMD3 | 42.45 | N/A |
|  | 15 | IMD3 | 32.16 | N/A |
|  | 10 | IMD5 | 31.82 | N/A |
|  | 15 | IMD5 | 21.19 | N/A |
| CA_7A-46A | 10 | IMD2 | 37.97 | 34.23 |
|  | 15 | IMD2 | 30.73 | 27.02 |
|  | 10 | IMD5 | 0 | 0 |
|  | 15 | IMD5 | 0 | 0 |

Values in many cases are large. Please note that the difference in IMD5’s between these two combinations mostly comes from the different frequency relation; B3+B46 is 4*B3-B46 while B7+B46 is 2*B46-3*B7. We noticed that in many cases the dominating factor is LNA non-linearity, especially if the PA mixing characteristics are improved from the ones used in this analysis. We will more analysis for the next meeting but at the moment it seems inevitable that MSD will need to be defined for licensed band in case of UL CA including B46 UL.

# 3	Conclusion

Intermodulation for CA_3A-46A and CA_7A-46A was briefly analysed with initial parameters. While there is much room for improvement, the current results are so large that MSD for licensed band seems inevitable in case of UL CA including B46 UL.

Preliminary analysis shows the following IMD results, excluding CF.

| CA configuration | Antenna ISO | IMD order | Separate TX-antennas | Common TX-antennas |
| --- | --- | --- | --- | --- |
| CA_3A-46A | 10 | IMD3 | 42.45 | N/A |
|  | 15 | IMD3 | 32.16 | N/A |
|  | 10 | IMD5 | 31.82 | N/A |
|  | 15 | IMD5 | 21.19 | N/A |
| CA_7A-46A | 10 | IMD2 | 37.97 | 34.23 |
|  | 15 | IMD2 | 30.73 | 27.02 |
|  | 10 | IMD5 | 0 | 0 |
|  | 15 | IMD5 | 0 | 0 |

# 4	References

[1] R4-166139, “Intermodulation considerations for UL CA including B46 UL”, Huawei, Hisilicon, RAN4#80