3GPP TSG-RAN WG4 Meeting #78	                                                                         R4-160080

St. Julian’s, Malta, 15 – 19 February, 2016

Source: 	Samsung

Title: 	Simulation Results for 256QAM PDSCH Demodulation with 4RX

Agenda Item:	6.8.2.1

Document for:	Discussion

Introduction

In the RAN4 email discussion before the RAN4 #77 meeting, the necessity of introducing 256QAM demodulation for two layer transmission has been discussed, which resulted in the understanding that the original Test 3 (for TM4) can be further extended for 256QAM demodulation with the proposed FRC [1].  During RAN4#77, the simulation results have been provided by different companies for alignment purpose [2].

In this contribution, we will provide our simulation results for 256QAM PDSCH demodulation tests for alignment based on the agreed simulation assumption.

256QAM PDSCH demodulation Tests for 2-Layer

Based on the detailed simulation assumption provided on RAN4 email reflector, we would like to provide FDD simulation results for alignment purpose. Specifically, the following FRC is utilized in our simulation.

| Parameter | Unit |
| --- | --- |
| Reference channel |  | R.xx FDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks (Note 4) |  | 50 |
| Allocated subframes per Radio Frame |  | 9 |
| Modulation |  | 256QAM |
| Target Coding Rate |  | 0.62 |
| Information Bit Payload (Note 4) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 31704 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 31704 |
| Number of Code Blocks (Notes 3 and 4) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  | 3 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 |  | 3 |
| Binary Channel Bits (Note 4) |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 51200 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 48128 |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 28.534 |
| UE Category |  | ≥ 2 |

The simulation result is illustrated in the following figure.

Figure 1.  PDSCH Demodulation Performance for TM3 with 3-Layer

Based on our simulation results, we can get the reference SNR value corresponding to 70 percentage of maximum throughput, which is provided in the following table:

Table 1. Reference Demodulation SNR Point for Test Cases

| Test cases | 256QAM |
| --- | --- |
| Reference SNR Value | 16.4dB |

Conclusion

In this contribution, we provide our simulation results for256QAM PDSCH demodulation, based on the detailed simulation assumption provided on RAN4 email reflector. For the reference SNR values corresponding to 70 percentage of maximum throughput for this test case, we summarized in the following table:

Table 1. Reference Demodulation SNR Point for Test Cases

| Test cases | 256QAM |
| --- | --- |
| Reference SNR Value | 16.4dB |

References

RAN4 email discussion: [4Rx demod] 256QAM test with 4Rx.

R4-158119, Summary of simulation results for PDSCH demodulation test for 4Rx.