3GPP TSG-RAN WG4 Meeting #78	R4-160270

St. Julian’s, Malta, 15 – 19 February, 2016

Agenda item:			6.12.1.2

Source:	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

Title:	Cat-M PRACH Simulation Assumptions

Document for:		Approval

# Introduction

In [1], we discussed the PRACH performance requirements for supporting Cat-M UEs. In this paper, we provide the link level simulation assumptions for defining Cat-M PRACH performance requirements.

# Simulation Assumptions

Tables 1 and 2 provide the simulation assumptions for Cat-M PRACH detection requirements based on the discussion in [1], the current PRACH detection requirements [2,3], and simulation assumptions presented in [4]. The false alarm probability shall be less than or equal to 0.1%, and the probability of detection shall be equal to or exceed 99%.

As described in TS 36.141 [3], the timing offset base value is set to 50% of Ncs. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.9us. Then the loop is being reset and the timing offset is set again to 50% of Ncs. The timing offset scheme is presented in Figure 1.

Table 3 provides a template for the Cat-M PRACH missed detection requirements with different repetition levels.

| Parameters | Values | Comments |
| --- | --- | --- |
| PRACH format | Format 0, Format 1, Format 2, Format 3 |  |
| Propagation conditions | AWGN ETU30 | Due to the low mobility of Cat-M UEs, using ETU30 as in RLM requirements |
| Number of TX antennas | 1 |  |
| Number of RX antennas | 2,4,8 |  |
| Antenna correlation | Low |  |
| Noise model | AWGN |  |
| Timing offset |  | See Figure 1 |
| Repetition level | Format 0 and Format 1: {8,32,[128]} Format 2 and Format 3: {4,16,[64]} | May not need to simulate all levels of {1,2,4, 8,16,32,64,128} |
| Number of signatures reserved for Cat-M UEs | 16 | The size of preamble pool has impact on detection performance |
| Frequency offset (Hz) | 0 for AWGN, 270 for ETU30 |  |
| Frequency hopping | OFF |  |

Table 1.	Cat-M PRACH Simulation Assumptions

Table 2. Test preambles for Normal Mode (From Table A..6-1 in TS 36.104 [2])

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |
| 4 | 10 | 0 | 0 |

Figure 1. Timing offset scheme (copied from [4])

Table 3. Cat-M PRACH missed detection requirements (1Tx)

| Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 2 | AWGN | 0 | 4 | - | - | TBD | TBD |
|  |  |  | 8 | TBD | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  |  |  | 64 | - | - | TBD | TBD |
|  |  |  | 128 | TBD | TBD | - | - |
|  | ETU 30 Low | 270 Hz | 4 | - | - | TBD | TBD |
|  |  |  | 8 | TBD | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  |  |  | 64 | - | - | TBD | TBD |
|  |  |  | 128 | TBD | TBD | - | - |
| 4 | AWGN | 0 | 4 | - | - | TBD | TBD |
|  |  |  | 8 | TBD | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  |  |  | 64 | - | - | TBD | TBD |
|  |  |  | 128 | TBD | TBD | - | - |
|  | ETU 30 Low | 270 Hz | 4 | - | - | TBD | TBD |
|  |  |  | 8 | TBD | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  |  |  | 64 | - | - | TBD | TBD |
|  |  |  | 128 | TBD | TBD | - | - |
| 8 | AWGN | 0 | 4 | - | - | TBD | TBD |
|  |  |  | 8 | TBD | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  |  |  | 64 | - | - | TBD | TBD |
|  |  |  | 128 | TBD | TBD | - | - |
|  | ETU 30 Low | 270 Hz | 4 | - | - | TBD | TBD |
|  |  |  | 8 | TBD | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  |  |  | 64 | - | - | TBD | TBD |
|  |  |  | 128 | TBD | TBD | - | - |

# Summary

In this contribution, we provided the link level simulation assumptions for Cat-M PRACH performance requirements.

# References

- R4-16xxxx, Cat-M PRACH Simulation Assumptions, Nokia Networks, Alcatel-Lucent

- 3GPP TS 36.104, E-UTRA: Base Station (BS) radio transmission and reception.

- 3GPP TS 36.141, E-UTRA: Base Station (BS) conformance testing.

- R4- 131776, Simulation assumptions for UL 8 antenna receiver, China Telecom.