3GPP TSG-RAN WG4 Meeting #78 	R4-160375

# Malta, Malta, 15 – 20, Feb. 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3422 | rev | - | Current version: | 13.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FRC for non-TM10 with CRS assistance information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CRSIM-Perf |  |  |  |  |  |  | Date: |  | 2016-2-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The FRC for non-TM10 cases are not captured when CRS assistance information is configured |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add FRC for non-TM10 cases when CRS assistance informaiton is configured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no FRC for non-TM10 cases when CRS assistance information is configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

### A.3.1.1	Overview of DL reference measurement channels

In Table A.3.1.1-1 are listed the DL reference measurement channels specified in annexes A.3.2 to A.3.10  of this release of TS 36.101. This table is informative and serves only to a better overview. The reference for the concrete reference measurement channels and corresponding implementation’s parameters as to be used for requirements are annexes A.3.2 to A.3.10  as appropriate.

Table A.3.1.1-1: Overview of DL reference measurement channels

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD, Receiver requirements |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-1 |  | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| FDD | Table A.3.2-1 |  | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 1.4 | QPSK | 1/3 | 6 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 3 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 5 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 10 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 15 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD / HD-FDD | Table A.3.2-1a |  | 20 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD, Receiver requirements |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-2 |  | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| TDD | Table A.3.2-2a |  | 1.4 | QPSK | 1/3 | 6 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 3 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 5 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 10 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 15 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 20 | QPSK | 1/3 | 14 |  | 0 |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 3-5 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3 |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD | Table A.3.2-3 |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| FDD | Table A.3.2-3 |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| FDD | Table A.3.2-3 |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| FDD | Table A.3.2-3 |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| FDD | Table A.3.2-3 |  | 20 | 64QAM | 3/4 | 100 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 1 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3a |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD | Table A.3.2-3a |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| FDD | Table A.3.2-3a |  | 5 | 64QAM | 3/4 | 18 |  | - |  |
| FDD | Table A.3.2-3a |  | 10 | 64QAM | 3/4 | 17 |  | - |  |
| FDD | Table A.3.2-3a |  | 15 | 64QAM | 3/4 | 17 |  | - |  |
| FDD | Table A.3.2-3a |  | 20 | 64QAM | 3/4 | 17 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 2 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3b |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| FDD | Table A.3.2-3b |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| FDD | Table A.3.2-3b |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| FDD | Table A.3.2-3b |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| FDD | Table A.3.2-3b |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| FDD | Table A.3.2-3b |  | 20 | 64QAM | 3/4 | 83 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE DL Categories 0 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-3c |  | 1.4 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 3 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 5 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 10 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 15 | 64QAM | 3/4 | 2 |  | - |  |
| FDD | Table A.3.2-3c |  | 20 | 64QAM | 3/4 | 2 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 3-5 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4 |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| TDD | Table A.3.2-4 |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| TDD | Table A.3.2-4 |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| TDD | Table A.3.2-4 |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| TDD | Table A.3.2-4 |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| TDD | Table A.3.2-4 |  | 20 | 64QAM | 3/4 | 100 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 1 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4a |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| TDD | Table A.3.2-4a |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| TDD | Table A.3.2-4a |  | 5 | 64QAM | 3/4 | 18 |  | - |  |
| TDD | Table A.3.2-4a |  | 10 | 64QAM | 3/4 | 17 |  | - |  |
| TDD | Table A.3.2-4a |  | 15 | 64QAM | 3/4 | 17 |  | - |  |
| TDD | Table A.3.2-4a |  | 20 | 64QAM | 3/4 | 17 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 2 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4b |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| TDD | Table A.3.2-4b |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| TDD | Table A.3.2-4b |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| TDD | Table A.3.2-4b |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| TDD | Table A.3.2-4b |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| TDD | Table A.3.2-4b |  | 20 | 64QAM | 3/4 | 83 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE DL Categories 0 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4c |  | 1.4 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 3 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 5 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 10 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 15 | 64QAM | 3/4 | 2 |  | - |  |
| TDD | Table A.3.2-4c |  | 20 | 64QAM | 3/4 | 2 |  | - |  |
| FDD, Receiver requirements, Maximum input level for UE Categories 11/12/13/14/15 and UE DL categories 13,14 |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.2-5 |  | 1.4 | 256QAM | 4/5 | 6 |  | - |  |
| FDD | Table A.3.2-5 |  | 3 | 256QAM | 4/5 | 15 |  | - |  |
| FDD | Table A.3.2-5 |  | 5 | 256QAM | 4/5 | 25 |  | - |  |
| FDD | Table A.3.2-5 |  | 10 | 256QAM | 4/5 | 50 |  | - |  |
| FDD | Table A.3.2-5 |  | 15 | 256QAM | 4/5 | 75 |  | - |  |
| FDD | Table A.3.2-5 |  | 20 | 256QAM | 4/5 | 100 |  | - |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 11/12/13/14/15 and UE DL categories 13,14 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-6 |  | 1.4 | 256QAM | 4/5 | 6 |  | - |  |
| TDD | Table A.3.2-6 |  | 3 | 256QAM | 4/5 | 15 |  | - |  |
| TDD | Table A.3.2-6 |  | 5 | 256QAM | 4/5 | 25 |  | - |  |
| TDD | Table A.3.2-6 |  | 10 | 256QAM | 4/5 | 50 |  | - |  |
| TDD | Table A.3.2-6 |  | 15 | 256QAM | 4/5 | 75 |  | - |  |
| TDD | Table A.3.2-6 |  | 20 | 256QAM | 4/5 | 100 |  | - |  |
| FDD, PDSCH Performance, Single-antenna transmission (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-1 | R.4 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.42 FDD | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.42-1 FDD | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.42-2 FDD | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.42-3 FDD | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-1 | R.2 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-2 | R.3-1 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-2 | R.3 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-3 | R.5 FDD | 3 | 64QAM | 3/4 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3 | R.6 FDD | 5 | 64QAM | 3/4 | 25 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-3 | R.7 FDD | 10 | 64QAM | 3/4 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-3 | R.8 FDD | 15 | 64QAM | 3/4 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-3 | R.9 FDD | 20 | 64QAM | 3/4 | 100 |  | ≥ 3 |  |
| FDD | Table A.3.3.1-3a | R.6-1 FDD | 5 | 64QAM | 3/4 | 18 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3a | R.7-1 FDD | 10 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3a | R.8-1 FDD | 15 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3a | R.9-1 FDD | 20 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-3a | R.9-2 FDD | 20 | 64QAM | 3/4 | 83 |  | ≥ 2 |  |
| FDD | Table A.3.3.1-6 | R.41 FDD | 10 | QPSK | 1/10 | 50 |  | ≥ 1 |  |
| FDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (Channel edge) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-4 | R.0 FDD | 3 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| FDD | Table A.3.3.1-4 | R.1 FDD | 10 / 20 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| FDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (MBSFN Configuration) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-5 | R.29 FDD | 10 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| FDD, PDSCH Performance: Carrier aggregation with power imbalance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.1-7 | R.49 FDD | 20 | 64QAM | 0.84-0.87 | 100 |  | ≥ 5 |  |
| FDD | Table A.3.3.1-7 | R.49-1 FDD | 10 | 64QAM | 0.84-0.87 | 50 |  | ≥2 |  |
| FDD | Table A.3.3.1-7 | R.49-2 FDD | 5 | 64QAM | 0.84-0.86 | 25 |  | ≥2 |  |
| FDD, PDSCH Performance, Multi-antenna transmission (CRS), Two antenna ports |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.2.1-1 | R.10 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.10-2 FDD | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.11-2 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11-3 FDD | 10 | 16QAM | 1/2 | 40 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11-4 FDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.30 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.30-1 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35-1 FDD | 20 | 64QAM | 0.39 | 100 |  | 4 |  |
| FDD | Table A.3.3.2.1-1 | R.35-2 FDD | 15 | 64QAM | 0.39 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35-3 FDD | 10 | 64QAM | 0.39 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.35-4 FDD | 10 | 64QAM | 0.47 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.46 FDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.47 FDD | 10 | 16QAM |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-5 FDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-6 FDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-7 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.11-8 FDD | 10 | QPSK | 3/5 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.11-9 FDD | 10 | QPSK | 0.58 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-10 FDD | 10 | QPSK | 0.67 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.10-3 FDD | 10 | 16QAM | 0.58 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.65 FDD | 10 | 256QAM | 0.6 | 50 |  | 11-15 |  |
| FDD | Table A.3.3.2.1-3 | R. 62 FDD | 10 | 16QAM | 1/2 | 3 |  | 0 |  |
| FDD | Table A.3.3.2.1-3 | R.63 FDD | 10 | 64QAM | 1/2 | 1 |  | 0 |  |
| FDD, PDSCH Performance, Multi-antenna transmission (CRS), Four antenna ports |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.2.2-1 | R.12 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.13 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.14-1 FDD | 10 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-2 FDD | 10 | 16QAM | 1/2 | 3 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-3 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.36 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.14-4 FDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-5 FDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-6 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-7 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD, PDSCH Performance (UE specific RS) without CSI-RS |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.3.0-1 | R.70 FDD | 10 | QPSK | 0.65 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.0-1 | R.71 FDD | 10 | 16QAM | 0.6 | 50 |  | ≥ 2 |  |
| FDD, PDSCH Performance (UE specific RS) Two antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.3.1-1 | R.51 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.1-1 | R.51-1 FDD | 10 | 16QAM | 0.54 | 50 |  | ≥ 2 |  |
| FDD, PDSCH Performance (UE specific RS) Two antenna ports (CSI-RS, non Quasi Co-located) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.3.1-2 | R.52 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.1-2 | R.53 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.1-2 | R.54 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD, PDSCH Performance (UE specific RS) Four antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.3.2-1 | R.43 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-1 | R.50 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.2-2 | R.44 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-2 | R.45 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.3.2-2 | R.45-1 FDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-1 | R.48 FDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-2 | R.60 FDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.3.2-3 | R.64 FDD | 10 | QPSK | 1/3 | 6 |  | 0 |  |
| FDD | Table A.3.3.3.2-1 | R.66 FDD | 10 | 64QAM |  | 50 |  | 11-15 |  |
| FDD | Table A.3.3.3.2-1 | R.69 FDD | 10 | QPSK | 0.74-0.8 | 50 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Single-antenna transmission (CRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.1-1 | R.4 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.42 TDD | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.2 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.2A TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.42-1 TDD | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.42-2 TDD | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-1 | R.42-3 TDD | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-2 | R.3-1 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-2 | R.3 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-3 | R.5 TDD | 3 | 64QAM | 3/4 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3 | R.6 TDD | 5 | 64QAM | 3/4 | 25 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-3 | R.7 TDD | 10 | 64QAM | 3/4 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-3 | R.8 TDD | 15 | 64QAM | 3/4 | 75 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-3 | R.9 TDD | 20 | 64QAM | 3/4 | 100 |  | ≥ 3 |  |
| TDD | Table A.3.4.1-3a | R.6-1 TDD | 5 | 64QAM | 3/4 | 18 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3a | R.7-1 TDD | 10 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3a | R.8-1 TDD | 15 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3a | R.9-1 TDD | 20 | 64QAM | 3/4 | 17 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-3a | R.9-2 TDD | 20 | 64QAM | 3/4 | 83 |  | ≥ 2 |  |
| TDD | Table A.3.4.1-6 | R.41 TDD | 10 | QPSK | 1/10 | 50 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (Channel edge) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.1-4 | R.0 TDD | 3 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| TDD | Table A.3.4.1-4 | R.1 TDD | 10 / 20 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Single-antenna transmission (CRS), Single PRB (MBSFN Configuration) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.1-5 | R.29 TDD | 10 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| TDD, PDSCH Performance: Carrier aggregation with power imbalance |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.1-7 | R.49 TDD | 20 | 64QAM | 0.81-087 | 100 |  | ≥ 5 |  |
| TDD | Table A.3.4.1-7 | R.49-1 TDD | 15 | 64QAM | 0.80-0.86 | 75 |  | ≥ 3 |  |
| TDD, PDSCH Performance, Multi-antenna transmission (CRS), Two antenna ports |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.2.1-1 | R.10 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.11-1 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.11-2 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11-3 TDD | 10 | 16QAM | 1/2 | 40 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11-4 TDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.30 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30-1 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30-2 TDD | 20 | 16QAM | 1/2 | 100 |  | 3 |  |
| TDD | Table A.3.4.2.1-1 | R.35 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.35-1 TDD | 20 | 64QAM | 0.39 | 100 |  | 4 |  |
| TDD | Table A.3.4.2.1-2 | R.35-2 TDD | 10 | 64QAM | 0.47 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.46 TDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.47 TDD | 10 | 16QAM |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-5 TDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-6 TDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-7 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-8 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-9 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-10 TDD | 10 | QPSK | 3/5 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-11 TDD | 10 | QPSK | 0.48-0.58 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-12 TDD | 10 | QPSK | 0.54-0.66 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.10-3 TDD | 10 | 16QAM | 0.57-0.58 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-3 | R.62 TDD | 10 | 16QAM | 1/2 | 3 |  | 0 |  |
| TDD | Table A.3.4.2.1-3 | R.63 TDD | 10 | 64QAM | 1/2 | 1 |  | 0 |  |
| TDD | Table A.3.4.2.1-4 | R.65 TDD | 20 | 256QAM | 0.6 | 100 |  | 11-15 |  |
| TDD | Table A.3.4.2.1-5 | R.67 TDD | 10 | 16QAM | 0.4 | 50 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Multi-antenna transmission (CRS), Four antenna ports |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.2.2-1 | R.12 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.13 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.14 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.14-1 TDD | 10 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.14-2 TDD | 10 | 16QAM | 1/2 | 3 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥2 |  |
| TDD | Table A.3.4.2.2-1 | R.36 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.43-1 TDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-2 TDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-3 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-4 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.43-5 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| TDD, PDSCH Performance, Single antenna port (DRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.1-1 | R.25 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.1-1 | R.26 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.1-1 | R.26-1 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.1-1 | R.27 TDD | 10 | 64QAM | 3/4 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.1-1 | R.27-1 TDD | 10 | 64QAM | 3/4 | 18 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.1-1 | R.28 TDD | 10 | 16QAM | 1/2 | 1 |  | ≥ 1 |  |
| TDD, PDSCH Performance, Two antenna ports (DRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.2-1 | R.31 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.2-1 | R.32 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.2-1 | R.32-1 TDD | 5 | 16QAM | 1/2 | [25] |  | ≥ 1 |  |
| TDD | Table A.3.4.3.2-1 | R.33 TDD | 10 | 64QAM | 3/4 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.2-1 | R.33-1 TDD | 10 | 64QAM | 3/4 | [18] |  | ≥ 1 |  |
| TDD | Table A.3.4.3.2-1 | R.34 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.2 | R.70 TDD | 10 | QPSK | 0.54-0.65 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.2 | R.71 TDD | 10 | 16QAM | 0.5-0.6 | 50 |  | ≥ 2 |  |
| TDD, PDSCH Performance (UE specific RS) Two antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.3-1 | R.51 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.3-1 | R.51-1 TDD | 10 | 16QAM | 0.57 | 50 |  | ≥ 2 |  |
| TDD, PDSCH Performance (UE specific RS) Two antenna ports (CSI-RS, non Quasi Co-located) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.3-2 | R.52 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.3-2 | R.53 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.3-2 | R.54 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD, PDSCH Performance (UE specific RS) Four antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.4-1 | R.44 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.4-1 | R.48 TDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.4-2 | R.60 TDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.4-2 | R.61 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.4-2 | R.61-1 TDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.4-3 | R.64 TDD | 10 | QPSK | 1/3 | 6 |  | 0 |  |
| TDD | Table A.3.4.3.4-1 | R.66 TDD | 20 | 256QAM |  | 100 |  | 11-15 |  |
| TDD | Table A.3.4.3.4-1 | R.69 TDD | 10 | QPSK | 0.61-0.8 | 50 |  | ≥ 1 |  |
| TDD, PDSCH Performance (UE specific RS) Eight antenna ports (CSI-RS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.3.5-1 | R.50 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.3.5-2 | R.45 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.3.5-2 | R.45-1 TDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| FDD, PDCCH / PCFICH Performance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.15 FDD | 10 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.15-1 FDD | 10 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.15-2 FDD | 10 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.16 FDD | 10 | PDCCH |  |  |  |  |  |
| FDD | Table A.3.5.1-1 | R.17 FDD | 5 | PDCCH |  |  |  |  |  |
| TDD, PDCCH / PCFICH Performance |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.15 TDD | 10 | PDCCH |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.15-1 TDD | 10 | PDCCH |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.15-2 TDD | 10 | PDCCH |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.16 TDD | 10 | PDCCH |  |  |  |  |  |
| TDD | Table A.3.5.2-1 | R.17 TDD | 5 | PDCCH |  |  |  |  |  |
| FDD / TDD, PHICH Performance |  |  |  |  |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.18 | 10 | PHICH |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.19 | 10 | PHICH |  |  |  |  |  |
| FDD | Table A.3.6.1 | R.19-1 | 5 | PHICH |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.20 | 5 | PHICH |  |  |  |  |  |
| FDD / TDD | Table A.3.6-1 | R.24 | 10 | PHICH |  |  |  |  |  |
| FDD / TDD, PBCH Performance |  |  |  |  |  |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.21 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.22 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.23 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD, PMCH Performance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.8.1-1 | R.40 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-1 | R.37 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-2 | R.38 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39-1 FDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| TDD, PMCH Performance |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.8.2-1 | R.40 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-1 | R.37 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-2 | R.38 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39-1 TDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39 TDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD, Sustained data rate (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.9.1-1 | R.31-1 FDD | 10 | 64QAM | 0.40 |  |  | ≥ 1 |  |
| FDD | Table A.3.9.1-1 | R.31-2 FDD | 10 | 64QAM | 0.59-0.64 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-1 | R.31-3 FDD | 20 | 64QAM | 0.59-0.62 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-1 | R.31-3A FDD | 10 | 64QAM | 0.85-0.90 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-1 | R.31-3C FDD | 15 | 64QAM | 0.87-0.91 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.1-1 | R.31-4 FDD | 20 | 64QAM | 0.87-0.90 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.1-1 | R.31-4B FDD | 15 | 64QAM | 0.85-0.88 |  |  | ≥ 4 |  |
| FDD | Table A.3.9.1-1 | R.31-5 FDD | 15 | 64QAM | 0.85-0.91 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.1-2 | R.31-6 FDD | 15 | 64QAM | 0.83-0.85 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-3 | R.68 FDD | 20 | 256QAM | 0.74-0.85 |  |  | 11-12 |  |
| FDD | Table A.3.9.1-3 | R.68-1 FDD | 15 | 256QAM | 0.74-0.88 |  |  | 11-12 |  |
| FDD | Table A.3.9.1-3 | R.68-2 FDD | 10 | 256QAM | 0.74-0.85 |  |  | 11-12 |  |
| FDD | Table A.3.9.1-3 | R.68-3 FDD | 5 | 256QAM | 0.77-0.85 |  |  | 11-12 |  |
| TDD, Sustained data rate (CRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.9.2-1 | R.31-1 TDD | 10 | 64QAM | 0.40 |  |  | ≥ 1 |  |
| TDD | Table A.3.9.2-1 | R.31-2 TDD | 10 | 64QAM | 0.59-0.64 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.2-1 | R.31-3 TDD | 20 | 64QAM | 0.59-0.62 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.2-1 | R.31-3A TDD | 15 | 64QAM | 0.87-0.90 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.2-1 | R.31-4 TDD | 20 | 64QAM | 0.87-0.90 |  |  | ≥ 3 |  |
| TDD | Table A.3.9.2-1 | R.31-4A TDD | 20 | 64QAM | 0.87-0.90 |  |  | ≥ 3 |  |
| TDD | Table A.3.9.2-1 | R.31-5 TDD | 15 | 64QAM | 0.85-0.88 |  |  | ≥ 3 |  |
| TDD | Table A.3.9.2-1 | R.31-5A TDD | 15 | 64QAM | 0.85-0.88 |  |  | ≥ 3 |  |
| TDD | Table A.3.9.2-2 | R.68 TDD | 20 | 256QAM |  |  |  | 11-12 |  |
| TDD | Table A.3.9.2-2 | R.68-1 TDD | 15 | 256QAM |  |  |  | 11-12 |  |
| TDD | Table A.3.9.2-2 | R.68-2 TDD | 10 | 256QAM |  |  |  | 11-12 |  |
| TDD | Table A.3.9.2-2 | R.68-3 TDD | 20 | 256QAM |  |  |  | 11-12 |  |
| TDD | Table A.3.9.2-2 | R.68-4 TDD | 15 | 256QAM |  |  |  | 11-12 |  |
| FDD, Sustained data rate test with EPDCCH scheduling (CRS) |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.9.3-1 | R.31E-1 FDD | 10 | 64QAM | 0.40-0 |  |  | ≥ 1 |  |
| FDD | Table A.3.9.3-1 | R.31E-2 FDD | 10 | 64QAM | 0.59-0.66 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.3-1 | R.31E-3 FDD | 20 | 64QAM | 0.59-0.63 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.1-1 | R.31E-3C FDD | 15 | 64QAM | 0.87-0.92 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.3-1 | R.31E-3A FDD | 10 | 64QAM | 0.85-0.92 |  |  | ≥ 2 |  |
| FDD | Table A.3.9.3-1 | R.31E-4 FDD | 20 | 64QAM | 0.87-0.91 |  |  | ≥ 3 |  |
| FDD | Table A.3.9.1-1 | R.31E-4B FDD | 15 | 64QAM | 0.87-0.90 |  |  | ≥ 4 |  |
| TDD, Sustained data rate test with EPDCCH scheduling (CRS) |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.9.4-1 | R.31E-1 TDD | 10 | 64QAM | 0.40-0.41 |  |  | ≥ 1 |  |
| TDD | Table A.3.9.4-1 | R.31E-2 TDD | 10 | 64QAM | 0.59-0.65 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.4-1 | R.31E-3 TDD | 20 | 64QAM | 0.59-0.63 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.4-1 | R.31E-3A TDD | 15 | 64QAM | 0.87-0.92 |  |  | ≥ 2 |  |
| TDD | Table A.3.9.4-1 | R.31E-4 TDD | 20 | 64QAM | 0.87-0.90 |  |  | ≥ 3 |  |
| FDD, ePDCCH performance |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.55 FDD | 10 | EPDCCH |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.56 FDD | 10 | EPDCCH |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.57 FDD | 10 | EPDCCH |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.58 FDD | 10 | EPDCCH |  |  |  |  |  |
| FDD | Table A.3.10.1-1 | R.59 FDD | 10 | EPDCCH |  |  |  |  |  |
| TDD, ePDCCH performance |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.55 TDD | 10 | EPDCCH |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.56 TDD | 10 | EPDCCH |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.57 TDD | 10 | EPDCCH |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.58 TDD | 10 | EPDCCH |  |  |  |  |  |
| TDD | Table A.3.10.2-1 | R.59 TDD | 10 | EPDCCH |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### A.3.3.2.1	Two antenna ports

Table A.3.3.2.1-1: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.10 FDD | R.11 FDD | R.11-1 FDD | R.11-2 FDD | R.11-3 FDD Note 5 | R.11-4 FDD | R.30 FDD | R.30-1 FDD | R.35-1 FDD | R.35 FDD | R.35-2 FDD | R.35-3 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 5 | 10 | 10 | 20 | 15 | 20 | 10 | 15 | 10 |
| Allocated resource blocks (Note 4) |  | 50 | 50 | 50 | 25 | 40 | 50 | 100 | 75 | 100 | 50 | 75 | 50 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 8 | 8 | 9 | 8 | 8 |
| Modulation |  | QPSK | 16QAM | 16QAM | 16QAM | 16QAM | QPSK | 16QAM | 16QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 0.39 | 1/2 | 0.39 | 0.39 |
| Information Bit Payload (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 4392 | 12960 | 12960 | 5736 | 10296 | 6968 | 25456 | 19080 | 30576 | 19848 | 22920 | 15264 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 4392 | 12960 | N/A | 4968 | 10296 | 6968 | 25456 | N/A | N/A | 18336 | N/A | N/A |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 1 | 3 | 3 | 1 | 2 | 2 | 5 | 4 | 5 | 4 | 4 | 3 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 1 | 3 | N/A | 1 | 2 | 2 | 5 | N/A | N/A | 3 | N/A | N/A |
| Binary Channel Bits (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 13200 | 26400 | 26400 | 12000 | 21120 | 13200 | 52800 | 39600 | 79200 | 39600 | 59400 | 39600 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 12384 | 24768 | N/A | 10368 | 19488 | 12384 | 51168 | N/A | N/A | 37152 | N/A | N/A |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 3.953 | 11.664 | 10.368 | 5.086 | 9.266 | 6.271 | 22.910 | 15.264 | 24.461 | 17.712 | 18.336 | 12.211 |
| UE Category |  | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | 4 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: Given per component carrier per codeword. Note 5: For R.11-3 resource blocks of RB6–RB45 are allocated. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.2.1-2: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.46 FDD | R.47 FDD | R.35-4 FDD | R.11-5 FDD | R.11-6 FDD | R.11-7 FDD | R.11-8 FDD | R.11- 9 FDD | R.11- 10 FDD | R.65 FDD | R.10-2 FDD | R.10-3 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 1.4 | 3 | 15 | 10 | 10 | 10 | 10 | 5 | 10 |
| Allocated resource blocks (Note 4) |  | 50 | 50 | 50 | 6 | 15 | 75 | 50 | 50 | 50 | 50 | 25 | 50 |
| Allocated number of PDCCH symbols |  | 2 | 2 | 2 | 4 | 3 | 2 | 2 | 3 | 3 | 2 | 3 | 2 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 8 | 8 | 9 | 9 | 9 |
| Modulation |  | QPSK | 16QAM | 64QAM | 16QAM | 16QAM | 16QAM | QPSK | QPSK | QPSK | 256QAM | QPSK | 16QAM |
| Target Coding Rate |  |  |  | 0.47 | 1/2 | 1/2 | 1/2 | 3/5 | 0.58 | 0.67 | 0. 55 | 1/3 | 0.58 |
| Information Bit Payload (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 5160 | 8760 | 18336 | 1352 | 3368 | 19080 | 7992 | 6968 | 7992 | 31704 | 1800 | 15264 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | n/a | n/a |
| For Sub-Frame 0 | Bits | 5160 | 8760 | 16416 | N/A | 2664 | 19080 | 6968 | N/A | N/A | N/A | 1800 | 14112 |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 1 | 2 | 3 | 1 | 1 | 4 | 2 | 2 | 2 | 6 | 1 | 3 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | n/a | n/a |
| For Sub-Frame 0 | Bits | 1 | 2 | 3 | 1 | 1 | 4 | 2 | N/A | N/A | N/A | 1 | 3 |
| Binary Channel Bits (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 13200 | 26400 | 39600 | 2592 | 7200 | 39600 | 13200 | 12000 | 12000 | 57600 | 6000 | 26400 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | n/a | n/a |
| For Sub-Frame 0 | Bits | 12384 | 24768 | 37152 | N/A | 5568 | 37968 | 12384 | N/A | N/A | N/A | 5184 | 24768 |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 4.644 | 7.884 | 16.310 | 1.082 | 2.961 | 17.172 | 7.0904 | 5.5744 | 6.3936 | 25.363 | 1.620 | 13.6224 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 1 | ≥ 1 | 11-12 | ≥ 1 | ≥ 1 |
| UE DL Category |  | ≥ 6 | ≥ 6 | ≥ 6 | ≥ 6 | ≥ 6 | ≥ 6 | ≥ 6 |  |  | ≥ 13 | ≥ 6 |  |
| Note 1: Void Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Given per component carrier per codeword. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.2.1-3: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.62 FDD | R.63 FDD |
| Channel bandwidth | MHz | 10 | 10 |
| Allocated resource blocks (Note 4) |  | 3 | 1 |
| Allocated DL subframes per 4 Radio Frames (Note 3) |  | 15 | 15 |
| Modulation |  | 16QAM | 64QAM |
| Target Coding Rate |  | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 744 | 408 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Code blocks | 1 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 1584 | 792 |
| Max. Throughput averaged over 4 frames | Mbps | 0.279 | 0.153 |
| UE DL Category |  | 0 | 0 |
| Note 1: 2 symbols allocated to PDCCH Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4] Note 3: The downlink subframes are scheduled at the 0th, 1st, 2nd, 8th, 9th, 10th, 16th, 17th, 18th, 24th, 25th, 26th, 32nd, 33rd, 34th subframes every 40ms. Information bit payload is available if downlink subframe is scheduled. Note 4: Allocated PRB positions start from {9, 10, …, 9+N-1}, where N is the number of allocated resource blocks. |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### A.3.3.3.1	Two antenna port (CSI-RS)

The reference measurement channels in Table A.3.3.3.1-1 apply for verifying demodulation performance for UE-specific reference symbols with two cell-specific antenna ports and two CSI-RS antenna ports.

Table A.3.3.3.1-1: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.51 FDD | R.51-1 FDD |
| Channel bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 (Note 3) | 50 (Note 3) |
| Allocated subframes per Radio Frame |  | 9 | 9 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 0.54 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 11448 | 12960 |
| For Sub-Frames 2,3,7,8 | Bits | 11448 | 12960 |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 9528 | 10680 |
| Number of Code Blocks (Note 4) |  |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 2 | 3 |
| For Sub-Frames 2,3,7,8 | Code blocks | 2 | 3 |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 2 | 2 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 24000 | 24000 |
| For Sub-Frames 2,7 |  | 23600 | 23600 |
| For Sub-Frames 3,8 |  | 23200 | 23200 |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 19680 | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 10.1112 | 11.436 |
| UE Category |  | ≥ 2 | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

The reference measurement channels in Table A3.3.3.1-2 apply for verifying demudlation performance for UE-specific reference symbols with two cell specific antenna ports and two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS in same subframe.

Table A.3.3.3.1-2: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | R.52 FDD | R.53 FDD | R.54 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 (Note 3) | 50 (Note 3) | 50 (Note 3) |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 |
| Modulation |  | 64QAM | 64QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 1,3,4,6,8,9 | Bits | 18336 | 18336 | 11448 |
| For Sub-Frames 2,7 | Bits | 16416 | 16416 | 11448 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 14688 | 14688 | 9528 |
| Number of Code Blocks (Note 4) |  |  |  |  |
| For Sub-Frames 1,3,4,6,8,9 | Code blocks | 3 | 3 | 2 |
| For Sub-Frames 2, 7 | Code blocks | 3 | 3 | 2 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 3 | 3 | 2 |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 1,3,4,6,8,9 | Bits | 36000 | 36000 | 24000 |
| For Sub-Frames 2,7 |  | 34200 | 33600 | 22800 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 29520 | 29520 | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 15.7536 | 15.7536 | 10.1112 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### A.3.4.2.1	Two antenna ports

Table A.3.4.2.1-1: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  | R.10 TDD | R.11 TDD | R.11-1 TDD | R.11-2 TDD | R.11-3 TDD Note 6 | R.11-4 TDD | R.30 TDD | R.30-1 TDD | R.30-2 TDD | R.35 TDD | R.35-1 TDD |  |
| Channel bandwidth |  |  | MHz |  | 10 | 10 | 10 | 5 | 10 | 10 | 20 | 20 | 20 | 10 | 20 |  |
| Allocated resource blocks (Note 5) |  |  |  |  | 50 | 50 | 50 | 25 | 40 | 50 | 100 | 100 | 100 | 50 | 100 |  |
| Uplink-Downlink Configuration (Note 3) |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |
| Allocated subframes per Radio Frame (D+S) |  |  |  |  | 3+2 | 3+2 | 2+2 | 3+2 | 3+2 | 2 | 3+2 | 2+2 | 2 | 2+2 | 2 |  |
| Modulation |  |  |  |  | QPSK | 16QAM | 16QAM | 16QAM | 16QAM | QPSK | 16QAM | 16QAM | 16QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  |  |  |  | 1/3 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 0.39 |  |
| Information Bit Payload (Note 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  |  | Bits |  | 4392 | 12960 | 12960 | 5736 | 10296 | 6968 | 25456 | 25456 | 25456 | 19848 | 30576 |  |
| For Sub-Frames 1,6 |  |  |  |  | 3240 | 9528 | 9528 | 5160 | 9144 | N/A | 22920 | 21384 | N/A | 15840 | N/A |  |
| For Sub-Frame 5 |  |  | Bits |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Sub-Frame 0 |  |  | Bits |  | 4392 | 12960 | N/A | 4968 | 10296 | N/A | 25456 | N/A | N/A | N/A | N/A |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  |  |  |  | 1 | 3 | 3 | 1 | 2 | 2 | 5 | 5 | 5 | 4 | 5 |  |
| For Sub-Frames 1,6 |  |  |  |  | 1 | 2 | 2 | 1 | 2 | N/A | 4 | 4 | N/A | 3 | N/A |  |
| For Sub-Frame 5 |  |  |  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Sub-Frame 0 |  |  |  |  | 1 | 3 | N/A | 1 | 2 | N/A | 5 | N/A | N/A | N/A | N/A |  |
| Binary Channel Bits (Note 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  |  | Bits |  | 13200 | 26400 | 26400 | 12000 | 21120 | 13200 | 52800 | 52800 | 52800 | 39600 | 79200 |  |
| For Sub-Frames 1,6 |  |  |  |  | 10656 | 21312 | 21312 | 10512 | 16992 | 10656 | 42912 | 42912 | N/A | 31968 | N/A |  |
| For Sub-Frame 5 |  |  | Bits |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Sub-Frame 0 |  |  | Bits |  | 12528 | 25056 | N/A | 10656 | 19776 | 12528 | 51456 | N/A | N/A | N/A | N/A |  |
| Max. Throughput averaged over 1 frame (Note 5) |  |  | Mbps |  | 1.966 | 5.794 | 4.498 | 2.676 | 4.918 | 1.39 | 12.221 | 9.368 | 5.091 | 7.138 | 6.115 |  |
| UE Category |  |  |  |  | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | 3 | ≥ 2 | 4 |  |
| Note 1: 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For subframe 1&6, only 2 OFDM symbols are allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Given per component carrier per codeword. Note 6: For R.11-3 resource blocks of RB6–RB45 are allocated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.4.2.1-2: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.46 TDD | R.47 TDD | R.35-2 TDD | R.11-5 TDD | R.11-6 TDD | R.11-7 TDD | R.11-8 TDD | R.11-9 TDD | R.11-10 TDD | R.11-11 TDD | R.11-12 TDD | R.10-3 TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 1.4 | 3 | 5 | 10 | 15 | 10 | 10 | 10 | 10 |
| Allocated resource blocks (Note 5) |  | 50 | 50 | 50 | 6 | 15 | 25 | 50 | 75 | 50 | 50 | 50 | 50 |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated number of PDCCH symbols in normal subframes |  | 2 | 2 | 2 | 4 | 3 | 3 | 2 | 2 | 2 | 3 | 3 | 2 |
| Allocated number of PDCCH symbols in special subframes |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 | 3+2 | 2+2 | 2+2 | 2+2 | 2+2 | 2+2 | 2+2 | 3+2 | 2+2 | 2+2 | 3+2 |
| Modulation |  | QPSK | 16QAM | 64QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | QPSK | QPSK | QPSK | 16QAM |
| Target Coding Rate |  |  |  | 0.47 | 1/2 | 1/2 | 1/2 | 1/2 | 1/2 | 3/5 |  |  |  |
| For Sub-Frames 4,9 |  |  |  |  |  |  |  |  |  |  | 0.58 | 0.66 | 0.58 |
| For Sub-Frames 1,6 |  |  |  |  |  |  |  |  |  |  | 0.48 | 0.54 | 0.57 |
| Information Bit Payload (Note 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 5160 | 8760 | 18336 | 1352 | 3368 | 5736 | 12960 | 19080 | 7992 | 6968 | 7992 | 15264 |
| For Sub-Frames 1,6 |  | 3880 | 7480 | 14688 | 1128 | 3112 | 5160 | 10680 | 15840 | 5736 | 5160 | 5736 | 12216 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 5160 | 8760 | N/A | N/A | N/A | N/A | N/A | N/A | 7992 | N/A | N/A | 14112 |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  | 1 | 2 | 3 | 1 | 1 | 1 | 3 | 4 | 2 | 2 | 2 | 3 |
| For Sub-Frames 1,6 |  | 1 | 2 | 3 | 1 | 1 | 1 | 2 | 3 | 1 | 1 | 1 | 2 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 1 | 2 | N/A | N/A | N/A | N/A | N/A | N/A | 2 | N/A | N/A | 3 |
| Binary Channel Bits (Note 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 13200 | 26400 | 39600 | 2592 | 7200 | 12000 | 26400 | 39600 | 13200 | 12000 | 12000 | 26400 |
| For Sub-Frames 1,6 |  | 10656 | 21312 | 31968 | 2304 | 6192 | 10512 | 21312 | 32112 | 10656 | 10656 | 10656 | 21312 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 12528 | 25056 | N/A | N/A | N/A | N/A | N/A | N/A | 12528 | N/A | N/A | 25056 |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 2.324 | 4.124 | 6.604 | 0.496 | 1.296 | 2.179 | 4.498 | 6.984 | 3.5448 | 2.4256 | 2.7456 | 6.9072 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 1 | ≥ 1 | ≥ 1 |
| Note 1: Void Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Given per component carrier per codeword |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.4.2.1-3: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.62 TDD | R.63 TDD |
| Channel bandwidth | MHz | 10 | 10 |
| Allocated resource blocks (Note 4) |  | 3 | 1 |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 |
| Allocated subframes per Radio Frame (D+S) |  | 4+2 | 4+2 |
| Modulation |  | 16QAM | 64QAM |
| Target Coding Rate |  | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 0,4,5,9 | Bits | 744 | 408 |
| For Sub-Frames 1,6 | Bits | 440 | 280 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 0,4,5,9 | Code blocks | 1 | 1 |
| For Sub-Frames 1,6 | Clode blocls | 1 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 0,4,5,9 | Bits | 1584 | 792 |
| For Sub-Frames 1,6 |  | 1296 | 648 |
| Max. Throughput averaged over 1 frame | Mbps | 0.3856 | 0.2192 |
| UE DL Category |  | 0 | 0 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: Allocated PRB positions start from {9, 10, …, 9+N-1}, where N is the number of allocated resource blocks. |  |  |  |

Table A.3.4.2.1-4: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.65 TDD |  |
| Channel bandwidth | MHz | 20 |  |
| Allocated resource blocks (Note 5) |  | 100 |  |
| Uplink-Downlink Configuration (Note 3) |  | 1 |  |
| Allocated subframes per Radio Frame (D+S) |  | 2+2 |  |
| Modulation |  | 256QAM |  |
| Target Coding Rate |  |  |  |
| Information Bit Payload (Note 5) |  |  |  |
| For Sub-Frames 4,9 | Bits | 63776 |  |
| For Sub-Frames 1,6 |  | 46888 |  |
| For Sub-Frame 5 | Bits | N/A |  |
| For Sub-Frame 0 | Bits | N/A |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |
| For Sub-Frames 4,9 |  | 11 |  |
| For Sub-Frames 1,6 |  | 9 |  |
| For Sub-Frame 5 |  | N/A |  |
| For Sub-Frame 0 |  | N/A |  |
| Binary Channel Bits (Note 5) |  |  |  |
| For Sub-Frames 4,9 | Bits | 115200 |  |
| For Sub-Frames 1,6 |  | 95424 |  |
| For Sub-Frame 5 | Bits | N/A |  |
| For Sub-Frame 0 | Bits | N/A |  |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 22.133 |  |
| UE Category |  | 11-12 |  |
| UE DL Category |  | ≥ 13 |  |
| Note 1: 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For subframe 1&6, only 2 OFDM symbols are allocated to PDCCH. For 256QAM reference channel 1 symbol is allocated. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Given per component carrier per codeword |  |  |  |

Table A.3.4.2.1-5: Fixed Reference Channel two antenna ports when 
EIMTA-MainConfigServCell-r12 is configured

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.67 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |  |  |
| Allocated resource blocks (Note 5) |  | 50 |  |  |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |  |  |
| Target Coding Rate |  | 0.4 |  |  |  |  |  |  |
| Dynamic Uplink-Downlink Configuration (Note 3) |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| Allocated subframes per Radio Frame (D+S) |  | 1+2 | 3+2 | 5+2 | 5+1 | 6+1 | 7+1 | 2+2 |
| Information Bit Payload (Note 5) |  |  |  |  |  |  |  |  |
| For Sub-Frame 0 | Bits | 9912 | 9912 | 9912 | 9912 | 9912 | 9912 | 9912 |
| For Sub-Frame 1 | Bits | 7480 | 7480 | 7480 | 7480 | 7480 | 7480 | 7480 |
| For Sub-Frame 2 | Bits | NA | NA | NA | NA | NA | NA | NA |
| For Sub-Frame 3 | Bits | NA | NA | 9912 | NA | NA | 9912 | NA |
| For Sub-Frame 4 | Bits | NA | 9912 | 9912 | NA | 9912 | 9912 | NA |
| For Sub-Frame 5 | Bits | NA | NA | NA | NA | NA | NA | NA |
| For Sub-Frame 6 | Bits | 7480 | 7480 | 7480 | 9912 | 9912 | 9912 | 7480 |
| For Sub-Frame 7 | Bits | NA | NA | NA | 9912 | 9912 | 9912 | NA |
| For Sub-Frame 8 | Bits | NA | NA | 9912 | 9912 | 9912 | 9912 | NA |
| For Sub-Frame 9 | Bits | NA | 9912 | 9912 | 9912 | 9912 | 9912 | 9912 |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |  |  |  |
| For Sub-Frame 0 |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| For Sub-Frame 1 |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| For Sub-Frame 2 |  | NA | NA | NA | NA | NA | NA | NA |
| For Sub-Frame 3 |  | NA | NA | 2 | NA | NA | 2 | NA |
| For Sub-Frame 4 |  | NA | 2 | 2 | NA | 2 | 2 | NA |
| For Sub-Frame 5 |  | NA | NA | NA | NA | NA | NA | NA |
| For Sub-Frame 6 |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| For Sub-Frame 7 |  | NA | NA | NA | 2 | 2 | 2 | NA |
| For Sub-Frame 8 |  | NA | NA | 2 | 2 | 2 | 2 | NA |
| For Sub-Frame 9 |  | NA | 2 | 2 | 2 | 2 | 2 | 2 |
| Binary Channel Bits (Note 5) |  |  |  |  |  |  |  |  |
| For Sub-Frame 0 | Bits | 25056 | 25056 | 25056 | 25056 | 25056 | 25056 | 25056 |
| For Sub-Frame 1 | Bits | 21312 | 21312 | 21312 | 21312 | 21312 | 21312 | 21312 |
| For Sub-Frame 2 | Bits | NA | NA | NA | NA | NA | NA | NA |
| For Sub-Frame 3 | Bits | NA | NA | 26400 | NA | NA | 26400 | NA |
| For Sub-Frame 4 | Bits | NA | 26400 | 26400 | NA | 26400 | 26400 | NA |
| For Sub-Frame 5 | Bits | NA | NA | NA | NA | NA | NA | NA |
| For Sub-Frame 6 | Bits | 21312 | 21312 | 21312 | 26112 | 26112 | 26112 | 21312 |
| For Sub-Frame 7 | Bits | NA | NA | NA | 26400 | 26400 | 26400 | NA |
| For Sub-Frame 8 | Bits | NA | NA | 26400 | 26400 | 26400 | 26400 | NA |
| For Sub-Frame 9 | Bits | NA | 26400 | 26400 | 26400 | 26400 | 26400 | 26400 |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 2.49 | 4.47 | 6.45 | 5.70 | 6.70 | 7.69 | 3.48 |
| Max. Throughput averaged over 1 frame and over all dynamic UL-DL configurations (Note 5) | Mbps | 5.28 |  |  |  |  |  |  |
| UE Category |  | ≥ 1 |  |  |  |  |  |  |
| Note 1: 2 OFDM symbols are allocated to PDCCH in all subframes Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Given per component carrier per codeword. |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### A.3.4.3.3	Two antenna ports (CSI-RS)

The reference measurement channels in Table A.3.4.3.3-1 apply for verifying demodulation performance for CDM-multiplexed UE specific reference symbols with two cell-specific antenna ports and two CSI-RS antenna ports.

Table A.3.4.3.3-1: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.51 TDD | R.51-1 TDD |
| Channel bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 (Note 5) | 50 (Note 5) |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 | 3+2 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 0.57 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | 11448 | N/A |
| For Sub-Frame 4,9 | Bits | 11448 | 12960 |
| For Sub-Frames 1,6 | Bits | 7736 | 9144 |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 9528 | 10680 |
| Number of Code Blocks (Note 4) |  |  |  |
| For Sub-Frames 4, 9 (non CSI-RS subframe) | Code blocks | 2 | N/A |
| For Sub-Frames 4,9 | Code blocks | 2 | 3 |
| For Sub-Frames 1,6 | Code blocks | 2 | 2 |
| For Sub-Frame 5 |  | N/A | N/A |
| For Sub-Frame 0 | Code blocks | 2 | 2 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 4, 9 (non CSI-RS subframe) | Bits | 24000 | N/A |
| For Sub-Frames 4,9 |  | 22800 | 22800 |
| For Sub-Frames 1,6 |  | 15744 | 15744 |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 19680 | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 4.7896 | 5.4888 |
| UE Category |  | ≥ 2 | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: as per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: 50 resource blocks are allocated in sub-frames 4,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. |  |  |  |

The reference measurement channels in Table A3.4.3.3-2 apply for verifying demudlation performance for UE-specific reference symbols with two cell specific antenna ports and two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS in same subframe.

Table A.3.4.3.3-2: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | R.52 TDD | R.53 TDD | R.54 TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 (Note 5) | 50 (Note 5) | 50 (Note 5) |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 | 3+2 | 3+2 |
| Modulation |  | 64QAM | 64QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frame 4,9 | Bits | 16416 | 16416 | 11448 |
| For Sub-Frames 1,6 | Bits | 11832 | 11832 | 7736 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 14688 | 14688 | 9528 |
| Number of Code Blocks (Note 4) |  |  |  |  |
| For Sub-Frames 4,9 | Code blocks | 3 | 3 | 2 |
| For Sub-Frames 1,6 | Code blocks | 2 | 2 | 2 |
| For Sub-Frame 5 |  | n/a | n/a | n/a |
| For Sub-Frame 0 | Code blocks | 3 | 3 | 2 |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 4,9 |  | 34200 | 33600 | 22800 |
| For Sub-Frames 1,6 |  | 23616 | 23616 | 15744 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 29520 | 29520 | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 7.1184 | 7.1184 | 4.7896 |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: as per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1, 6. |  |  |  |  |

-----------------------------------------------------------------End of Change ------------------------------------------------------------