3GPP TSG-RAN WG4 Meeting #78 	R4-160470

# St. Julian’s, Malta, 15-19 February, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3314 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on LAA measurement accuracy requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Networks, Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-02-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Accuracy requirements for measurements under operation with frame structure 3 were left TBD in the last meeting and need to be updated to complete the Rel-13 LAA work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We propose to reuse Rel-12 accuracy requirements for discovery signal measurements for operation under FS3. References to sections with corresponding RSRP/RSRQ/CSI-RSRP measurement accuracy requirements are included in new section 9.1.18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Accuracy requirements for LAA operation will be left unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3

#### 9.1.18.1	Introduction

The accuracy requirements in this section are defined for the following physical layer measurements: RSRP, RSRQ, CSI-RSRP, and RSSI, where the measurements are performed on cells of E-UTRA carriers during the configured DMTC occasion [2] under operation with frame structure 3 [16].

#### 9.1.18.2	RSRP measurements

##### 9.1.18.2.1	RSRP measurement report mapping

The measurement report mapping for RSRP measurements is as defined in Section 9.1.4.

##### 9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements

Inter-frequency absolute RSRP measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.3.1, provided that CRS is transmitted.

##### 9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements

Inter-frequency relative RSRP measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.3.2, provided that CRS is transmitted.

##### 9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements

Intra-frequency absolute RSRP measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.2.1, provided that CRS is transmitted.

##### 9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements

Intra-frequency relative RSRP measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.2.2, provided that CRS is transmitted.

#### 9.1.18.3	RSRQ measurements

##### 9.1.18.3.1	RSRQ measurement report mapping

The measurement report mapping for RSRQ measurements is as defined in Section 9.1.7.

##### 9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements

Inter-frequency absolute RSRQ measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.6.1, provided that CRS is transmitted.

##### 9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements

Inter-frequency relative RSRQ measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.6.2, provided that CRS is transmitted.

##### 9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements

Intra-frequency absolute RSRQ measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.5.1, provided that CRS is transmitted.

#### 9.1.18.4	CSI-RSRP measurements

##### 9.1.18.4.1	CSI-RSRP measurement report mapping

The measurement report mapping for CSI-RSRP measurements is as defined in Section 9.1.14.3.3.

##### 9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements

Inter-frequency absolute CSI-RSRP measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.14.3.1.1, provided that CSI-RS is transmitted.

##### 9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements

Inter-frequency relative CSI-RSRP measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.14.3.1.2, provided that CSI-RS is transmitted.

##### 9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements

Intra-frequency absolute CSI-RSRP measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.14.3.2.1, provided that CSI-RS is transmitted.

##### 9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements

Intra-frequency absolute CSI-RSRP measurement accuracy requirements for measurements under operation with frame structure 3 are the same as specified in Section 9.1.14.3.2.2, provided that CSI-RS is transmitted.