3GPP TSG-RAN WG4 Meeting #77 	R4-160498

# St. Julian’s, Malta, 15 Feb-19 Feb, 2016

Source:	Ericsson

Title:	TP to 36.852-13, Co-existence analysis for CA Combinations with LAA band 46

Agenda item:				5.5

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.852-13 for co-existence analysis for 2DL/1UL CA combinations with LAA operation, that is, CA between Band 41 and band 46.

We propose to include the text proposal below in Section 8 of 2DL TR 36.852- 13.

# 2. Text Proposal

----- Unchanged sections omitted -----

##### 8.2.6.1.2	Co-existence studies for CA_41A-46A (1UL/2DL)

Table 8.2.6.1.2-1 gives the UL to DL harmonic products for CA band 41 + band 46 with 1UL. It can be seen that 2nd harmonics fall into band 46.

Table 8.2.6.1.2-1: Band 41 and Band 46 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 4992 to 5380 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 7488 to 8070 |  | 15450 to 17775 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 41 and band 46 DL carriers is calculated and shown in table 8.2.6.1.2-2 below:

Table 8.2.6.1.2-2: Band 41 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2496 | 2690 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 4992 to 5380 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 7488 to 8070 |  | 15450 to 17775 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 2460 to 3429 |  | 7646 to 8615 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 230 to 933 10142 to 11305 |  | 7610 to 9354 12796 to 14540 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 2476 to 2710 |  | 5130 to 5945 |  |

It can be seen from Table 8.2.6.1.2-2 that the 2rd harmonics fall into the E-UTRA operating BS receive band 46. It can also be seen none of the 3rd harmonics any E-UTRA operating BS receive bands.

It can also be seen that two-tone 3rd order IMD products may fall into Band 7, 22, 38, 41 and 42 and that the three-tone 3rd order IMD products may fall into Band 5, 6, 8, 12, 13, 14, 15, 16, 17, 18, 19, 20, 26, 27, 28, 29, 31, 32, 44, 45, 67 and 68. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 5, 6, 7, 8, 12, 13, 14, 15, 16, 17, 18, 19, 22, 20, 26, 27, 28, 29, 31, 32, 38, 41, 42, 44, 45, 67 and 68 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 41 and Band 46 BS transmitters do not share the same antenna with any of these receivers.

----- Unchanged sections omitted -----