3GPP TSG-RAN WG4 Meeting #78	R4-160815

St Julians, Malta,  15-19 Feb, 2016

Agenda Item:	6.8.2.3

Source: 		Ericsson

Title: 	PDSCH, SDR demodulation performance

Document for:	Discussion

# Introduction

During the email discussion on the reflector the transport formats for SDR testing were proposed by Qualcomm.

The proposal is given in the following tables

Table 1: Proposed transport formats for FDD SDR TM3 with 64QAM

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MCS 27 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM |  |  | CFI |  | PRB | tones/ RB | CRS tones/ RB |  | PSS/SSS/PBCH |  |  | data tones |  | chan bits (2L) |  | TBS | TBS + CRC | code rate |
| 20MHz | normal SF |  | 1 |  | 100 | 156 | 20 | 0 |  |  | 13600 |  |  | 163200 |  | 128496 | 129024 | 0.791 |
|  | SF 0 |  | 1 |  | 100 | 156 | 20 | 384 |  |  | 13216 |  |  | 158592 |  | 128496 | 129024 | 0.814 |
|  | SF 5 |  | 1 |  | 96 | 156 | 20 | 144 |  |  | 12912 |  |  | 154944 |  | 124464 | 124992 | 0.807 |
| 15MHz | normal SF |  | 1 |  | 75 | 156 | 20 | 0 |  |  | 10200 |  |  | 122400 |  | 93800 | 94208 | 0.77 |
|  | SF 0 |  | 1 |  | 75 | 156 | 20 | 384 |  |  | 9816 |  |  | 117792 |  | 93800 | 94208 | 0.8 |
|  | SF 5 |  | 1 |  | 71 | 156 | 20 | 144 |  |  | 9512 |  |  | 114144 |  | 90816 | 91200 | 0.799 |
| 10MHz | normal SF |  | 1 |  | 50 | 156 | 20 | 0 |  |  | 6800 |  |  | 81600 |  | 63776 | 64064 | 0.785 |
|  | SF 0 |  | 1 |  | 50 | 156 | 20 | 384 |  |  | 6416 |  |  | 76992 |  | 63776 | 64064 | 0.832 |
|  | SF 5 |  | 1 |  | 47 | 156 | 20 | 144 |  |  | 6248 |  |  | 74976 |  | 59256 | 59520 | 0.794 |

Table 2 Proposed transport formats for FDD SDR TM3 with 256QAM

|  |  |  |  |  |  |  |  |  |  | MCS 26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256QAM |  | CFI |  | PRB | tones/ RB | CRS tones/ RB | PSS/SSS/ PBCH | data tones | chan bits (2L) | TBS | TBS + CRC | code rate |
| 20MHz | normal SF | 1 |  | 100 | 156 | 20 | 0 | 13600 | 217600 | 169544 | 170240 | 0.782 |
|  | SF 0 | 1 |  | 100 | 156 | 20 | 384 | 13216 | 211456 | 169544 | 170240 | 0.805 |
|  | SF 5 | 1 |  | 96 | 156 | 20 | 144 | 12912 | 206592 | 161760 | 162432 | 0.786 |
| 15MHz | normal SF | 1 |  | 75 | 156 | 20 | 0 | 10200 | 163200 | 128496 | 129024 | 0.791 |
|  | SF 0 | 1 |  | 75 | 156 | 20 | 384 | 9816 | 157056 | 128496 | 129024 | 0.822 |
|  | SF 5 | 1 |  | 71 | 156 | 20 | 144 | 9512 | 152192 | 124464 | 124992 | 0.821 |
| 10MHz | normal SF | 1 |  | 50 | 156 | 20 | 0 | 6800 | 108800 | 84760 | 85120 | 0.782 |
|  | SF 0 | 1 |  | 50 | 156 | 20 | 384 | 6416 | 102656 | 84760 | 85120 | 0.829 |
|  | SF 5 | 1 |  | 47 | 156 | 20 | 144 | 6248 | 99968 | 81176 | 81536 | 0.816 |

Table 3: Proposed transport formats for TDD SDR TM3 with 64QAM

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MCS 27 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM |  |  | CFI |  | PRB | tones/RB | CRS tones/RB |  | PSS/SSS/PBCH |  |  | data tones |  | chan bits (2L) |  | TBS | TBS + CRC | code rate |
| 20MHz | normal SF |  | 1 |  | 100 | 156 | 20 | 0 |  |  | 13600 |  |  | 163200 |  | 128496 | 129024 | 0.791 |
|  | SF 0 |  | 1 |  | 100 | 156 | 20 | 312 |  |  | 13288 |  |  | 159456 |  | 128496 | 129024 | 0.809 |
|  | SF 5 |  | 1 |  | 96 | 156 | 20 | 72 |  |  | 12984 |  |  | 155808 |  | 124464 | 124992 | 0.802 |
| 15MHz | normal SF |  | 1 |  | 75 | 156 | 20 | 0 |  |  | 10200 |  |  | 122400 |  | 93800 | 94208 | 0.77 |
|  | SF 0 |  | 1 |  | 75 | 156 | 20 | 312 |  |  | 9888 |  |  | 118656 |  | 93800 | 94208 | 0.794 |
|  | SF 5 |  | 1 |  | 71 | 156 | 20 | 72 |  |  | 9584 |  |  | 115008 |  | 90816 | 91200 | 0.793 |
| 10MHz | normal SF |  | 1 |  | 50 | 156 | 20 | 0 |  |  | 6800 |  |  | 81600 |  | 63776 | 64064 | 0.785 |
|  | SF 0 |  | 1 |  | 50 | 156 | 20 | 312 |  |  | 6488 |  |  | 77856 |  | 63776 | 64064 | 0.823 |
|  | SF 5 |  | 1 |  | 47 | 156 | 20 | 72 |  |  | 6320 |  |  | 75840 |  | 59256 | 59520 | 0.785 |

Table 3: Proposed transport formats for TDD SDR TM3 with 64QAM

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MCS 26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256QAM |  |  | CFI |  | PRB | tones/RB | CRS tones/RB |  | PSS/SSS/PBCH |  |  | data tones |  | chan bits (2L) |  | TBS | TBS + CRC | code rate |
| 20MHz | normal SF |  | 1 |  | 100 | 156 | 20 | 0 |  |  | 13600 |  |  | 217600 |  | 169544 | 170240 | 0.782 |
|  | SF 0 |  | 1 |  | 100 | 156 | 20 | 312 |  |  | 13288 |  |  | 212608 |  | 169544 | 170240 | 0.801 |
|  | SF 5 |  | 1 |  | 96 | 156 | 20 | 72 |  |  | 12984 |  |  | 207744 |  | 161760 | 162432 | 0.782 |
| 15MHz | normal SF |  | 1 |  | 75 | 156 | 20 | 0 |  |  | 10200 |  |  | 163200 |  | 128496 | 129024 | 0.791 |
|  | SF 0 |  | 1 |  | 75 | 156 | 20 | 312 |  |  | 9888 |  |  | 158208 |  | 128496 | 129024 | 0.816 |
|  | SF 5 |  | 1 |  | 71 | 156 | 20 | 72 |  |  | 9584 |  |  | 153344 |  | 124464 | 124992 | 0.815 |
| 10MHz | normal SF |  | 1 |  | 50 | 156 | 20 | 0 |  |  | 6800 |  |  | 108800 |  | 84760 | 85120 | 0.782 |
|  | SF 0 |  | 1 |  | 50 | 156 | 20 | 312 |  |  | 6488 |  |  | 103808 |  | 84760 | 85120 | 0.82 |
|  | SF 5 |  | 1 |  | 47 | 156 | 20 | 72 |  |  | 6320 |  |  | 101120 |  | 81176 | 81536 | 0.806 |

# Simulation results

The simulation results are given in the plots and summarized in the tables separately for 64 and 256 QAM.

The results are also introduced in [1].

## SDR simulations with 64QAM

Table 4: Simulated  SNR to achieve 85% of maximum T-put

| Case | FDD |  |  | TDD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 10 MHz | 15 MHz | 20 MHz | 10 MHz | 15 MHz | 20 MHz |
| Target SNR [dB] | 18.29 | 17.76 | 18.65 | 17.91 | 17.69 | 17.96 |

## SDR simulations with 256QAM

Table 5: Simulated  SNR for 4Rx SDR with 245 QAM to achieve 85% of maximum T-put

| Case | FDD |  |  | TDD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 10 MHz | 15 MHz | 20 MHz | 10 MHz | 15 MHz | 20 MHz |
| Target SNR [dB] | 23.77 | 23.86 | 23.85 | 24.36 | 24.67 | 24.38 |

# References

- R4-160804: Summary of results for PDSCH SDR demodulation, Ericsson