3GPP TSG-RAN WG4 Meeting #78bis	R4- 161932

San Jose Del Cabo, Mexico, 11 – 15 April, 2016

Source: 	CATR

Title: 	Proposal for revision of EMC standards for base station and mobile terminal equipment

Agenda Item:	4.2.1, 4.2.2

Document for:	Discussion

Introduction

The objectives of this study item include revise the following EMC standards:

- 3GPP TS 36.113 V13.0.0 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)”

- 3GPP TS 36.124 V13.0.0 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); ElectroMagnetic Compatibility (EMC) requirements for mobile terminals and ancillary equipment”;

- 3GPP TS 37.113 V13.1.0 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network;   E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)”

It is known that the Information Communication Technology (ICT) is increasingly developing, as related standards moving forward, the previous EMC standards should be revised up to date. Although EMC test technologies fall behinds the development of ICT, it has changed in nearly a decade. However, the present 3GPP EMC standards still referring to old standards, e.g. IEC 61000-6-3: 1996; "Electromagnetic compatibility (EMC) - Part 6: Generic standards - Section 3: Emission standard for residential, commercial and light industrial environments". Many EMC test method or limits are not suitable for the newly requirements for mainstream ICT equipment.

This contribution provides update for the references of standards and related parts of the references.

Discussion

Studying the 3GPP TS 36.113 V13.0.0, 3GPP TS 36.124 V13.0.0, and 3GPP TS 37.113 V13.1.0, many referenced standards are found to be updated.  There are two kinds of revision for 3GPP TS 36.113 V13.0.0, 3GPP TS 36.124 V13.0.0, and 3GPP TS 37.113 V13.1.0:

The first kind of revision is only update the name and version of referenced standards, including: IEC 61000-6-3: 1996: "Electromagnetic compatibility (EMC) - Part 6: Generic standards – Section 3: Emission standard for residential, commercial and light industrial environments" would be revised to IEC 61000-6-3:2011: "Electromagnetic compatibility (EMC) - Part 6-3: Generic standards – Emission standard for residential, commercial and light-industrial environments". Similar revision would be applied to referenced standards: IEC 61000-6-1 (1997), ITU-R Recommendation SM.1539 (2001), and ETSI ETR 027 (1991).

The second kind of revision is update name, version and test methods, including: IEC 61000-3-2 (2004): "Electromagnetic compatibility (EMC) - Part 3: Limits - Section 2: Limits for harmonic current emissions (equipment input current ≤ 16 A)" would be revised to IEC 61000-3-2:2014: "Electromagnetic compatibility (EMC) - Part 3-2: Limits - Limits for harmonic current emissions (equipment input current ≤ 16 A per phase) ". It has changed to a more accurate specification of the general test conditions for information technology equipment and the addition of optional test conditions for information technology equipment with external power supplies or battery chargers. Such details would be revised in the standards as well. This kind of revision would be applied to referenced standards: ISO 7637-1 (1990), ISO 7637-2 (1990), IEC 61000-3-3 (2002), and IEC 61000-3-12 (2005).

Conclusion

This contribution provides the possibility to bring fresh brain to the EMC standards 3GPP TS 36.113 V13.0.0, 3GPP TS 36.124 V13.0.0, and 3GPP TS 37.113 V13.1.0 makes the standards in accordance with most recent standards which 3GPP TS 36.113 V13.0.0, 3GPP TS 36.124 V13.0.0, and 3GPP TS 37.113 V13.1.0 referred to. And clauses in the standards would be revised as well.