3GPP TSG-RAN WG4 Meeting#79    						     							       R4-163809

# Nanjing, China,  23-27 May 2016

Title:	Discussion on the working scope of multicarrier operation for V2V

Source:	CMCC

Agenda Item:	7.13.1

Document for:	Approval

- Introduction

In RAN1 #84bis, a LS on V2X multicarrier configuration was approved [1]. RAN1 has agreed that it is necessary that for LTE can support multichannel operation for PC5 V2V. Additionally, RAN1 has agreed that multicarrier operation with Uu and V2V should be supported:

Conclusion:

- RAN1 may investigate following coexistence issues of PC5-based V2V operation and legacy Uu operation

  - Inter-UE coexistence in the same carrier frequency

    - This includes the case where in-band emission of PC5-V2V transmission from UE A interferes with UL transmission from UE B in the same carrier and vice versa.

  - Intra-UE coexistence in the same carrier frequency

    - This includes the following cases:

      - (1) a single UE is scheduled to transmit PC5-V2V and UL at the same time.

      - (2) a single UE is monitoring PC5-V2V and DL simultaneously.

  - Intra-UE coexistence in different carrier frequency

    - This is similar to the issue of ‘intra-UE coexistence in the same carrier frequency’ but differs from the fact that the transmissions/receptions are in two different carrier frequencies.

  - Note that existing scheme/solution is not precluded to solve coexistence issues

- If existing scheme/solution are not sufficient, detailed solutions are FFS

In this paper, we would like to overview RAN4 work for multicarrier operation for V2V.

- Discussion

  - Support of multicarrier operation with Uu and V2V

When the given UE is under E-UTRAN coverage, resources can be allocated to UE through Uu in order to increase spectrum efficiency. What’s more, considering large coverage of E-UTRAN eNBs and fruitful information through Uu, under the help of the messages from eNBs, the given UE can get more information than message exchanges only between nearby UEs. So, it is reasonable and feasible for multicarrier operation with Uu and V2V.

Observation#1: multicarrier operation could help V2V work better.

  - Support of simultaneous transmission of Uu and V2V

Considering supporting of multicarrier operation with Uu and V2V, there is a probability of simultaneous transmission through Uu and V2V. It is not clear if RAN1 or/and RAN2 have solutions for this problem. Even there is some signalling or scheme to settle the issue mentioned above, UE may miss a certain number of transmit opportunities and complexity at BS side will be increased.

Observation#2: It is unclear if RAN1 or/and RAN2 have solution to avoid simultaneous transmission through Uu and V2V

Proposal#1: Simultaneous transmission on Uu and V2V should better be supported to keep flexibility

  - Support of the 23dBm maximum transmission power for both carriers

Considering the UE of a given vehicle transmit and receive safety-related V2V messages, the maximum transmission power should not be reduced as a result of supporting simultaneous transmission of Uu and V2V. What’s more, the power consumption of vehicle based UE is not the problem as the traditional UEs.

Proposal#2: simultaneous 23dBm maximum transmission power on both carriers should be supported

  - Impact on work plan

Considering the limited meeting cycles in RAN4 meeting, the features mentioned above may not be full completed by the end of WI. We recommend the approach used for CA WIs, i.e. simultaneous transmission of Uu and V2V can be discussed continuously in the TEI of Rel-14 after simultaneous reception done.

Proposal#3: Simultaneous transmission through Uu and V2V is recommended to be done after the completion of work on simultaneous reception.

- Conclusion

Observation#1: multicarrier operation could help V2V work better.

Observation#2: It is unclear if RAN1 or/and RAN2 have solution to avoid simultaneous transmission through Uu and V2V

Proposal#1: Simultaneous transmission on Uu and V2V should better be supported to keep flexibility

Proposal#2: simultaneous 23dBm maximum transmission power on both carriers should be supported

Proposal#3: Simultaneous transmission through Uu and V2V is recommended to be done after the completion of work on simultaneous reception.

- Reference

[1] R1-163746, LS on V2X multicarrier configuration, RAN WG1, Ericsson