3GPP TSG-RAN WG4 Meeting #80	R4-165034

Gothenburg, Sweden, 22 - 26 Aug, 2016

Agenda Item:	8.10.3.2

Source: 	Intel Corporation

Title: 	Discussion on UE demodulation performance for unidirectional HST SFN channels

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. One objective of this WI is to identify downlink demodulation requirements under the identified high speed scenarios. According to the high speed Work Item Description (WID) [1], the target high speed train (HST) moving speed is at least 350km/h and at most 750km/h. When the train’s moving speed is greater than 350km/h, one candidate solution is the so-called “unidirectional” RRH SFN arrangement, where RRHs with unidirectional antenna are deployed along the railway. In the ideal case, the antenna side-lobe’s impact is assumed to be negligible for the unidirectional deployment. Therefore, in contrast to the “bidirectional” HST SFN, a legacy receiver can be used, and a normal baseline MMSE receiver is sufficient to maintain satisfactory demodulation performance in unidirectional HST SFN [2]. However, controversial results have been shown in [3] that there is noticeable side-lobe’s impact (i.e. referred to as echo channel in [3]), which causes throughput degradation when a train is approaching a RRH. Moreover, in RAN4#79 Meeting, a number of factors that may have impact on the performance degradation for UE in unidirectional HST SFN channels were captured in the way forward [4].

In this contribution, we first share our view on the unidirectional HST SFN channels, and then show our throughput simulation results for legacy UE under unidirectional HST SFN deployment. There are two basic aspects:

- Doppler shift estimation range for train speed up to 750km/h;

- The impact on the path loss profile caused by antenna radiation pattern and the tilt angle at the RRH.

- Issues in unidirectional HST SFN

  - Doppler shift estimation for train speed up to 750km/h

Given the carrier frequency fc = 2.7GHz and train speed is v = 750km/h, the maximum Doppler shift fd for the high speed train can be up to (c is the speed of light)

fd=fc×vc=1875Hz.

In general, for a legacy UE, the Doppler shift can be estimated by CRS, which is inserted in each subframe as shown in Fig. 1 below.

Fig. 1. Locations of CRS within a resource block of a subframe for a normal CP system.

The Doppler shift estimation range would be determined by the shortest time difference Δt between two CRS-inserted OFDM symbols, given by

-π<2π∙fd∙∆t<π.

Considering the duration of one subframe is 1ms, for a legacy UE, the capability of CRS-based Doppler shift estimation would be around +/–2000Hz. This mean that:

- Observation 1: When a train is running at speed up to 750km/h, the Doppler shift would almost reach the CRS-based estimation limit for a legacy UE. It would be very challenging for a legacy UE to accurately estimate that high Doppler shift, especially when SNR is low, in the unidirectional RRH SFN.

Apparently, this high Doppler shift in such high speed (750km/h) scenario would be a problem for bidirectional RRH SFN as well.

  - Impact on path loss of the directional antenna radiation pattern and its tilt angle at RRH

In [2], a directional antenna radiation pattern is provided, which is similarly produced as in Fig.2.

Fig. 2: Assumed antenna radiation pattern for unidirectional RRH SFN scenario

Fig. 3 demonstrates a train running in the unidirectional RRH SFN from RRH2 (blue) to RRH3 (black), with antenna tilt angle of θ. Moreover, Fig. 4 (a) – (d) shows the path loss of the four consecutive RRHs with different tilt angles, i.e. 5o, 30o, 60o and 80o, respectively. The other necessary parameters are summarised in Table 1.

| Parameters | Value |
| --- | --- |
| Ds | 1000m |
| Dmin | 300m |
| v | 350km/h |
| fd | 875Hz |

Table 1. Parameters for Scenario 1

Fig. 3 Unidirectional RRH SFN

Fig. 4 Path loss profile of four RRHs when train runs from RRH2 (blue) to RRH3 (black)

Fig. 4 illustrates the path loss of tilt angle θ being (a) 5o, (b) 30 o, (c) 60 o and (d) 80 o. It can be seen that for the smaller angle (θ = 5o), received power from RRH1 (red) and RRH2 (blue) has smaller gap; while, for the larger angle (θ = 80o), a more mixed power profile is observed due to the larger side-lobe impact. Similar to the previous findings in bi-directional RRH SFN, the side-lobe’s impact is twofold: one is to introduce multiple paths with large and different Doppler shifts; the other one is to cause frequency tracking problem when the strongest path is switching very quickly from one lobe to another, and the Doppler shift difference of the two lobes is rather large. Unlike bi-directional RRH SFN, the most detrimental case for uni-directional RRH SFN happens when a train/UE is close to one RRH. In Fig. 5, it shows the simulation results for throughput performance with MCS=19, and assuming the train speed is 350km/h.

Figure 5 : Throughput performance of legacy UE under unidirectional HST deployment

- Observation 2: The antenna tilt angle of θ (combined with the antenna radiation pattern) at RRHs has great impact on the throughput performance. The selection of tilt angle θ can be an optimization problem.

- Proposal 1: To specify the antenna radiation pattern and the tilt angle of θ at RRHs for further study on unidirectional HST SFN. For the current antenna pattern assumption and RRH SFN deployment, we propose to select θ = 60 o, as it provides the best throughput performance

- Proposal 2: To further study the cause of the performance degradation and the corresponding enhanced channel estimation scheme for unidirectional HST SFN.

  - Performance impacts from the side-lobe beams from RRHs

  - UE behaviours tracking dynamic Doppler spread associated with unidirectional beams

- Conclusion

In this contribution, we share our view on the unidirectional HST SFN channels:

- Observation 1: When a train is running at speed up to 750km/h, the Doppler shift would almost reach the CRS-based estimation limit for a legacy UE. It would be very challenging for a legacy UE to accurately estimate that high Doppler shift, especially when SNR is low, in the unidirectional RRH SFN.

- Observation 2: The antenna tilt angle of θ (combined with the antenna radiation pattern) at RRHs has great impact on the throughput performance. The selection of tilt angle θ can be an optimization problem.

- Proposal 1: To specify the antenna radiation pattern and the tilt angle of θ at RRHs for further study on unidirectional HST SFN. For the current antenna pattern assumption and RRH SFN deployment, we propose to select θ = 60 o, as it provides the best throughput performance

- Proposal 2: To further study the cause of the performance degradation and the corresponding enhanced channel estimation scheme for unidirectional HST SFN.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DoCoMo Inc., Huawei, HiSilicon, Dec. 2015.[2] R4-162469, “Discussion on the specification of performance requirements for unidirectional deployment”, Ericsson, April 2016.

[3] R4-161883, “UE downlink performance investigation for HST unidirectional SFN”, ITRI, April 2016.

[4] R4-164765, “Way forward on uni-directional RRH arrangement,” Ericsson, Mediatek, RAN WG4 Meeting #79, Nanjing, China, 23-27 May, 2016