3GPP TSG-WG4 Meeting #80	R4-166487

# Gothenburg, Sweden, 22nd – 26th August 2016

Agenda item: 	6.7.4.1.2

Source: 	Qualcomm Incorporated

Title: 	NPDCCH Demodulation Simulation Results

Document for:	Discussion

# 1	Introduction

For NPDCCH, RAN4 agreed with the following simulation assumption, and related main configurations listed in Table 1. Table 2 lists the corresponding RMCs. This paper provides the simulation results with the proposed number of repetitions to satisfy 1% Pm-dsg around the target SNR.

# Table 1: Minimum performance for NPDCCH

| Simulation number | BW | Deployment Mode | Reference Channel | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) (Note 1) | Target SNR (dB |
| 1 | 180KHz | In-band | R.NB1.4 | Note 1 | [EPA5] | 2 | 1 | [-6] |
| 2 | 180KHz | In-band | R.NB1.4 | Note 1 | [ETU1] | 2 | 1 | [-12] |
| 3 | 180KHz | Standalone/Guard-band | R.NB1.3 | Note 1 | [EPA5] | 1 | 1 | [-6] |
| 4 | 180KHz | Standalone/Guard-band | R.NB1.3 | Note 1 | [ETU1] | 1 | 1 | [-12] |
| Note 1: Choose one repetition level from {16,32,64,128,256,512} to satisfy 1% BLER at target SNR=-6dB for tests 1 and 3, and SNR=-12dB for tests 2 and 4. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. |  |  |  |  |  |  |  |  |

# Table 2: RMC for NPDCCH

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.3 | R.NB1.4 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | kHz | 180 | 180 |
| Aggregation level | NCCE | 2 | 2 |
| DCI Format |  | N1 | N1 |
| Cell ID |  | 0 | 0 |
| Payload (without CRC) | Bits | 23 | 23 |

The simulation assumes that NRS is available in every subframe in the frame except subframe 5 which contains NPSS and subframe 9 in every other frame which contains NSSS. This enables channel estimation to be performed using time domain filtering over destaggered NRS tones. A low filter coefficient value is chosen since the channel model considered is a slow fading channel with 1Hz/5Hz doppler.

# 2	Simulation Results

The following plot shows the NPDCCH BLER performance with respect to SNR(dB) for simulation number 1 in Table 1.

| Simulation number | BW | Deployment Mode | Reference Channel | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) (Note 1) | Target SNR (dB |
| 1 | 180KHz | In-band | R.NB1.4 | Note 1 | [EPA5] | 2 | 1 | [-6] |
| Note 1: Choose one repetition level from {16,32,64,128,256,512} to satisfy 1% BLER at target SNR=-6dB for tests 1 and 3, and SNR=-12dB for tests 2 and 4. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. |  |  |  |  |  |  |  |  |

Observation 1a: For simulation number 1, 1% Pm-dsg is achieved at -4.5dB target SNR using 32 repetitions.

Observation 1b: For simulation number 1, 1% Pm-dsg is achieved at -0.8dB target SNR using 16 repetitions.

The following plot shows the NPDCCH BLER performance with respect to SNR(dB) for simulation number 2 in Table 1.

| Simulation number | BW | Deployment Mode | Reference Channel | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) (Note 1) | Target SNR (dB |
| 2 | 180KHz | In-band | R.NB1.4 | Note 1 | [ETU1] | 2 | 1 | [-12] |
| Note 1: Choose one repetition level from {16,32,64,128,256,512} to satisfy 1% BLER at target SNR=-6dB for tests 1 and 3, and SNR=-12dB for tests 2 and 4. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. |  |  |  |  |  |  |  |  |

Observation 2a: For simulation number 2, 1% Pm-dsg is achieved at -9dB target SNR using 128 repetitions.

Observation 2b: For simulation number 2, 1% Pm-dsg is achieved at -6.5dB target SNR using 64 repetitions.

The following plot shows the NPDCCH BLER performance with respect to SNR(dB) for simulation number 3 in Table 1.

| Simulation number | BW | Deployment Mode | Reference Channel | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) (Note 1) | Target SNR (dB |
| 3 | 180KHz | Standalone/Guard-band | R.NB1.3 | Note 1 | [EPA5] | 1 | 1 | [-6] |
| Note 1: Choose one repetition level from {16,32,64,128,256,512} to satisfy 1% BLER at target SNR=-6dB for tests 1 and 3, and SNR=-12dB for tests 2 and 4. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. |  |  |  |  |  |  |  |  |

Observation 3a: For simulation number 3, 1% Pm-dsg is achieved at -6dB target SNR using 64 repetitions.

Observation 3b: For simulation number 3, 1% Pm-dsg is achieved at -1dB target SNR using 32 repetitions.

The following plot shows the NPDCCH BLER performance with respect to SNR(dB) for simulation number 4 in Table 1.

| Simulation number | BW | Deployment Mode | Reference Channel | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) (Note 1) | Target SNR (dB |
| 4 | 180KHz | Standalone/Guard-band | R.NB1.3 | Note 1 | [ETU1] | 1 | 1 | [-12] |
| Note 1: Choose one repetition level from {16,32,64,128,256,512} to satisfy 1% BLER at target SNR=-6dB for tests 1 and 3, and SNR=-12dB for tests 2 and 4. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. |  |  |  |  |  |  |  |  |

Observation 4a: For simulation number 4, 1% Pm-dsg is achieved at -9.5dB target SNR using 256 repetitions.

Observation 4b: For simulation number 4, 1% Pm-dsg is achieved at -5.5dB target SNR using 128 repetitions.

# 3	Conclusions

This paper contains NPDCCH simulation results for alignment and made the following observations:

Observation 1a: For simulation number 1, 1% Pm-dsg is achieved at -4.5dB target SNR using 32 repetitions.

Observation 1b: For simulation number 1, 1% Pm-dsg is achieved at -0.8dB target SNR using 16 repetitions.

Observation 2a: For simulation number 2, 1% Pm-dsg is achieved at -9dB target SNR using 128 repetitions.

Observation 2b: For simulation number 2, 1% Pm-dsg is achieved at -6.5dB target SNR using 64 repetitions.

Observation 3a: For simulation number 3, 1% Pm-dsg is achieved at -6dB target SNR using 64 repetitions.

Observation 3b: For simulation number 3, 1% Pm-dsg is achieved at -1dB target SNR using 32 repetitions.

Observation 4a: For simulation number 4, 1% Pm-dsg is achieved at -9.5dB target SNR using 256 repetitions.

Observation 4b: For simulation number 4, 1% Pm-dsg is achieved at -5.5dB target SNR using 128 repetitions.

# 4	References

- R4-79AH-0161, “Updated simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, Samsung, ZTE