3GPP TSG-RAN WG4 Meeting #80bis                                                                                          R4-167375

Ljubljana, Slovenia, 10 – 14 Oct, 2016

Agenda Item:	6.3.4.2.1

Source: 	Samsung

Title: 	Simulation results for NB-IoT PRACH

Document for:	Discussion

Introduction

In this contribution, we give simulation results for NB-IoT PRACH.

Discussion

Evaluation assumptions were summarized in table below.

|  | Parameter | Values |
| --- | --- | --- |
|  | NPRACH Preamble format | 0 |
|  | Number of Repetitions | 8, 32 |
|  | Number of subcarriers | 12 |
|  | Antenna configuration | 1x2 |
|  | Cell ID | 0 |
|  | NPRACH signature | 0 |
|  | Frequency offset | 0Hz (AWGN); [200Hz] EPA1 |
|  | Propagation channels | AWGN, EPA1 |
|  | Detection performance | Missed detection rate (1%) with false alarm rate (0.1%) |
|  | Timing estimation | Timing error probability with limits 2.5us, [5us] |

| 1. AWGN with frequency offset=0Hz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Table1. Simulation results for missed detection probability[=1%, option1 of R4-167115] with limits[=2.5us, 5us] for AWGN[frequency offset=0Hz] |  |  |  |  |  |  |  |  |  |
| Repetition level | SNR@2.5us(1%) | SNR@5us(1%) |  |  |  |  |  |  |  |
| 8 | -1.6 | -5.3 |  |  |  |  |  |  |  |
| 32 | -6.6 | -9.3 |  |  |  |  |  |  |  |
| 2. EPA1 with frequency offset=200Hz |  |  |  |  |  |  |  |  |  |
| Table2. Simulation results for missed detection probability[=1%, option1 of R4-167115] with limits[=2.5us, 5us] for EPA1[frequency offset=200Hz] |  |  |  |  |  |  |  |  |  |
| Repetition level | SNR@2.5us(1%) | SNR@5us(1%) |  |  |  |  |  |  |  |
| 8 | 4.5 | 2.9 |  |  |  |  |  |  |  |
| 32 | -1.4 | -2.6 |  |  |  |  |  |  |  |

Conclusion

In this contribution, we give simulation results for NB-IoT PRACH.