3GPP TSG RAN WG4 Meeting #81	R4-1609246

Reno, USA, 14 – 18 NOV 2016

Agenda Item:	6.3.4.2.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Updated NPRACH Simulation Results

Document for:	Discussion

# Introduction

In RAN4#80bis, a WF on NPRACH performance requirements was agreed [1], where it states

Practical NB-IoT PRACH Simulation Results with Implemenration Margin

- The SNR for NPRACH minimum performance

- defined based on Missed detection probability <=1%, including: detecting different preamble than the one that was sent, not detecting a preamble at all, or detecting the correct preamble detection but with the wrong timing estimation.

- Timing limit:

- 7*16Ts=3.646us

- Proposals for defining NPRACH performance requirements

- Interested companies are encouraged to

- provide the ideal simulation results as per the simulation assumptions in the following slide, and distribute the results in RAN4 reflector no later than Oct 28;

- provide the practical simulation results and distribute the results in RAN4 reflector no later than Nov.1;

- NPRACH performance requirements will be determined in RAN4#81 based on submitted practical simulation results

- If there are still large difference in practical simulation results from interested companies, additional margin [x dB] may be added into the averaged practical simulation results. The additional margin [x dB] will be determined in RAN4#81.

In addition, the NPRACH simulation assumptions were updated as follows:

| Parameter | Values |
| --- | --- |
| NPRACH preamble format | 0 |
| Number of Repetitions | 8, 32 |
| Number of Subcarriers | 12 |
| Antenna configuration | 1 x 2 |
| Cell-ID | 0 |
| NPRACH signature | 0 |
| frequency offset | 0Hz (AWGN); 200Hz EPA1 |
| Propagation channels | AWGN, EPA1 |
| Detection performance | Missed detection rate (1%) with false alarm rate (0.1%) |
| Timing estimation | Timing error probability with the limit of 3.646us |
| Timing Offset | [30]us |

In RAN4#80bis, we have provided the ideal simulation results [2] based on [3]. In this paper, we provide the updated ideal simulation results as well as the practical simulation results with the implementation margin based on the WF [1] agreed in RAN4#80bis.

# Simulation Results

Based on the WF [1], the ideal simulation results are shown in Table 1 and 2. The practical simulation results are shown in Table 3 and 4.

Table 1. Ideal NPRACH simulation results with repetition 8

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -4.27 | -4.27 |
|  |  | EPA1 Low | 200 Hz | 4.37 | 4.37 |

Table 2 Ideal NPRACH simulation results with repetition 32

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -9.27 | -9.27 |
|  |  | EPA1 Low | 200 Hz | -1.54 | -1.54 |

Table 3 Practical NPRACH simulation results with repetition 8

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -1.27 | -1.27 |
|  |  | EPA1 Low | 200 Hz | 7.37 | 7.37 |

Table 4 Practical NPRACH simulation results with repetition 32

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | -6.27 | -6.27 |
|  |  | EPA1 Low | 200 Hz | 1.46 | 1.46 |

# Summary

In this paper, we provided the updated ideal simulation results as well as the practical simulation results with the implementation margin based on the WF agreed in RAN4#80bis.

# References

- R4-168745, “WF on NB-IoT NPRACH demodulation performance”, Nokia, Ericsson, Samsung, Huawei, ZTE

- R4-167869, “Ideal NB-IoT PRACH demodulation performance”, Nokia, Alcatel-Lucent Shanghai Bell

- R4-167115, WF on NPRACH performance requirements, Nokia, Ericsson, ZTE

- 3GPP TS 36.104