3GPP TSG RAN WG4 Meeting #81	R4-1609840

Reno, Nevada, US November 14 – 18, 2016

Agenda Item:	8.20.4.7

Source: 	Ericsson

Title:	V2X S-RSRP simulation results

Document for:	Discussion

# Introduction

In the V2X work item [1] RAN4 is requested to specify RRM core requirements. RAN1 has agreed on working assumptions stating that absolute velocity of 250km/h and relative velocity of 500km/h is to be supported for V2V operation in the 5.9GHz band. In this contribution we provide simulation results for the simulation parameters agreed in [2] at the RAN4#80bis meeting.

The following channel conditions are investigated:

- AWGN,

- ETU500, and

- EVA2700.

For each of the channel conditions, carrier frequency offsets of 0, 0.2 and 0.3ppm are investigated. The carrier frequency offsets are with respect to the sidelink carrier that is assumed to be 5.9 GHz.

The contribution is organized as follows. DMRS and the S-RSRP measurements conducted on those are described in Section 2 along with a sensitivity analysis with respect to carrier frequency offset (CFO) for the S-RSRP estimator. Simulation assumptions and simulation results for AWGN, ETU500 and EVA2700 with CFOs 0, 0.2 and 0.3ppm are provided in Section 3. The simulations are summarized and concluded in Section 4.

# S-RSRP measurements

S-RSRP measurements are carried out on DMRS that are transmitted over a sidelink in PSBCH subframes; see Figure 1. RAN1 has agreed that DMRS shall be transmitted in OFDM symbols #4, #6 and #9. The repetition period for PSBCH is under discussion in RAN1. The working agreement from RAN1#86 states that it shall be repeated every 160ms, to be compared with every 40ms in Rel-13.

For S-RSRP the measurements are carried out by a combination of coherent and non-coherent averaging operations. The DMRS in a PSBCH subframe are de-rotated to obtain channel samples, and then an average is calculated based on a number of adjacent channel samples. The simulation results provided in this contribution are based on coherent averaging on symbol basis of channel samples, where each average comprises 12 REs as indicated in blue in Figure 1.

In order to estimate the power, the absolute values of the channel sample averages are squared and averaged non-coherently. The non-coherent averaging may extend over multiple subframes, where in legacy LTE the baseline was to average non-coherently over a period of 200ms. The simulation results here are based on non-coherent averaging over 2 or 4 PSBCH subframes.

Figure 1: DMRS layout in PSBCH subframe (shaded REs). Blue boxes indicate the REs in each PRB pair over which coherent averaging is carried out for S-RSRP measurements (UE implementation-specific).

The measurement accuracy as function of frequency offset for the S-RSRP estimator used in the simulation is shown in Figure 2. The SNR is 0 dB which means that ideally the S-RSRP/N should also be 0 dB. However, due to averaging coherently over a finite number of channel samples – 12 in this case – some noise remains after the averaging and gives rise to bias. In this case the bias is less than 0.5 dB for the 50th percentile. At a frequency offset of ±2700Hz the measured S-RSRP drops off about 0.5 dB.

Observation 1: Symbol-based S-RSRP measurements are robust to carrier frequency offsets of the magnitude foreseen in V2V operation.

Figure 2: Impact of frequency offsets in the range -7.5 to 7.5 kHz on S-RSRP estimates. Propagation condition AWGN, SNR 0 dB. Percentiles: 5th, 50th and 95th.

# Simulations

## Simulation assumptions

The simulation assumptions are based on the proposal in [2]. The parameters used in the simulation are provided in Table 1.

Table 1: Simulation assumptions

| Parameters | Value | Comments |
| --- | --- | --- |
| Frame structure type | 1 | FDD |
| Measurement bandwidth | 6 resource blocks |  |
| System bandwidth | 50 resource blocks |  |
| L1 measurement period | 320ms, 640ms | Sliding average over 2 and 4 most recent snapshots, respectively |
| Snapshot periodicity | 160ms | Measure S-RSRP on DMRS in each PSBCH subframe |
| L3 filtering | Disabled |  |
| Transmit antenna | 1 |  |
| Receive antennas | 2 | Strongest value after L1 filtering over RX1 and RX2 branches is reported |
| Propagation conditions | AWGN, ETU500, EVA2700 |  |
| CP length | Normal |  |
| Carrier frequency | 5.9GHz | Sidelink carrier |
| Carrier frequency offset | 0, 0.2ppm, 0.3ppm | Sidelink carrier |
| Es/Iot | -6 dB … 3 dB | AWGN noise |

## AWGN simulation results

### Layer 1 measurement period 320ms

Figure 3: S-RSRP measurement performance for AWGN propagation condition for 0, 0.2 and 0.3 ppm CFO and 320ms measurement period.

Table 2: 5th, 50th and 95th percentiles of deviation between ideal and measured S-RSRP for AWGN propagation condition with 0, 0.2 and 0.3 ppm CFO and 320ms measurement period.

| SNR[dB] | Delta [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 ppm |  |  | 0.2 ppm |  |  | 0.3 ppm |  |  |
|  | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile |
| -10 | 2,15 | 2,95 | 3,73 | 2,12 | 2,90 | 3,65 | 1,97 | 2,83 | 3,67 |
| -9 | 1,80 | 2,53 | 3,30 | 1,73 | 2,45 | 3,24 | 1,68 | 2,42 | 3,19 |
| -8 | 1,41 | 2,10 | 2,83 | 1,31 | 2,03 | 2,77 | 1,26 | 2,02 | 2,79 |
| -7 | 1,08 | 1,78 | 2,40 | 0,96 | 1,73 | 2,44 | 0,97 | 1,67 | 2,32 |
| -6 | 0,90 | 1,51 | 2,16 | 0,82 | 1,44 | 2,08 | 0,71 | 1,35 | 1,94 |
| -5 | 0,70 | 1,23 | 1,79 | 0,52 | 1,11 | 1,75 | 0,45 | 1,09 | 1,67 |
| -4 | 0,50 | 1,03 | 1,68 | 0,44 | 0,97 | 1,51 | 0,32 | 0,89 | 1,39 |
| -3 | 0,43 | 0,87 | 1,38 | 0,31 | 0,79 | 1,30 | 0,19 | 0,68 | 1,20 |
| -2 | 0,30 | 0,71 | 1,17 | 0,17 | 0,62 | 1,09 | 0,06 | 0,55 | 0,98 |
| -1 | 0,19 | 0,60 | 1,02 | 0,15 | 0,52 | 0,95 | -0,03 | 0,40 | 0,86 |
| 0 | 0,11 | 0,50 | 0,84 | 0,09 | 0,43 | 0,79 | -0,08 | 0,31 | 0,70 |
| 1 | 0,06 | 0,41 | 0,77 | -0,01 | 0,31 | 0,65 | -0,11 | 0,23 | 0,60 |
| 2 | 0,03 | 0,32 | 0,63 | -0,03 | 0,27 | 0,58 | -0,13 | 0,16 | 0,49 |
| 3 | 0,01 | 0,28 | 0,54 | -0,06 | 0,19 | 0,50 | -0,17 | 0,11 | 0,38 |
| 4 | 0,00 | 0,23 | 0,48 | -0,07 | 0,16 | 0,41 | -0,18 | 0,05 | 0,29 |
| 5 | 0,00 | 0,21 | 0,47 | -0,09 | 0,12 | 0,36 | -0,19 | 0,01 | 0,26 |
| 6 | 0,00 | 0,19 | 0,38 | -0,11 | 0,10 | 0,28 | -0,22 | -0,02 | 0,19 |
| 7 | -0,02 | 0,14 | 0,32 | -0,12 | 0,06 | 0,23 | -0,23 | -0,07 | 0,12 |
| 8 | -0,03 | 0,12 | 0,29 | -0,13 | 0,04 | 0,22 | -0,22 | -0,07 | 0,10 |
| 9 | -0,03 | 0,10 | 0,26 | -0,11 | 0,02 | 0,17 | -0,21 | -0,08 | 0,05 |
| 10 | -0,03 | 0,07 | 0,22 | -0,12 | 0,01 | 0,14 | -0,22 | -0,10 | 0,03 |

### Layer 1 measurement period 640ms

Figure 4: S-RSRP measurement performance for AWGN propagation condition for 0, 0.2 and 0.3 ppm CFO and 640ms measurement period.

Table 3: 5th, 50th and 95th percentiles of deviation between ideal and measured S-RSRP for AWGN propagation condition with 0, 0.2 and 0.3 ppm CFO and 640ms measurement period.

| SNR[dB] | Delta [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 ppm |  |  | 0.2 ppm |  |  | 0.3 ppm |  |  |
|  | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile |
| -10 | 2,34 | 2,87 | 3,47 | 2,22 | 2,79 | 3,33 | 2,21 | 2,74 | 3,30 |
| -9 | 1,91 | 2,46 | 3,04 | 1,87 | 2,37 | 2,91 | 1,75 | 2,32 | 2,82 |
| -8 | 1,54 | 2,09 | 2,56 | 1,39 | 1,98 | 2,56 | 1,45 | 1,91 | 2,39 |
| -7 | 1,11 | 1,67 | 2,18 | 1,19 | 1,64 | 2,07 | 1,05 | 1,59 | 2,09 |
| -6 | 0,97 | 1,44 | 1,86 | 0,86 | 1,30 | 1,81 | 0,83 | 1,29 | 1,76 |
| -5 | 0,75 | 1,17 | 1,65 | 0,66 | 1,09 | 1,54 | 0,68 | 1,04 | 1,51 |
| -4 | 0,58 | 0,94 | 1,32 | 0,54 | 0,91 | 1,33 | 0,46 | 0,84 | 1,19 |
| -3 | 0,49 | 0,81 | 1,16 | 0,42 | 0,74 | 1,07 | 0,30 | 0,63 | 0,98 |
| -2 | 0,38 | 0,66 | 1,00 | 0,28 | 0,61 | 0,96 | 0,20 | 0,50 | 0,83 |
| -1 | 0,27 | 0,53 | 0,82 | 0,15 | 0,47 | 0,74 | 0,08 | 0,38 | 0,69 |
| 0 | 0,21 | 0,45 | 0,77 | 0,14 | 0,37 | 0,64 | 0,02 | 0,32 | 0,56 |
| 1 | 0,12 | 0,37 | 0,60 | 0,08 | 0,29 | 0,51 | -0,02 | 0,22 | 0,44 |
| 2 | 0,10 | 0,31 | 0,52 | 0,02 | 0,23 | 0,44 | -0,08 | 0,13 | 0,37 |
| 3 | 0,08 | 0,25 | 0,44 | -0,02 | 0,16 | 0,38 | -0,11 | 0,06 | 0,27 |
| 4 | 0,06 | 0,22 | 0,40 | -0,05 | 0,12 | 0,31 | -0,17 | 0,03 | 0,20 |
| 5 | 0,02 | 0,17 | 0,33 | -0,08 | 0,08 | 0,28 | -0,16 | -0,01 | 0,14 |
| 6 | 0,01 | 0,14 | 0,29 | -0,06 | 0,07 | 0,21 | -0,17 | -0,04 | 0,11 |
| 7 | 0,00 | 0,12 | 0,26 | -0,09 | 0,03 | 0,17 | -0,19 | -0,05 | 0,07 |
| 8 | 0,00 | 0,11 | 0,22 | -0,10 | 0,02 | 0,12 | -0,19 | -0,08 | 0,03 |
| 9 | -0,01 | 0,08 | 0,17 | -0,10 | 0,00 | 0,10 | -0,21 | -0,11 | -0,02 |
| 10 | -0,02 | 0,08 | 0,17 | -0,09 | -0,01 | 0,08 | -0,21 | -0,12 | -0,02 |

## ETU500 simulation results

### Layer 1 measurement period 320ms

Figure 5: S-RSRP measurement performance for ETU500 propagation condition for 0, 0.2  and 0.3 ppm CFO and 320ms measurement period..

Table 4: 5th, 50th and 95th percentiles of deviation between ideal and measured S-RSRP for ETU500 propagation condition with 0, 0.2 and 0.3 ppm CFO for 320ms measurement period.

| SNR[dB] | Delta [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 ppm |  |  | 0.2 ppm |  |  | 0.3 ppm |  |  |
|  | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile |
| -10 | 1,17 | 2,92 | 5,02 | 1,30 | 2,88 | 4,85 | 1,13 | 2,86 | 4,80 |
| -9 | 0,49 | 2,39 | 4,54 | 0,56 | 2,33 | 4,39 | 0,57 | 2,38 | 4,44 |
| -8 | 0,22 | 2,23 | 4,30 | 0,21 | 2,28 | 4,68 | 0,04 | 1,91 | 4,16 |
| -7 | -0,37 | 1,82 | 4,25 | -0,13 | 1,76 | 4,24 | -0,30 | 1,63 | 4,04 |
| -6 | -0,76 | 1,43 | 3,75 | -0,48 | 1,68 | 4,07 | -0,71 | 1,40 | 3,68 |
| -5 | -1,32 | 1,01 | 3,88 | -0,82 | 1,47 | 3,82 | -1,33 | 1,22 | 3,70 |
| -4 | -1,04 | 1,31 | 3,91 | -1,20 | 0,88 | 3,34 | -1,23 | 0,84 | 3,50 |
| -3 | -1,73 | 0,78 | 3,59 | -1,57 | 0,91 | 3,79 | -1,54 | 0,99 | 3,48 |
| -2 | -1,81 | 0,75 | 3,47 | -1,76 | 0,68 | 3,66 | -2,06 | 0,30 | 2,91 |
| -1 | -2,05 | 0,74 | 3,34 | -1,72 | 0,76 | 3,58 | -1,83 | 0,52 | 3,26 |
| 0 | -2,18 | 0,41 | 3,25 | -1,70 | 0,76 | 3,31 | -2,20 | 0,44 | 3,44 |
| 1 | -2,45 | 0,41 | 3,25 | -2,28 | 0,49 | 3,33 | -2,28 | 0,43 | 3,19 |
| 2 | -2,16 | 0,50 | 3,55 | -2,47 | 0,41 | 3,25 | -2,38 | 0,37 | 3,04 |
| 3 | -2,04 | 0,74 | 3,51 | -2,30 | 0,44 | 3,21 | -2,47 | 0,61 | 3,41 |
| 4 | -2,34 | 0,68 | 3,73 | -2,22 | 0,42 | 3,33 | -2,62 | 0,36 | 3,28 |
| 5 | -2,30 | 0,77 | 3,57 | -2,56 | 0,47 | 3,44 | -2,75 | 0,14 | 2,93 |
| 6 | -2,70 | 0,20 | 3,22 | -2,52 | 0,39 | 3,59 | -2,49 | 0,31 | 3,06 |
| 7 | -2,49 | 0,38 | 3,42 | -2,54 | 0,47 | 3,43 | -2,42 | 0,47 | 3,33 |
| 8 | -2,65 | 0,39 | 3,30 | -2,24 | 0,57 | 3,20 | -2,38 | 0,39 | 3,32 |
| 9 | -2,58 | 0,66 | 3,66 | -2,20 | 0,46 | 3,21 | -2,55 | 0,27 | 3,36 |
| 10 | -2,38 | 0,29 | 3,28 | -2,66 | 0,46 | 3,45 | -2,79 | 0,25 | 3,01 |

### Layer 1 measurement period 640ms

Figure 6: S-RSRP measurement performance for ETU500 propagation condition for 0, 0.2  and 0.3 ppm CFO and 640ms measurement period..

Table 5: 5th, 50th and 95th percentiles of deviation between ideal and measured S-RSRP for ETU500 propagation condition with 0, 0.2 and 0.3 ppm CFO for 640ms measurement period.

| SNR[dB] | Delta [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 ppm |  |  | 0.2 ppm |  |  | 0.3 ppm |  |  |
|  | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile |
| -10 | 1,97 | 2,94 | 4,37 | 1,52 | 2,75 | 4,26 | 1,52 | 2,74 | 4,32 |
| -9 | 1,17 | 2,38 | 3,98 | 0,99 | 2,37 | 3,96 | 0,92 | 2,25 | 3,82 |
| -8 | 0,75 | 2,16 | 3,88 | 0,48 | 1,82 | 3,47 | 0,54 | 1,81 | 3,35 |
| -7 | 0,11 | 1,54 | 3,27 | 0,12 | 1,73 | 3,30 | 0,06 | 1,54 | 3,28 |
| -6 | 0,02 | 1,74 | 3,48 | -0,02 | 1,55 | 3,19 | -0,25 | 1,27 | 2,99 |
| -5 | -0,32 | 1,34 | 3,07 | -0,73 | 0,98 | 2,73 | -0,46 | 1,16 | 2,83 |
| -4 | -0,57 | 1,19 | 3,14 | -0,89 | 0,81 | 2,81 | -0,72 | 0,88 | 2,74 |
| -3 | -0,94 | 0,73 | 2,59 | -0,87 | 1,09 | 3,04 | -0,97 | 0,80 | 2,59 |
| -2 | -0,81 | 1,04 | 3,11 | -1,29 | 0,57 | 2,34 | -0,99 | 0,98 | 2,70 |
| -1 | -1,11 | 0,66 | 2,77 | -1,08 | 0,67 | 2,59 | -1,45 | 0,42 | 2,52 |
| 0 | -1,48 | 0,40 | 2,22 | -1,54 | 0,57 | 2,64 | -1,49 | 0,44 | 2,53 |
| 1 | -1,89 | 0,00 | 1,96 | -1,30 | 0,34 | 2,25 | -1,57 | 0,34 | 2,69 |
| 2 | -1,49 | 0,52 | 2,75 | -1,46 | 0,41 | 2,71 | -1,98 | 0,31 | 2,46 |
| 3 | -1,63 | 0,66 | 2,63 | -1,74 | 0,36 | 2,34 | -2,10 | -0,06 | 2,05 |
| 4 | -1,63 | 0,51 | 2,75 | -1,84 | 0,23 | 2,18 | -1,70 | 0,38 | 2,45 |
| 5 | -1,64 | 0,31 | 2,37 | -2,29 | 0,29 | 2,43 | -1,86 | 0,32 | 2,16 |
| 6 | -1,79 | 0,36 | 2,40 | -1,47 | 0,44 | 2,70 | -1,79 | 0,07 | 2,28 |
| 7 | -1,84 | 0,24 | 2,45 | -1,87 | 0,37 | 2,56 | -1,98 | 0,18 | 2,20 |
| 8 | -1,63 | 0,34 | 2,45 | -2,06 | -0,07 | 2,03 | -1,68 | 0,20 | 2,36 |
| 9 | -2,20 | 0,07 | 2,43 | -1,92 | 0,20 | 2,36 | -1,80 | 0,34 | 2,45 |
| 10 | -1,89 | 0,53 | 2,48 | -1,71 | 0,29 | 2,24 | -2,00 | 0,22 | 2,41 |

## EVA2700 simulation results

### Layer 1 measurement period 320ms

Figure 7: S-RSRP measurement performance for EVA2700 propagation condition with 0, 0.2 and 0.3 ppm CFO and 320ms measurement period.

Table 6: 5th, 50th and 95th percentiles of deviation between ideal and measured S-RSRP for EVA2700 propagation condition with 0, 0.2 and 0.3 ppm CFO and 320ms measurement period.

| SNR[dB] | Delta [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 ppm |  |  | 0.2 ppm |  |  | 0.3 ppm |  |  |
|  | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile |
| -10 | 1,69 | 2,96 | 4,22 | 1,60 | 2,89 | 4,11 | 1,59 | 2,81 | 4,30 |
| -9 | 1,18 | 2,57 | 4,01 | 1,21 | 2,45 | 3,75 | 1,02 | 2,36 | 3,83 |
| -8 | 0,74 | 2,10 | 3,62 | 0,63 | 2,10 | 3,44 | 0,67 | 2,01 | 3,43 |
| -7 | 0,17 | 1,58 | 3,03 | 0,11 | 1,65 | 3,24 | 0,10 | 1,78 | 3,49 |
| -6 | -0,05 | 1,40 | 3,03 | -0,15 | 1,38 | 3,03 | -0,32 | 1,33 | 3,06 |
| -5 | -0,55 | 1,15 | 2,71 | -0,37 | 1,14 | 2,84 | -0,45 | 1,13 | 2,88 |
| -4 | -0,58 | 0,98 | 2,82 | -0,62 | 0,99 | 2,89 | -0,64 | 0,96 | 2,62 |
| -3 | -0,81 | 0,92 | 2,63 | -0,92 | 0,84 | 2,67 | -0,74 | 0,93 | 2,54 |
| -2 | -1,03 | 0,82 | 2,62 | -0,82 | 0,85 | 2,86 | -1,24 | 0,65 | 2,23 |
| -1 | -1,13 | 0,63 | 2,58 | -1,10 | 0,87 | 2,64 | -1,07 | 0,57 | 2,41 |
| 0 | -1,12 | 0,74 | 2,57 | -1,10 | 0,65 | 2,72 | -1,50 | 0,42 | 2,34 |
| 1 | -1,19 | 0,51 | 2,39 | -1,13 | 0,55 | 2,32 | -1,34 | 0,56 | 2,28 |
| 2 | -1,80 | 0,30 | 2,05 | -1,64 | 0,34 | 2,46 | -1,74 | 0,18 | 1,82 |
| 3 | -1,44 | 0,38 | 2,45 | -1,33 | 0,44 | 2,40 | -1,33 | 0,35 | 2,23 |
| 4 | -1,40 | 0,52 | 2,38 | -1,65 | 0,12 | 1,79 | -1,64 | 0,25 | 2,05 |
| 5 | -1,42 | 0,37 | 2,39 | -1,53 | 0,49 | 2,36 | -1,69 | 0,29 | 2,24 |
| 6 | -1,52 | 0,39 | 2,17 | -1,38 | 0,42 | 2,25 | -1,72 | 0,13 | 2,32 |
| 7 | -1,64 | 0,22 | 1,96 | -1,75 | 0,32 | 1,99 | -1,75 | 0,11 | 2,13 |
| 8 | -1,90 | 0,04 | 1,84 | -1,87 | 0,23 | 2,07 | -1,67 | 0,13 | 2,07 |
| 9 | -1,23 | 0,55 | 2,26 | -1,74 | 0,30 | 2,10 | -1,80 | 0,15 | 1,99 |
| 10 | -1,51 | 0,33 | 2,40 | -1,64 | 0,29 | 2,28 | -1,88 | 0,00 | 2,00 |

### Layer 1 measurement period 640ms

Figure 8: S-RSRP measurement performance for EVA2700 propagation condition with 0, 0.2 and 0.3 ppm CFO and 640ms measurement period.

Table 7: 5th, 50th and 95th percentiles of deviation between ideal and measured S-RSRP for EVA2700 propagation condition with 0, 0.2 and 0.3 ppm CFO and 640ms measurement period.

| SNR[dB] | Delta [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 ppm |  |  | 0.2 ppm |  |  | 0.3 ppm |  |  |
|  | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile | 05 %tile | 50 %tile | 95 %tile |
| -10 | 1,84 | 2,77 | 3,66 | 1,73 | 2,68 | 3,58 | 1,85 | 2,69 | 3,65 |
| -9 | 1,40 | 2,31 | 3,31 | 1,23 | 2,32 | 3,41 | 1,46 | 2,25 | 3,25 |
| -8 | 1,04 | 2,12 | 3,27 | 0,80 | 1,85 | 2,95 | 0,94 | 2,01 | 3,33 |
| -7 | 0,70 | 1,67 | 2,71 | 0,52 | 1,52 | 2,65 | 0,34 | 1,55 | 2,84 |
| -6 | 0,21 | 1,30 | 2,74 | 0,23 | 1,27 | 2,49 | 0,14 | 1,15 | 2,43 |
| -5 | 0,00 | 1,13 | 2,60 | -0,10 | 1,05 | 2,55 | -0,01 | 1,20 | 2,68 |
| -4 | -0,28 | 0,93 | 2,19 | -0,23 | 0,96 | 2,18 | -0,51 | 0,66 | 2,12 |
| -3 | -0,79 | 0,59 | 1,76 | -0,70 | 0,69 | 2,04 | -0,90 | 0,61 | 2,07 |
| -2 | -0,83 | 0,60 | 1,90 | -0,54 | 0,82 | 2,25 | -0,80 | 0,48 | 1,75 |
| -1 | -0,63 | 0,66 | 1,91 | -0,83 | 0,55 | 2,11 | -0,92 | 0,38 | 1,76 |
| 0 | -0,97 | 0,53 | 1,83 | -0,64 | 0,49 | 1,83 | -0,72 | 0,48 | 1,76 |
| 1 | -0,88 | 0,53 | 1,85 | -1,21 | 0,12 | 1,69 | -0,99 | 0,42 | 2,01 |
| 2 | -1,16 | 0,36 | 1,96 | -0,96 | 0,29 | 1,65 | -0,72 | 0,49 | 1,85 |
| 3 | -0,86 | 0,60 | 1,95 | -1,25 | 0,44 | 1,86 | -1,52 | 0,16 | 1,58 |
| 4 | -1,15 | 0,26 | 1,86 | -1,03 | 0,16 | 1,60 | -1,39 | -0,04 | 1,64 |
| 5 | -1,30 | 0,14 | 1,33 | -1,16 | 0,20 | 1,94 | -1,18 | 0,11 | 1,40 |
| 6 | -1,07 | 0,23 | 1,92 | -1,29 | 0,11 | 1,46 | -1,34 | 0,07 | 1,38 |
| 7 | -1,30 | -0,02 | 1,53 | -1,26 | 0,01 | 1,47 | -1,37 | 0,08 | 1,41 |
| 8 | -1,09 | 0,18 | 1,74 | -1,20 | 0,41 | 1,87 | -1,07 | 0,17 | 1,70 |
| 9 | -1,03 | 0,36 | 1,63 | -1,22 | 0,04 | 1,49 | -1,46 | 0,23 | 1,86 |
| 10 | -1,24 | 0,31 | 1,80 | -1,14 | 0,36 | 1,62 | -1,68 | -0,05 | 1,52 |

# Summary and Conclusion

In this contribution we have provided simulation results for S-RSRP measurements for a relative speed of 500km/h in accordance with the simulation assumptions in [2]. We make the following observations-

Observation 1: For the symbol-based S-RSRP estimator there is little impact of carrier frequency offset on bias and variance of the S-RSRP estimate, as can be seen by comparing the results for 0, 0.2 and 0.3ppm offset.

Observation 2: For the longer L1 measurement period 640ms, the bias is similar to the 320ms case, whereas the variance is significantly reduced, as can be seen by comparing results for 320 and 640ms, respectively, for each of the carrier frequency offsets 0, 0.2 and 0.3ppm.

Observation 3: The largest deviation of the 5 and 95th percentiles from the ideal S-RSRP at SINR -6dB is 4.07dB which is achieved for ETU500 with a measurement period of 320ms.

# References

- RP-161894, “Revised WI proposal: LTE-based V2X Services”, LG Electronics, Huawei, CATT

- R4-167232, “Simulation assumption of S-RSRP measurement for V2X”, LG Electronics