3GPP TSG RAN4 Meeting #81	R4-1609987

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 19+21+42 2UL/3DL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Approval

# 1	Introduction

This contribution discusses MSD requirements for 19+21+42 2UL/3DL CA.

# 2	Discussion

This CA combination has IMD4 falling on top of own B19 RX and B42RX . The test point was approved in [1]:

| B19+B21+B42 | B19 | IMD4 | 3*fB21 - fB19 | 840 | 5 | 25 | 3511.2 | 5 | TBD | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B21 |  |  | 1450.4 | 5 | 25 |  |  |  |  |
|  | B21 | IMD4 | 3*fB21 - fB42 | 1455.4 | 5 | 25 | 883.7 | 5 | TBD | TBD |
|  | B42 |  |  | 3482.5 | 5 | 25 |  |  |  |  |

We use conventional common triplexer based architecture in our analysis with the following component assumptions. Diplexer/Triplexer is not assumed in diversity.

| Component | IP4 (dBm) |
| --- | --- |
| Antenna switch | 56 |
| Diplexer/Triplexer | 55 |
| Duplexer | 55 |
| LNA | -6 |

B21 duplexer TX attenuation at B19 RX							        23

B19 duplexer TX attenuation at B42 RX							        40

B21 PA forward mixing IMD4 at PA output with 60dB isolation	       -40

B42 FE loss (switch+trace+filter)										    6

B21 duplexer TX attenuation at B19 RX							        32

B42 filter TX attenuation at B19 RX							              40

B21 PA forward mixing IMD4 at PA output with 60dB isolation	       -45

B19 FE loss (switch+trace+filter)										    5

Antenna isolation 														  15

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Triplexer isolation														  15

Triplexer IL															       1.5

LNA NF																   	    5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s. CF is 1dB in this analysis.

IMD4 dominant factors for B42:

IMD4 dominant factors for B42:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B19+B21+B42 | B19 | IMD4 | 3*fB21 - fB19 | 840 | 5 | 25 | 3511.2 | 5 | 1 | 14.2 |
|  | B21 |  |  | 1450.4 | 5 | 25 |  |  |  |  |
|  | B21 | IMD4 | 3*fB21 - fB42 | 1455.4 | 5 | 25 | 883.7 | 5 | 1 | 12.4 |
|  | B42 |  |  | 3482.5 | 5 | 25 |  |  |  |  |

# 3	Conclusion

MSD analysis for CA_19A-21A-42A was provided with the following results.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B19+B21+B42 | B19 | IMD4 | 3*fB21 - fB19 | 840 | 5 | 25 | 3511.2 | 5 | 1 | 14.2 |
|  | B21 |  |  | 1450.4 | 5 | 25 |  |  |  |  |
|  | B21 | IMD4 | 3*fB21 - fB42 | 1455.4 | 5 | 25 | 883.7 | 5 | 1 | 12.4 |
|  | B42 |  |  | 3482.5 | 5 | 25 |  |  |  |  |

# 4	References

[1] R4-168857, “MSD Test configurations for xDL/2UL CA band combinations with self interference issues”, LG, RAN4#80bis

[2] R4-168105, “2UL 3DL Possible testpoints”, Qualcomm Inc, RAN4#80bis