3GPP TSG-RAN Working Group 4 (Radio) meeting #81	R4-1610499

Reno, Nevada, USA, 14th – 18th November 2016

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	OTA EVM requirements of eAAS

Agenda item:	8.12.2.1 [AASenh_BS_LTE_UTRA]

Document for:	Approval

## Introduction

At the 3GPP TSG-RAN WG4 meeting #80bis in Ljubljana, Slovenia, an agreement on OTA EVM requirements of cell-specific beams was reported in the way forward [1]. In addition, [1] outlines three open issues that need to be addressed in the forthcoming meetings, which are quoted below:

- “whether OTA EVM requirements apply per beam or over the declared EVM direction range with only one EVM direction range”

- “how to declare the coverage direction range, either adopting the same as EIRP direction set or as a new EVM direction set, or something else, is still FFS”

- “how to apply EVM requirements for AAS with multiple transceivers but without the ability to user beam steer is FFS”

In this document, we address the aforementioned issues.

## Discussion

According to Clause 7.2.2., TR 37.842, an eAAS base station can generate two types of beams (see Figure 1):

- A user-specific beam which is used to provide communication to the intended UE. As shown in Figure 1, the eAAS base station can simultaneously form multiple user-beams in both the azimuth and elevation planes using the same frequency channel to different UEs. The OTA EVM requirement for user-specific beams was agreed and captured in [2].

- A cell-specific beam which is used to provide communication to all UEs in a cell. An eAAS base station can split a cell into multiple coverage areas (or sectors) horizontally and/or vertically. The latter is depicted in Figure 2 in which each of the sectorized areas is served by a unique cell-specific beam using the same frequency and time resources. The decisions made at the last meeting concerning cell-specific beams are captured in [1] and quoted below:

- “manufacturer declares the intended EVM directions range”

- “the BS shall meet the OTA EVM requirement across the declared EVM directions range”

- “conformance to OTA EVM requirements shall be demonstrated at the extreme directions of the declared coverage direction range and at the centre”

Based on the definition of cell-specific beam, the footprint of the coverage area is dictated by the 3-D radiation pattern of cell-specific beams. Thus, we can define “the extreme directions of the declared coverage direction range” in c) above as the half-power beamwidth in the azimuth and elevation planes as show in Figure 3. The EVM centre direction corresponds to the centre of the main lobe.

z

Azimuth (φ)

Cell-specific beam

x

y

UE1

UE2

Elevation (θ)

Antenna Array

Figure 1: 3-D beamforming of an eAAS base station

z

Antenna Array

x

Elevation (θ)

Cell-specific beam 1

Cell-specific beam 2

Azimuth (φ)

y

z

x

y

Antenna Array

Elevation (θ)

Azimuth (φ)

Cell-specific beam 1

Cell-specific beam 2

z

x

y

Antenna Array

Elevation (θ)

Azimuth (φ)

Cell-specific beam 1

Cell-specific beam 2

Elevation (θ)

Antenna Array

x

Cell-specific beam 1

Cell-specific beam 2

Azimuth (φ)

y

Figure 2: Vertical cell splitting by an AAS base station

EVM centre direction

θ

Cell-specific beam

φ

Elevation directions range = elevation beamwidth

Azimuth directions range = azimuth beamwidth

Figure 3: Coverage direction range declaration for OTA EVM requirements of cell-specific beams

We make the following proposal on EVM requirements for cell-specific beams:

Proposal 1: the EVM coverage directions range consists of an EVM centre direction, azimuth-plane half-power beamwidth and elevation-plane half-power beamwidth. The EVM centre direction corresponds to the centre of the main lobe.

In order to validate Proposal 1, we conducted an experiment by which an antenna radio-frequency system generates beams that mimics eAAS cell-specific beams. Based on our measurement results, the OTA EVM measured in various positions including those in Proposal 1 fulfil the EVM requirement except for the EVM in the back lobe (i.e., 180° from the main lobe). By analogy, we can draw the same conclusion for eAAS.

## Conclusions

This document has addressed the issues of OTA EVM for cell-specific beams. Based on our experimental findings, we have proposed the following on EVM requirements for cell-specific beams:

Proposal 1: the EVM coverage directions range consists of an EVM centre direction, azimuth-plane half-power beamwidth and elevation-plane half-power beamwidth. The EVM centre direction corresponds to the centre of the main lobe.

## References

[1]   R4-168873, Way Forward on the OTA EVM for eAAS, NEC, Huawei, Ericsson

[2]   R4-166930, Way Forward on eAAS, Ericsson