3GPP TSG RAN4 Meeting #81	R4-1610765

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 28+42 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.6.2

Document for:	Approval

# 1	Introduction

This contribution discusses MSD requirements for 28+42 UL CA.

# 2	Discussion

This CA combination has IMD5 falling on top of own B28 RX. The testpoint was suggested in [1]:

MSD for UL CA_28A-42A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| CA_28A-42A | 28 | 705.5 | 5 | 25 | 760.5 | TBD | FDD |
|  | 42 | 3582.5 | 5 | 25 | 3582.5 | NA | TDD |

We use the conventional common diplexer based architecture in our analysis with the following component assumptions. Diplexer was not assumed in diversity path.

| Component | IP5 (dBm) |
| --- | --- |
| Antenna switch | 53 |  |
| Diplexer | 53 |  |
| Duplexer | 53 |  |
| LNA | -10 |  |

B28 PA forward mixing IMD at PA output with 60dB isolation		-37.5

B28 PA forward mixing RFIC Pin2Pin IMD at PA output				 -42

B28 duplexer TX rejection at B28 RX (forward)						  44

Antenna isolation 														  10

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

B28 duplexer TX  isolation											        53

B28 duplexer RX isolation												  53

Diplexer isolation															  15

B28 FE loss (diplexer+switch+duplexer)						    	       5.5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s. CF=2dB in this analysis.

IMD5 dominant factors:

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_28A-42A | 28 | 705.5 | 5 | 25 | 760.5 | 7.3 | FDD | IMD5 |
|  | 42 | 3582.5 | 5 | 25 | 3582.5 | NA | TDD | NA |

# 3	Conclusion

MSD analysis for CA_28A-42A was provided with the following results.

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_28A-42A | 28 | 705.5 | 5 | 25 | 760.5 | 7.3 | FDD | IMD5 |
|  | 42 | 3582.5 | 5 | 25 | 3582.5 | NA | TDD | NA |

# 4	References

[1] R4-168104, “2UL 2DL Possible testpoints”, Qualcomm Inc, RAN4#80bis